### Equivalent permeability analysis: method verification

To judge the accuracy of the computed keq, we used a conceptual 2828 m discrete fracture model (DFMC1) with 27 through-going equally spaced fractures each with a 0.1 mm aperture (Figure 5a). Matrix and fracture permeabilities were 10-15  m2 (1 mD) and 10-4212=8.33×10-10 m2 (833 D), respectively.

For this model an analytical solution can be found using effective medium theory. It is equal to the arithmetic thickness-weighted average along the fractures (x-direction)

|  | kxxeq=km1-1Lj=1nfaj+kfLj=1nfaj | (17) |
| --- | --- | --- |

Harmonic thickness-weighted averaging in the y-direction gives

|  | kyyeq=1-1Lj=1nfajkm+1Lj=1nfajkf-1 | (18) |
| --- | --- | --- |

where aj is the aperture of the j-th fracture, nf the number of fractures in the system and L the model length. The calculated equivalent permeability tensor therefore is

| keq=81.4001×10-15 m2 |  |
| --- | --- |

With a mesh refinement characteristic for the detailed models used in our study, this result was reproduced by the FEM with an accuracy of ±0.05%.

This verification of the method was complemented by tests where the orientation of the fractures in the model was rotated relative to the coordinate axes (Figure 5a).

The analytical solution to this problem is achieved by tensor rotation (Bear, 1972)

|  | keq=kxxcos2θ+kyysin2θkxx-kyysinθcosθkxx-kyysinθcosθkxxsin2θ+kyycos2θ | (19) |
| --- | --- | --- |

where θ is the rotation angle in clockwise direction. For a rotation of -45°, the equivalent permeability tensor becomes

| keq=41.2-40.4-40.441.2×10-15 m2 |  |
| --- | --- |

In comparison between analytical and numerical results for different orientation angles (Figure 5b), we found a maximum Mean Percentage Error (MPE) for the computed keq of less than 0.5%. This is small, indicating that the volume averaging method is sufficiently accurate for our study.

Figure 5. Method verification test: (a) DFMC1 model: A 2828 m discrete fracture model consisting 27 through-going equally spaced (θ=0) fractures each with a 0.1 mm aperture. Rock matrix (shaded gray area) with embedded fractures (horizontal black lines). Permeabilities are 10-15m2 1 mD and 10-4212=8.33×10-10m2 833 D for the matrix and fractures, respectively. (b) Comparison of analytical (dashed lines) and numerical results (circles) for different orientation angles of the verification model.