Complex Systems Modelling Platform

CSMP++

User Guide

copyright (c) S.K. Matthäi, last revised, Octobre 2019.

# Table of Contents

Table of Contents	2

1. What is CSMP++ ?	4

Design Philosophy	7

Design History	8

Getting Started	10

2. From a Real World System to a CSMP++ Model	12

Data Types: Scalar, Vector, Tensor, Array, FlaggedArray.. -Variable	22

3. Computation of a Steady-State Fluid Pressure Distribution	24

4. A Tour of CSMP++	33

Region	33

Boundaries	36

Split Boundaries	38

ErrorHandler	39

PropertyHandle	41

Interrelations	42

Operands	45

Visitors	46

PDE_Integrators	48

PDE Operators	50

5. Defining & Solving Finite-Element Equations	52

Example: Steady-state fluid pressure	54

Transient Fluid Pressure	58

Buoyancy-Driven Flow	59

Lumping	59

LinearElasticity	60

References Cited	63

6. Initial and Essential Conditions	64

7. Input to & Output from CSMP++	68

Model Configuration from File	68

Regular Meshes from Pixel Data	70

1. Drawing the Geologic Cross Section	71

2. Importing the Section into an Image Editing Program	71

3. Color Coding the Image	71

4. Output of the section as color-coded text file	73

Output via the VTK Interface	73

Output via the JPEG Interface	74

8. A Comprehensive Simulation Example: Carbonate Dissolution	75

0. Input Geometry	75

1. Physical Variables	75

2. Material Properties, Initial values, and Boundary Conditions	75

3. The Interrelations	76

4. The calculation	77

5. The Regions	77

6. The Visitors	77

7. FiniteDifferenceGrids	77

8. Speciation Calculations	78

9. Initial Conditions	79

10. The Overall Model	80

9. The CSMP++ Libraries	81

/source_code/algorithms/ Library	82

/source_code/applied_math/ Library	83

/source_code/computational_geometry/ Library	84

/source_code/computer_science/ Library	85

/source_code/eos/ (Equation of State) Library	86

/source_code/finite_elements/ Library	89

The Linear Triangular Element: LinearTriangle	89

The LinearTetrahedron	91

The IsoparametricQuadraticTriangle with 6 Nodes	94

The IsoparametricBarycentricQuadraticTriangle with 7 Nodes	99

The IsoparametricQuadraticTetrahedron with 10 Nodes	102

/source_code/generic_finite_volumes/ Library	104

/source_code/geometry_preprocessor/ Library	105

/source_code/hydrothermal/ Library	106

/source_code/interfaces/ Library	109

/source_code/interrelations/ Library	110

/source_code/main_library/ Library	113

/source_code/pde_operators/ Library	115

PDE Operators for Analytical Integration	118

PDE Operators for Numerical Integration	120

/source_code/solver/ Library	123

/source_code/triangular_finite_volumes/ Library	124

/source_code/unit_test/ Library	128

/source_code/visitors/ Library	129

BoundaryTransferVisitor (2D linear triangles only)	129

InterfaceVisitor (2D linear triangles only)	130

AlterationVisitor	130

H2OPropertiesVisitor	131

HydroFractureVisitor	131

TransportVisitor (2D only)	133

VelocityAndVolumeFluxVisitor	134

Appendix	I

A: The Distribution	I

B: Installation	12

CSMP++ Projects in CodeWarrior	12

PC Windows 2000 / XP	15

Linux	15

SGI / Irix 6.5 or Later	15

CSMP++ Glossary	16

## 1. What is CSMP++ ?

The Complex Systems Modelling Platform (CSMP++) is an application programmer interface (API) invented by S. K. Matthäi in 1995 for the simulation of geological processes and their interactions.  CSMP++ is a cross-platform toolkit for the PC, Macintosh, or Linux-Unix-Irix hardware environments (PC, SGI, Digital Alpha, Sun). The current version CSMP++ reflects the ANSI / ISO C++ standard (July 1998) and compiles with the latest C++ compilers capable of compiling the test program suite of the Blitz++ computational library by Todd Verhuizen.

CSMP++ is a finite element - finite volume computational environment, but some finite-difference functionality is implemented as well. CSMP++ makes extensive use of the C++ Standard Template Library (STL) and the solution of large (>1000 x 1000) (sparse) systems of algebraic equations is accomplished with the algebraic multigrid solver SAMG (e.g., Stüben 2002). Thus, matrix-inversion time relates to matrix size in a n log n fashion (n = rows = columns) as opposed to n2 for the more typically used (and also available) conjugate gradient and direct methods. Given enough computer (RAM) memory, this makes it possible to apply CSMP++ to very large models (>107 degrees of freedom).

CSMP++ implements line, surface and volume based computations. For simple 2D color-coded bitmaps, an integrated mesh generator discretizes input geometry as regular triangle meshes. Planar, straight line graphs (PSLG) of polygons, lines, and regional property information can be discretized using J. Setchuk’s constrained, conforming Delauney triangulation algorithm. Volumetric element meshes with attached material properties can be input to CSMP++ as ANSYS Tetra and hybrid element meshes, or via GoCad’s TSurf and TSolid files. In any of these formats, finite elements can be tagged to specify which geological objects they belong to. CSMP++ will automatically identify element neighbors and outer model boundaries.

Simulated component processes of complex natural systems may be restricted to specific regions of a model. This permits to benefit from the fact that certain subsurface processes affect rock units only above a certain temperature or pressure, relative permeabilities only need to be calculated where multiple fluid phases are present, and thermal volume strain must only be calculated where the rock is cooled or heated. To provide such flexibility, CSMP++ allows restriction of computations to groups of finite elements name regions. A Region is a mesh segment (1D), area (2D) or volume (3D) of finite elements that belong together on the basis of their spatial coordinates, material properties, property ranges or combinations thereof. This domain decomposition capability allows for spatially variable timestepping (certain regions can be solved more often than others), distributed computations (parallel or vector processing) or efficient swapping of restricted memory. CSMP++ is parallelized using MPI and the vectorized version of the AMG solver SAMGp.

Any CSMP++ region can also be monitored and-or output individually for visualization. In 3D, this allows to greatly reduce the number of finite elements that must be rendered and helps to selectively visualize features of interest. In 2D models, computed variables can also be output onto regular grids for faster rendering and animation.

Input-output parameter-range checking for all CSMP++ modules simulating process interactions ascertains the correctness of complex calculations and, in combination with a C++ exception handling scheme, permits the precise location of range violations, as well as occasional error recovery at runtime. To manage the spatial and dynamic complexity of geological systems, abstract data types are used in the implementation of geometric search capabilities and boolean operations on the discretized model geometry are available (union, intersection, difference etc.). In practice, this functionality assumes the same importance as the solution of differential equations.

CSMP++ has been designed with geological applications in mind. Material properties can vary over many orders of magnitude in time and in space and simulations with a high spatial resolution such as for models with cm-wide discrete fractures in km-thick rock packages are feasible. Various provisions have been made to solve linear and non-linear equations, including equations of state and other pressure and temperature-dependent physical variables. To do this, partial differential equations are integrated in a cell-by-cell fashion by PDE_Integrator objects. To integrate pre- or user-defined partial differential equation in space and in time, specific operators provide integral forms of differential operators and associate them with physical variables.

Geological applications necessitate the solution of weakly to fully coupled equations, as well as the post-processing of physical variables derived from the (primary) dependent variables in the global system of algebraic equations (for instance, the Darcy velocity from pressure gradients, permeability and dynamic viscosity). These secondary physical variables track constitutive relationships and / or component processes in the geological system such as change of permeability during sediment compaction. CSMP++ encapsulates such interactions in Visitor objects. They can compute a range of variable values at once as is done by the H2OPropertiesVisitor for the PT-dependent properties of water. To honor range of validity constraints, user-defined visitors have to perform a range checks on their input variables. By contrast, their output variables are range-checked automatically using bounds defined in a specific input file.

To directly modify, combine, or calculate distributed physical variables, one can create or associate variables with PropertyHandle objects.  They permit to add, subtract, and multiply operands, creating temporary variables at runtime, and modifying properties of model sub-regions identified by Region, Boundary or SplitBoundary objects, selectively.

Every CSMP++ model also contains a PropertyDatabase managing the physical variables for a specific simulation. For each variable, it keeps a Parameter record of the variable type (scalar, vector, tensor, array) and placement (node, element etc.). This database usually is initialized from a text file called “*-variables.txt”. Alternatively, variables can be defined and added to the database at runtime. Importantly, the storage for each variable is allocated dynamically and in association with the storage sites (nodes, elements etc.). This is essential because it allows the model to shrink during potential remeshing in a simulation.

CSMP++ represents an API (application programmer interface). This implies that users must write their own main programs. To learn how to accomplish this, the user will start by customizing examples or exisiting applications according to his / her needs. A wide range of examples is provided with CSMP++, including:

- Steady-state or transient topography-, pressure- and-or buoyancy driven Darcian fluid flows, including fluid sources and sinks and and a stream function solution to visualize flow lines in 2D.

- Diffusion and advection as well as dispersion of heat, solutes or reactants with the flow.

- Rate-controlled gradient reactions along the fluid flow path, such as calcite dissolution and precipitation.

- Two-phase flow of slightly compressible wetting and non-wetting phases using the Linear, Brooks-Corey or Van Genuchten displacement model.

- Two-phase fluid flow of brine and steam in intrusive-related hydrothermal settings.

- Linear-elastic deformation of a heterogeneous rock mass.

- etc.

- Combinations of the aforementioned processes plus a variety of interdependencies among physical variables.

CSMP++ is organized into a series of libraries with different themes like computational geometry or equations of state. The centerpiece of it is the main library that contains the base classes from which user-defined types are derived. Everybody is invited to add to this framework in order to create the capabilities regarding the simulation of natural systems.

CSMP++ also provides utility classes for geometric manipulations of models and data (for instance polygons, triangles, line segments and points, DXF file exchange etc.), for the pre-processing of input data and for the communication with other computational tools. A number of interface modules connect CSMP++ to the ANSYS mesh generator, the Visualization Toolkit (VTK, kitware.com), GoCad, Tekplot and other programs.

### Design Philosophy

CSMP++ adheres to an object-oriented design with the goal to disassemble the complexity of geological and other processes into components that are small enough that we can understand and express them mathematically, verify their accurate model behaviour, and ascertain that the interactions of such modular components occur within parameter value ranges for which their correct behavior has been verified. While we can understand component processes in isolation, without simulation, we cannot foresee how these will interact or what system behavior they may produce, especially when material properties have values which vary over many orders of magnitude. To facilitate the simulation of such complex interactions and the resulting overall system behaviour is the design objective of CSMP++.  Years of development following this objective have made CSMP++ a tool for simulation of complex earth-,  engineering-, and other systems that lends itself to:

- Investigate how processes that are well understood in isolation will interact,

- Carry out sensitivity analyses of how system behavior changes in response to critical variables,

- Investigate processes that cannot be observed directly because they occur over very long spans of time or at a great depth,

- Interpret and understand the meaning of observations and measurements on complex natural systems,

- Evaluate proposed cause-and-effect chains in complex geological systems,

- Determine which data should be collected and where in order to constrain specific processes,

- Forecast how man-made changes to natural systems are likely to affect their future behavior.

- Answer  the typical What-If?  questions of engineering.

### Design History

CSMP++ was invented and implemented by Stephan Matthäi using an object-oriented design and the programming language C++. The prototype was created in collaboration with Stephen Roberts (ANU, Australia) and applied in subsequently published research conducted at Stanford University, CA (USA). Bruce Eckel (Mindware.com) made very valuable suggestions that greatly improved the first incarnation of CSMP++ that became CSMP++ in 2008, after a complete rewrite.

Between September 1996 and December 1999, Stephan completely rewrote and extended CSMP++ while being a member of Chris Heinrich’s Fluids and Ore Deposits Group at the ETH Zürich, Switzerland. This time period constitutes CSMP++’s main implementation phase and resulted in the CSMP++ beta version 4.0. In January 2001, Stephan left the ETH to become part of the faculty of Imperial College, London (UK). Since then he has made CSMP++ version 4.0 fully compliant with the C++ ANSI-ISO standard and improved its interface with the commercial finite-element mesh and fracture-geometry generators ICEM Tetra (ICEM Technologies, Inc.) and FRED (Golder Associates, Intl.).

In collaboration with Chris Pain and Matthew Eaton at Imperial, Stephan added an implicit FEM-FV-based multi-phase transport module to CSMP++. In 2003, together with Andrey Mezentsev and Ulrike Wolf, Mike Hohmeyer (ICEM Technologies, Berkeley) this scheme was generalized to hybride meshes of tetrahedral, prism, pyramid and hexahedral finite elements.

In the time period 1999-2003, Thomas Driesner and Sebastian Geiger at the ETH contributed an H2O-NaCl equation of state and a finite-volume-based IMPES multi-phase fluid-flow simulation module for triangular finite elements, respectively.

At Imperial, Andrey Mezentsev implemented a number of additional finite-element types and corresponding finite-volume stencils. This work was conducted in Phase 1 of the industry funded research project “Improved Simulation of Fractured Reservoirs”.

In 2005, Adriana Paluszny refactored Mezentsev’s implementation of the general finite-voume framework in CSMP++ and added fracture-propagation and other mechanical capabilities in the years to come, identifying a need to depart from a centralized variable and mesh storage towards a more flexible framework. At the same time, Dim Coumou parallelized CSMP++ using MPI and the SAMGp multigrid solver.

Following his departure from Imperial College, SKM rewrote the kernel of CSMP++, changing from array to single-object mesh and variable storage. Together with Philipp Lang, a code-wide application of policy-based class design was used to simplify user interfaces and make the code base more compact and maintable. This effort was accompanied by the creation of a rigorous framework for unit testing and code verification, implemention via a SVN – buildbot framework by Julian Mindel. The development of CSMP++ was discontinued and the new code CSMP++ was adopted as the platform for all new developments at the Garmisch workshop in 2011.

CSMP++ now is developed by an international group of developers, whose objective it is to design and implement all future additions to CSMP++ as compatible extensions of the basic CSMP++ libraries. The rationale for this is to achieve synergies that will arise when independent researchers and developers can share future developments. The primary goal of such developments is to make the package as widely applicable and accessible for research and teaching in the international academic community. Currently, CSMP++ serves primarily as a research tool for the numerical investigation of reactive (multi-phase) fluid-flow and heat transport processes in porous media with an emphasis on fractured rock. However, we also wish to apply CSMP++ to problems of commercial importance. The main current interest comes from the resource exploration, production and management sector of primary industries, but applications are foreseen in geothermal energy extraction, environmental management, and possibly others.

### Getting Started

The CSMP++ User Guide would be esoteric without programming examples but there are plenty. Please compile, run, and modify the tutorials and example files in the subdirectory /example_suite of the /trunk/source_code directory to get familiar with CSMP++’s functionality. This documentation is no substitute for this activity but it provides the necessary theoretical background to understand the exercises with the code. There is also a detailed DOxygen-formatted documentation (nightly built) of CSMP’s functions and classes accessible through CSMP’s webpage (thmc-project.unileoben.ac.at). In the document in hand, a top-down review of CSMP is organized as follows:

Chapter 1:  (this chapter):  Introduction and CSMP++ design history

Chapter 2: A single-phase flow simulation example.

Chapter 3: A steady-state fluid pressure computation example illustrating how a CSMP++ simulation is structured.

Chapter 4:  Tour of CSMP++; description of high-level classes.

Chapter 5: Introduction to finite-element computations and partial differential operators.

Chapter 6: Assignment of initial conditions to a model of transient flow.

Chapter 7: Building your own two-dimensional finite-element model, simulating it and exporting results to text files, JPEG images and the VTK visualization tools.

Chapter 8: A summary of all steps involved in running transient CSMP++ simulations: workflow is illustrated with a reactive-transport simulation of dissolution-precipitation of carbonate in a fractured limestone.

Chapter 9: Library documentation of the classes and underpinning principles of the code base covered in the preceding chapters.

Appendix. Class listings for the CSMP++ library components in Appendix A, installation advice and internet sites for support in Appendix B; final glossary of terms for newbies.

The documentation of the CSMP++ API is structured into four segments, the User Guide (this document), the Developer’s Guide, the Interface Guide, and the DOxygen Reference Manual (online). While the Users Guide only documents the high-level classes and their interdependencies and is intended for a front-to-back read of all new users and developers, the online documentation covers the entire code base.

The Interface Guide deals with the pre- and post-processing of models for meshing and visualization. It briefly introduces third party tools, both freeware and commercial ones. The Developer’s Guide describes the CSMP++ API in depth, making recommendations how customized applications are built using it as a basis. Many modern C++ programming tools now integrate class browsers which allow a user to quickly examine class hierarchies and public class interfaces. These will ease the transition to CSMP++.

The objects themselves are documented in the CSMP++ Reference Manual which should always be open in a Browser window while code is being developed.

The following second chapter describes the high-level object hierarchy of CSMP++ and explains the conventions adhered to in writing this documentation. Before you start to read please note:

- The usage of the C++ language imposes its terminology on CSMP++. One should be familiar with object-oriented programming including concepts like class and subclass, interface, method, inheritance, polymorphism, object instantiation, templates, operator overloading etc. to make effective use of this document. Consult a C++ book and the UML Reference Manual (Rumbaugh et al. 1999, Addison & Wesley) to learn about OOP basics. Use the CSMP++ Glossary following the Appendix in this document to familiarize yourself with the most commonly used expressions.

- For people used to procedural programming, perhaps the most important principle to appreciate in C++ apart from user-defined classes combining operations with associated data, is that such objects have a lifecycle. It begins with their construction also called instantiation, when a named “instance” of the object is created. The construction is performed by a constructor function that initializes the object’s internal data and storage. The lifecycle of an object ends with its destruction, when the memory that was occupied by the object is returned back to the operating system.

- The diagrams used in this documentation are all drawn using the “unified modeling language” (UML), including “class”, “sequence”, “interaction”, and “collaboration” diagrams. The overview chapter of the aforementioned UML manual is highly recommended reading.

- In class listings, combined capitalized nouns illustrate the convention for class names in CSMP++: In class or method names, all words are spelled out and chained together. Where two capital letters would meet, these are separated by an underscore. Function names, object instances, and variable names are written in small print. Classes that do not adhere to this naming convention, usually do not belong to CSMP++ (and the ‘csmp’ namespace).

- General C++ style and design guidelines that are adhered to in CSMP are described in Sutter and Alexandrescu (2004, C++ Prof. Series, Addison Wesley).

- In the text, class names are written in boldface when they are first introduced and subsequently typed in courier font.

## 2. From a Real World System to a CSMP++ Model

Numerical simulation aims at understanding the expression of a physical process or process interactions in a natural system too complicated to be described by analytical mathematical formulae. The goal is to find the spatially variable value of a dependent variable – like temperature or fluid pressure – in the entire region of interest. Typically, the value of this variable is already known in a few specific locations. This data is incorporated into the model as boundary and initial conditions. Spatially variable material properties determined in the field are prescribed to the computer model. This, however, leads to a range of simplifications relative to the real-world scenario and process. Modelling also requires a bit of housekeeping for the variables and results, and, most importantly, a computational scheme that accomplishes the calculations with a desired accuracy. Numerical simulation also necessitates monitoring of potential errors and it should implement appropriate recovery mechanisms. At the very least, CSMP++ the software must tell the user if and where something went wrong so that he or she can properly diagnose such errors.

The choice of the numerical method leads to additional requirements:

- Discretization of the real-world system that is continuous to the atomic scale means that – given runtime and memory limitations – it can only be represented at a few selected points that link up into computational cells. The number of these points, excluding those where the property that shall be computed is known already, defines the degrees of freedom (DOF) or unknowns of the problem of interest.

- A set of physical variables is selected to capture the behavior of interest in the real-world system. This also requires a mapping of material properties onto the discretization, for example, the assignment of permeability values to each mesh cell.

- Spatial and temporal coupling mechanisms are needed to relate dependent variables with one another. These typically are basic laws of physics expressed by partial differential equations (PDEs) that apply to the entire or parts of the problem domain and are represented in their finite-element form by the PDE_Integrator. Numerical element-by-element integration of the PDEs over the model domain leads to linear algebraic equations the solution of which gives the result of interest.

- Interrelations among physical variables that apply locally and without a need for global coupling mechanism(s), can be expressed in the smallest units of the discretization, the finite elements. For example, the hydraulic conductivity discretized on the mesh cell can be calculated from its permeability and a temperature and pressure dependent fluid viscosity that is discretized on the element nodes.

- Visitors allow for a change in reference frame (Eulerian = attached to the material, Lagrangian = attached to the flowing fluid) when sampling model behaviour.

- Essential conditions and initial values of the dependent physical variables must be prescribed to the model as a prerequisite for a unique solution and because a transient model requires an initial state. The term essential condition is preferred over boundary condition because corresponding assignments may also be made somewhere inside the model domain.

- Input and output methods which transform the results of a computation into a human-readable format such as graphics, charts etc.

The CSMP++ code is built in modules organized into hierarchical layers implementing these features and capabilities. The remainder of this chapter introduces the top-level objects of this hierarchy.

A simple box-shaped geologic model (Fig. 1) illustrates the basic terminology and object relationships in CSMP++. Terminology is introduced via the description of the unifying modeling process.

Fig. 1: Box-shaped CSMP++ model of a folded rock sequence illustrating how CSMP++ class names relate to model entities: The computational model (=geometry + connectivity + data) is stored by the Model object “folded layer model”. It is discretized by a finite-element mesh to which “permeability” and “porosity” values are assigned as material properties. Selected sub-regions of the model are identified by Region objects. Each region has a unique name given by a string. The Region object “sandstone” is contiguous but this is not a requirement for regions. Its distinction as a named region could, for instance, be used to monitor the fluid flux in this unit. The names “LEFT, BOTTOM, RIGHT” and “TOP”, distinguish straight model boundaries that are parallel to the coordinate axes. Boundaries do not have to be flat. If so they are flagged IRREGULAR.

The Model connects, manages, and stores a hierarchy of class objects that represent the finite element mesh and the associated material properties and physical variables (Fig. 2). Their specific functionality relates to the finite element method as will be explained in a later section.

Fig. 2: Class hierarchy that captures the discretization and connectivity of a CSMP++ finite element mesh. NB: Apart from the top down connections there are also flat and bottom up connections. Thus, nodes know their parent elements and elements know their neighbors.

The CSMP++ main_library implements the aforementioned modelling essentials as follows:

- The class Model combines objects with complementary functionality to encapsulate the computer model and associated variables. The mesh with associated proerties can be written to a CSMP binary file:

- In the live Model the finite-element mesh is handled by the class MeshManager. The finite-element mesh connectivity imposes the class hierarchy Model > Region > Element > Face > Node & ConstraintPoint, see Figure 2.

- The class FiniteElementManager associates Elements, Faces and InterFaces with finite-element capabilities. This achieved via the “Bridge” design pattern in which these objects are associated dynamically with the respective finite element types contained in the manager. A similar facility exists for finite-volume capabilities in the form of the FiniteVolumeStencilManager.

- A PropertyDatabase with Parameter records is used to generate access keys to physical variables (enum CSMP++_INDEX and class CSMP++_Index). Global and local handles to distributed variables exist in the form of Operand and  CSMP++_Operand classes.

- Material properties as well as physical variables are stored in the LocalVariableStorage. This storage is distributed (on the Nodes, Elements …Model) and custom-build at runtime using information from the PropertyDatabase and the MeshManager.

- Region objects are also stored and managed by the Model. It treats them as named entities managed by the RegionInterface. A distinction is made between unique (=space exclusive) regions and general ones that can be overlapping. Regions know their boundary elements, their faces, and nodes.  Elements that have at least one face at a boundary are boundary elements. Regions can be queried for their volume, surface area and so forth. Each model has a master region “Model” that contains all its elements.

- Boundary objects are lower dimensional regions that consist of Face objects. Any visitation, PDE solve or post-processing that can be performed on a Region can also be performed on boundaries.  Boundary Face objects know the higher dimensional elements on either side to which they connect. This allows for specific boundary processing.

- SplitBoundary objects are (and are created from) Boundary instances. They are different because they contain multiple yet space-coincident nodes that support decoupling and jump discontinuities in the values of node variables.

- Computations are performed with three classes of objects listed in an order of increasing complexity:

- Interrelation objects perform computations that lead to a  single result variable. Example: Computation of the interstitial flow velocity from the Darcy velocity and the porosity of the elements.

- Visitor objects are for more complex but still group-by-group, element-by-element, face-by-face etc. computations. Example: AlterationVisitor computes chemical reactions while looping over the elements.

- PDE_Integrator objects assemble finite-element and finite-volume integrals into a global sparse matrix and essential conditions into a right-hand vector. These integrals are defined in MathOperator subclasses. The solution of the matrix equations is possible with a variety of numerical methods made available as subclasses of the Solver class. Example: Solution of the Laplace equation with Fraunhofer’s SAMG solver.

- Input and output interfaces for the import and export of data to and from computations are contained in the CSMP++ interface library. They provide the glue necessary to use CSMP++ in conjunction with other software packages for CAD, meshing, analysis and visualization. Example: 2D results can be written directly to JPEG images. This is accomplished by interpolation to a FiniteDifferenceGrid and the association of variable values with a ColorPalette.

- Users can also resort to CSMP++'s internal tools to analyze computation results: Example: the StatisticalAnalyzer creates flow velocity histograms for each model subregion using user-defined velocity bins.

Figure 3 shows the high-level objects that a typical user will interact with in a simulation. Additional classes come into play for the input and output of the model and associated variables.

To extend, while reusing the functionality of this class hierarchy, a developer will create new subclasses inherited from the PDE_Integrator, Visitor, and Interrelation base classes. This is alluded to in Figure 3 by the display of a few subclasses that are already part of the CSMP++ API library (e.g., HydraulicHead interrelation, AlterationVisitor visitor). Such classes are documented in the library chapters of this User Guide.

The placement of variables on the mesh (see Fig. 2) is defined by the enumeration PLACEMENT (NB: enumerations are a variable type in C++):

enum PLACEMENT { NODE,

FACE,

FACE_INTEGRATION_POINT,

FACE_SECTOR_INTEGRATION_POINT,

FACE_FACET_INTEGRATION_POINT,

INTER_FACE,

INTER_FACE_INTEGRATION_POINT,

INTER_FACE_SECTOR_INTEGRATION_POINT,

INTER_FACE_FACET_INTEGRATION_POINT,

ELEMENT,

ELEMENT_INTEGRATION_POINT,

SECTOR_INTEGRATION_POINT,

FACET_INTEGRATION_POINT,

REGION,

BOUNDARY,

SPLIT_BOUNDARY,

MODEL,

Element properties permit the representation of material interfaces across which material properties vary discontinuously, i.e. from element to element (Fig. 4).

Fig. 3: Basic high-level base class relationships from a viewpoint of the Model object. The Model permits Visitors, Interrelations, and PDE_Integrators to act on its data. With the help of the PropertyDatabase it performs range checks and controls read and write privileges for physical variables. If range violations or other problems are detected, the ErrorHandler is notified. It will report errors in a platform-dependent way using the std::exception hierarchy and log them to file. Each executable needs a main()function written by the application programmer unless the QT graphical user interface (GUI) is employed. Results can be visualized with various complementary graphics tools like Paraview, Matlab etc..

Fig. 4: Two-dimensional triangular element discretization of folded layers illustrating node- and element numbering and the placement of node and element variables. In this example from a mechanical model, grey-shading indicates domains with elevated Young’s modulus.

Each Node in the finite-element mesh can be assigned a unique unsigned integer value to create a mapping to a solution matrix or vector. Such indices can be set and retrieved via the Idx() member functions.

To facilitate finite-element computations, the spatially distributed physical variables are associated with constraint flags that tell a PDE_Integrator which values shall serve as essential conditions and at which points modifiable variable values lie (so called degrees of freedom = DOF). These flags are 1-byte enumerations. Supported flag values are:

enum VARIABLE_FLAG { PLAIN,          modifyable, dependent or indep. var.

INIT_GUESS,     convergence oriented

INIT_COND, 	physically meaningful initial condition

DIRICH,         Dirichlet boundary condition

NEUMANN,        Neumann boundary condition

CONSTANT_FLUX, 	for prescribed flux

ROBIN,          combination of derivative and value

PERIODIC,       linked via ID to opposite side of model

FIELD_DATA,     (Geological) field data for comparison

ANY };          unspecified default discriminator

The default value of a VARIABLE_FLAG flag is ANY.

Note that at the same node, fluid pressure may be flagged as DIRICH while temperature is flagged PLAIN. For this reason VARIABLE_FLAG flags are associated with variables and not with nodes or elements etc.. Note also that CSMP’s high-level functions and methods (PDE_Operator, Visitors, Interrelations etc.) modify variable values only when the flag of the output variable matches that of the spatially distributed variable; else no alteration occurs. Thus, to assign a new value, val, while retaining the distributed variable’s original flag one writes:

object.Store( propertyKey, makeScalar(object.Status(propertyKey), val) );

Flags are enforced, for instance, when one assigns conditions explicitly, for instance by using the overloaded Model method  AssignBoundaryConditions(). This operation will overwrite variables irrespective of their flags.

As already shown in Figure 1, the discretized area or volume of the real world system called the model domain is represented by a Model delimited by Boundary  objects with unique names. For a box-shaped model these are predefined by capitalized tags. These C++ enumeration variables are declared as follows:

enum SG_BOUNDARY { NOT, IRREGULAR = IRREGULAR_OUTSIDE,

TOP       = TOP_OUTSIDE,

BOTTOM    = BOTTOM_OUTSIDE,

LEFT      = LEFT_OUTSIDE,

RIGHT     = RIGHT_OUTSIDE,

FRONT     = FRONT_OUTSIDE,

BACK      = BACK_OUTSIDE,

CNR1      = CNR_MIN,

CNR2      = CNR_MIN_MAXX,

CNR3      = CNR_MAX_MAXX,

CNR4      = CNR_MAX_MINXZ,

CNR5      = CNR_MIN_MAXZ,

CNR6      = CNR_MIN_MAXXZ,

CNR7      = CNR_MAX,

CNR8      = CNR_MAX_MAXZ,

EDGE1     = BACK_BOTTOM,

EDGE2     = BACK_RIGHT,

EDGE3     = BACK_TOP,

EDGE4     = BACK_LEFT,

EDGE5     = BOTTOM_LEFT,

EDGE6     = BOTTOM_RIGHT,

EDGE7     = TOP_RIGHT,

EDGE8     = TOP_LEFT,

EDGE9     = FRONT_BOTTOM,

EDGE10    = FRONT_RIGHT,

EDGE11    = FRONT_TOP,

EDGE12    = FRONT_LEFT };

Individual values are defined by integer constants using #define directives (see CSMP++_definitions.h header file).  The flags starting with CNR_ refer to the corners of box-shaped models. Position qualifiers follow, making reference to the coordinate axes of the model. The corner that is the farthest removed from the origin of the model is CNR_MAX_MAXXZ, i.e. CNR8 at max x, max y, and max z.

The boundaries to which these enumeration strings correspond in a box-shaped model are shown in the following figure:

Fig. 5: Boundary flags of a box-shaped CSMP++ model.

Figure 5 shows that the boundary flags also comprise labels for 4 corners in two-dimensional models or 8 corners and 12 boundary edges in three-dimensional models (Fig. 5). Two-dimensional models have the coordinate axis X and Y equivalent to the BACK side of three-dimensional models. The Y-coordinate points upwards. If the original model does not comply with this convention, it should be rotated before initialization involving gravity forces (CSMP’s gravity vector points in the negative Y-direction).

For box-shaped models, boundary flags are assigned with the Model’s methods AssignConstraint(), AssignBoundaryValues(), AssignBoundaryFlags(), AssignEssentialValuesToRegion()). All of these methods take a BOX_BOUNDARY enumeration as an argument. The numbering of the model corners is counterclockwise following the right-hand coordinate rule and starting from the lower right (xyz=0). If one tries to generate a model with irregularly shaped boundaries using default (box) constructors, boundary assignments attempted by CSMP++ will prompt warnings.

For irregularly shaped models, boundaries must be tagged either as IRREGULAR or created from lower-dimensional regions with a unique name. This is done outside of CSMP++ in the CAD system used to create the model geometry. CSMP tracks the CAD entities concatenating names on the basis of the volumetric regions that are joint at the boundary (example: SANDSTONE_SHALE). Outer model boundaries will contain the string “Boundary.”

Having established the variable placement, flags, and tags necessary for CSMP++-based finite-element computations, what is missing, is the description of- and the management of the physical variables themselves. This is the subject of the next subsection.
Management of Physical Variables

In CSMP, the management of physical variables begins with the generation of a text file by the user. This “*-variables.txt” file may just be a spreadsheet output as tab-delimited ASCII text. Columns are separated by tabs so that variable names can contain white space, like “fluid pressure”. This implies, that there must be no leading or trailing whitespace in any of the files columns. The name of the text file is communicated to the PropertyDatabase that lives inside of the Model via its constructur, for instance:

const string   model_name("one_sphere_0.45_tetra");

ANSYS_Model3D  model( model_name.c_str(), "example25.txt");

printModelDimensions( model, true );

The initialization is performed before the Model is instantiated. The file’s default name is ‘CSMP++_variables’. If the user does not specify any variables file, the PropertyDatabase will attempt to read a binary file with this name when it is constructed.

The variables text file enlists the specifications of each variable used in the computation. These are (see example line below):

- The name represented by a text string, e.g., “fluid  pressure”,

- The notation that relates the name to the symbols used in the underlying mathematical equations, e.g., P,

- The SI unit of the variable (unit conversion utilities are available through the interface of the PropertyDatabase), e.g., Pa

- An integer or string that identifies the enum VARIABLE_TYPE for the variable (scalar (=1), vector (=2), tensor (=3), >3 for any array where the number signifies its size. These types are represented by the classes ScalarVariable, VectorVariable, TensorVariable, Array and FlaggedArray, see next subsection. The strings (SCALAR, VECTOR, TENSOR, ARRAY, FLAGGEDARRAY) are preferred to the integer codes. Flagged arrays can only be specified as FLAGGEDARRAY# where the number indicates its size.

- A physical meaningful value range for the variable, typically specified in scientific notation; this range should be reasonably specific for the simulation of interest because it will be used to check calculation results before they are written back to CSMP’s distributed storage,

- The variable placement in the finite-element mesh as identified by the enum PLACEMENT (see above).

- Optional auxiliary information that relates the variable to a potential written documentation of the simulation. The headline of the variables file gives the column titles and determines how many columns are being read.

For example, an entry for the scalar node variable “fluid pressure” may look like this:

name	MParameter	unit	index	min.	max.	place	usage

fluid pressure	P    Pa   1  0.00E+00  1.00E+10 node  input

Definitions of physical variables occupy a single line each. The min. max. value used to indicate the value range of each variable is very important because it gives CSMP++ the possibility to check whether a calculation went wrong at its source, ascertaining that calculation errors are caught where they arise instead of being propagated. Beyond this, it is the responsibility of the developer of new modules to define ranges for input values of these so that constitutive relations or EOS are not applied outside of their range of validity. Such checks have to be performed inside the new Interrelations or Visitor classes. For classes that constitute the CSMP++ API library, legitimate input ranges are spelled out in the DOxygen documentation.

Integration point variables have a special role: There is no one-to-one correspondence to a class for them, but they either belong to finite elements or finite volumes. How many of these exist has to be queried from instances of these entities, for example:

const size_t ip = element_ptr->IntegrationPoints();

This code will return how many integration points the element contains that the pointer points to.

In finite-element computations, dependent variables usually live on the nodes. Integration point variables (*_INTEGRATION_POINT) come into play, for instance, if variables vary discontinuously from element to element, like the tensor variables “stress” or “strain.”  In this case, values obtained by post-processing are only exact at the Gauss points and storing them on the nodes would average values in a physically meaningless way. As another example, a fluid-flow velocity vector variable stored at a material interface node would be problematic because the interface refracts the flow direction.

The specifications of physical variables are stored as Parameter records in the PropertyDatabase inside the Model object. When the PropertyDatabase is first initialized all variable names are listed on screen in alphabetical order. The database is then written to the binary file “CSMP++_variables.dat that is subsequently used when loading models in CSMP binary format. A simulation can also be conducted creating all variables at runtime and without storing any of them to file. This is done by calling a constructor of the PropertyHandle class, see examples.

At runtime, the main role of the PropertyDatabase is to return variable keys (instances of the csmp::Index struct) needed to access variable flags and values:

csmp::Index p_key = model.Database().StorageKey(“fluid pressure”);

…

Region<DIM>& rref=model.Region(“folded layer”);

for ( auto it=rref.NodesBegin(); it!=InteriorNodes(); ++it)

{

// retrieving a pointer to the current node

double64 p_value = (*it)->Read( p_key );

the PropertyDatabase can also be queried for specific variable (Parameter) records or simply to find out whether a variable is already defined as is shown in this code example:

// write the specifications of an existing variable to screen

Model.Database().FlushToScreen(“fluid pressure”);

// test whether ‘temperature’ is defined

if ( model.Database().IsDefined(“fluid pressure”) )

// we can use it…

The variable keys (Index instances) are used inside the Model to efficiently write and retrieve variable values (Fig. 6). ScalarVariable - FlaggedArray objects are inlined and stored inside of LocalVariableStorage objects that are data members of the Node – Model. Due to differences in their placement and type the memory required to store them differs. The corresponding amount of used RAM can grow or shrink at runtime when variables are created or deleted inside of CSMP modules. The local storage  provides property-access via its Status(), Read(), Store() interfaces:

// accessing variables

VARIABLE_FLAG condition = node_ptr->Status( P_key );

double64 v_min(1.0e-7), v_max(1.0);

bool in_correct_range = element_ptr->IsWithinRange( v_key, vmin, vmax );

// initialing tensor variable stored on element integration point variable

TensorVariable<2>  ts(ANY,1.,0.,0.,1.);

element_ptr->Store( ip, ts );

// starting at the end of a previous run

model_time  = model.Read( global_time_key );

Fig. 6: Steps involved in accessing physical variables stored in the Model, using string identifiers such as “permeability” and interfaces of the MemoryManager class. The comparably slow operation of finding a variable in the database is carried out only once in each call to a Model, Visitor, or Interrelation object. The MeshManager inside the Model grants access to the finite elements, constraint points, and nodes constituting the model.

There are three ways to access and modify variables:

- By name, e.g., “fluid pressure”

- By CSMP++_INDEX, a structure with the members placement, type, and property index (Fig. 6, and example above).

- Via Operand or CSMP++_Operand class objects which one can associate with an existing variable, or use to define or remove a variable at runtime.

The first way is used in all the high-level interfaces of the Model, PDE_Integrator, Visitor, Interrelation and other class objects with which a user will interact. Behind the scenes, the property database issues a CSMP++_INDEX for each variable that has been queried to rapidly carry out ‘Read()’ and ‘Store()’ operations. The third variable access strategy is described in the section on CSMP++_Operands.

### Data Types: Scalar, Vector, Tensor, Array, FlaggedArray.. -Variable

Each physical variable registered by the PropertyDatabase is stored in inline format in the model in an STL vector of a suitable type (values as double64, flags as one-byte enums). Implemented variable classes are ScalarVariable, VectorVariable, TensorVariable, ArrayVariable and FlaggedArrayVariable. Their default flag value is ANY. Corresponding enumerations used for their initialization are contained in the C++ header file “VariableEnums.h”. Here, the enumeration VARIABLE_TYPE is defined as.

enum VARIABLE_TYPE { SCALAR=1, VECTOR=2, TENSOR=3, ARRAY=4, FLAGGEDARRAY=5 };

To insure portability of the CSMP++ code between platforms with a different numerical precision, there is a stand-alone program ‘CSMP_number_types_main.cpp’ whose execution establishes the implementation- and machine-dependent precision of C++ inbuilt-variable types on the target platform. With the target compiler and settings, compile this program into an executable and run it. The output is the header file  ‘CSMP++_number_types.h’ whose automatic inclusion when the CSMP++ library is built establishes prescribed-size cross-platform compatible types. The default floating point type used by CSMP++ is double64 and has 8-byte precision. CSMP++ initializes each floating point value to a default value of NaN (not-a-number, see IEEE conventions). This approach highlights any uninitialized variable value as calculations involving it will also return NaN. Such values can also be tested for by the macro is_nan().

C arrays and STL random access containers like the vector and deque templates use integer types to access individual elements:

// create an instance of an STL vector with 200 CSMP++_float elements

vector<double64>  v(200,13.3);

cout << v[150] << endl;

The range of these integers depends on machine architecture. CSMP++ uses the unsigned integral type size_t in stead of the container-specific ::size_type to access data in these containers. It is normally typedef’ed as an unsigned 32-bit integer. Note that to avoid implicit type conversions the size of size_t must be consistent with that of the STL size_type parameter.

ScalarVariable, VectorVariable, and TensorVariable are partially specialized template classes that associate VARIABLE_TYPE double64’s with VARIABLE_FLAG enums (see above). Operations involving them and the inbuilt types have been optimized and overloaded operators allow the user to write mathematical expressions without a need of the usual low level expansions. For example, matrix multiplication is carried out automatically (for the following vector and tensor variables of a three dimensional model:

TensorVariable<3>  A, B;

VectorVariable<3>  n, u;

…//do some initializations of 3 x 3 matrices A and B as well as vector n;

TensorVariable<3>  C = A * B;

u = C * n;

The dimensions of vector and tensor variables must always be equivalent to the dimensionality of the model. A TensorVariable<2> will contain a 2 x 2 matrix of ScalarVariables.

The public interfaces of the basic data type classes are documented in detail in the DOxygen online reference.

As the name indicates, the ArrayVariable variable stores a linear vector of doubles, the size of which is chosen by the user. While there is just a single flag for this type, in the FlaggedArray there are as many flags (of VARIABLE type) as there are floating point numbers.

## 3. Computation of a Steady-State Fluid Pressure Distribution

The previous chapter explained how a discretized model geometry with associated material properties and physical variables is stored and managed by the Model class and associated CSMP++ modules. How this Model object interacts with other CSMP++ objects is illustrated in this chapter using an example program that calculates a steady-state fluid pressure distribution in an aquifer (Fig. 7). The steady-state fluid-pressure distribution that ensues as a consequence of spatial permeability variations and fixed boundary pressures (Dirichlet boundary conditions) is governed by the partial differential equation

..............................................................(1)

p, k, , and Q are the fluid pressure, the permeability, the dynamic viscosity of the fluid, and a fluid source term, respectively. Once this pressure equation has been solved, the piecewise constant Darcy velocity, q, discretized on the element is obtained by post-processing of the piecewise constant fluid-pressure gradients. For this calculation the hydraulic conductivity, K (K = k/) is introduced:

.........................................................(2)

The solution to the pressure equation is computed via a piece-wise integration over the finite elements of the finite-element form of equation 1, yielding the incremental change in fluid pressure over the domain of interest. Resulting volume integral terms for each finite element are assembled into a matrix equation of the form:

..............................................................(3)

The left-hand side of this system of algebraic equations contains element contributions to the global conductance matrix K; p is the solution vector which will hold the nodal fluid-pressure values, and the right-hand vector q stores the fluid source terms and boundary conditions. This global system of linear equations has a size determined by its degrees of freedom (DOF). The DOF count is equivalent to the number of nodes at which fluid pressure must be computed. This number is equal to the total number of nodes in the model minus the Dirichlet nodes at which the fluid pressure is prescribed. Dirichlet nodes impose fixed coefficients on the other algebraic equations constituting the solution matrix.

The element contributions to the global system of linear algebraic equations consist of element conductance matrices K(e) which are added to the empty global matrix K (equation 3). For (two-dimensional) linear triangular finite-elements, element conductance matrices have the form:

.............................(4)

where N1-3 are three element interpolation functions (one for each node), used to approximate the fluid-pressure variation over the element. These functions are listed in the documentation of the finite element library. A is the area of the element, and Kx,y is the directional hydraulic conductivity. Kx,y is constant within each element but is allowed to vary from element to element. In the example, Kx,y is isotropic such that Kx = Ky.

The global conductance matrix K is accumulated and solved by passing a PDE_Integrator object to the model. The integral in equation (4) is generated in matrix form by the finite-element PDE operator Integral_dNT_op_dN_dV. It is a subclass of the generic differential operator MathOperatorLHS. Mathematical operator names follow special naming conventions that were introduced because of a need for brevity (see Table 1). These names contain placeholders for physical variables. In this example, op is used represent the hydraulic conductivity and the N functions model the fluid pressure. They appear twice, first as a placeholder for the element weighting functions and, second, for the element interpolation functions. This is the case because in the Galerkin finite-element formulation used in this example, weighting and interpolation functions are the same.

| Notation | Spelled Out | Explanation |
| --- | --- | --- |
| Integral | analytical integral | exists only for straight-sided finite elements |
| NumInt | numerical integral | for arbitrarily shaped elements with integration points |
| N | FEM test functions | finite-element method, also called basis or interpolation functions |
| M | FV test functions | finite volume interpolation functions, typically piece-wise constant |
| T | transposed | used in conjunction with interpolation function vector or derivate matrix |
| dV | volume integration | normally by finite-element method |
| dS | surface integration | normally by finite-volume = control volume method |
| op | operand | normally the material property variable like permeability |
| lhs | left-hand side | contribution goes into global solution matrix |
| rhs | right-hand side | contribution goes into global right-hand vector |
| 2 | exponent | dN2 is the second derivative of the N functions |
| B | dN expanded | used if solution variable is a vector as opposed to a scalar |
| P | N expanded | solution variable is a vector property like displacement |

Table 1.: Naming conventions used for PDE operators that represent the finite element of finite volume integrals over partial differential equations (MathOperatorLHS and MathOperatorRHS objects).

Element contributions to the right-hand vector have the form:

In this example, s(e) is the fluid source term per unit area (m3 m-2 s-1), which could be envisaged as the rainfall on the land surface represented by the element or as the amount of fluid produced from a well. The factor 3 appears since the source term is distributed over the 3 nodes of each triangular element. Q(e) terms are accumulated by the Integral_NT_op_N_dV derived from the differential operator MathOperatorRHS base class.

model.Apply( evolve_pressure );

Model__

Fig. 7.: UML activity diagram for the simulation example1: calculation of a steady-state fluid pressure distribution.

In this simulation example, an adaptively refined finite-element mesh created by the program Triangle is used. It represents a rectangular domain (see CSMP++ Model Building Guide to learn about mesh creation). When the PDE_Integrator is applied to the Model it calculates the Darcy flow velocity from the computed fluid pressure gradients and the hydraulic conductivities of the elements (see equation 2). This is done by the post-processing operator VelocityAndVolumeFlux. In the example, the results of the computation are output as JPEG images (Fig. 6), text files, VTK files, and GoCad formatted TSurface files.

// inclusion of class declarations necessary for this simulation

#include "CSMP++_definitions.h"

// example_main file

#include "CSMP++_Examples.h"

// main library

#include "Model.h"

#include “ErrorHandler.h"

#include "CSMP_highLevelUtilities.h"

// pde operator library

#include "Integral_NT_op_N_dV.h"

#include "Integral_dNT_op_dN_dV.h"

#include "VelocityAndVolumeFlux.h"

// solver library

#include "SAMG_Solver.h"

// interrelations library

#include "ConductivityFromPerm.h"

// interface library

#include "Triangulator.h"

#include "CSMP++_JPEG_Interface.h"

#include "CSMP++_VTK_Interface.h"

using namespace std;

using namespace CSMP++;

// ***********************************************************************************

// (1) 2D steady-state calculation of fluid pressure distribution using triangular

//     element meshes generated by CSMP++'s Triangulator from color-coded bitmaps.

//     Linear finite element interpolation is used and Dirichlet essential

//     conditions  are applied.

//

//     Use 'granite_model1' as input geometry (needs 100MB of RAM memory)

//

//     Use example_viewer.tcl or MayaVi to visualize vtk output

//

// ***********************************************************************************

namespace CSMP++ {

int steady_state_pressure()

{

// 0. Setting global variables

// ---------------------------

// singletons (exist only once so that only a reference can be obtained)

ErrorHandler& csmp_error ( ErrorHandler::Instance() );

double64&     global_time( ModelTime::Instance().modelTime );

global_time = 0.;

const string  global_physvar_textfile("example1.txt");

// 1. A Model named "example1" is built

//    from a mesh based on a color coded image

// -------------------------------------------

Triangulator            mesh_triangulator;

Model<CSMP++_float, 2>  example1( mesh_triangulator );

const PropertyDatabase& p_ref = example1.Database();

// 2. Assignment of material properties & initial conditions

// ---------------------------------------------------------

example1.InputUniformScalarValue( "porosity",            0.1 );

example1.InputUniformScalarValue( "fluid volume source", 0.0 );

example1.InputUniformScalarValue( "fluid pressure",      0.0 );

// 3. Assignment of Dirichlet fluid pressure conditions

//    at top and bottom boundary of the model

// ---------------------------------------------------------------

example1.AssignBoundaryValues( TOP,    "fluid pressure", DIRICH, 1.0e+05, 1.0e+05 );

example1.AssignBoundaryValues( BOTTOM, "fluid pressure", DIRICH, 1.0e+07, 1.0e+07 );

// 4. Calculation of hydr. conductiv. from permeability using Interrelation subclass

// ---------------------------------------------------------------------------------

CSMP++_float fluid_viscosity(1.0e-03);

ConductivityFromPerm<double64>  conductivity( p_ref, fluid_viscosity );

example1.Pass( conductivity );

printRangeOfVariable( example1, "conductivity" );

// 5. Building PDE_Integrator "total_pressure" with the PDE operator subclasses

//    A post processing operation is added to compute the fluid velocities and

//    fluid fluxes (by setting the boolean variable in the constructor of the post-

//    processor to true, the element velocities are averaged to the nodes)

// ---------------------------------------------------------------------------------

SAMG_Solver  solver;

PDE_Integrator<2U, Region>  total_pressure;

total_pressure.SetSolver(solver));

// conductance matrix [K] on the left-hand side

Integral_dNT_op_dN_dV<double64>    conductance( p_ref, "conductivity",

"fluid pressure",  "fluid pressure" );

// source vector {Q} on the right-hand side

Integral_NT_op_N_dV<double64>      source( p_ref,  "fluid volume source",

"fluid pressure" );

// post-processing operation to compute flow velocities

VelocityAndVolumeFlux<double64>    velocity( example1,  "conductivity",

"porosity", "fluid pressure", true );

// add PDE operators to FE PDE_Integrator

total_pressure.Add( &conductance );

total_pressure.Add( &source );

// add post-process to the FE PDE_Integrator

total_pressure.AddPostProcess( &velocity );

// Solve [K]{p} = {Q} by passing the PDE_Integrator to the model

example1.Apply( total_pressure );

// 6. Screen ranges of computed variables

// --------------------------------------

printRangeOfVariable( example1, "fluid pressure" );

printRangeOfVariable( example1, "velocity" );

printRangeOfVariable( example1, "pore velocity" );

printRangeOfVariable( example1, "volume flux" );

printRangeOfVariable( example1, "nodal volume flux" );

// 7. Output results to VTK and JPG files

// --------------------------------------

CSMP++_VTK_Interface<CSMP++_float,2>  vtk_output;

// VTK output

vtk_output.OutputDataToVTK( example1, "fluid-pressure", "fluid pressure",    0 );

vtk_output.OutputDataToVTK( example1, "velocity",       "velocity",          0 );

vtk_output.OutputDataToVTK( example1, "nvelocity",      "nodal velocity",    0 );

vtk_output.OutputDataToVTK( example1, "volume-flux",    "volume flux",       0 );

vtk_output.OutputDataToVTK( example1, "nvolume-flux",   "nodal volume flux", 0 );

// JPG output (only scalar variables can be visualized)

CSMP++_JPEG_Interface              jpg_output;

jpg_output.OutputDataToJPG( example1, "fluid-pressure", "fluid pressure",    0 );

jpg_output.OutputDataToJPG( example1, "volume-flux",    "volume flux",       0 );

// 8. Form a Region named "granite" in order to access

//    intrusion in the center of the model.

//    Region granite is build from finite elements in

//    the permeability range between 1.0e-21 to 1.0e-18 m2

// -------------------------------------------------------

cout << "\nFORMING A GROUP CALLED 'granite'" << endl;

example1.FormAndAddGroup( "granite", "permeability", 1.0e-21, 1.0e-18 );

// add fluid volume source term to region granite and repeat computation

const CSMP++_float source_term(1.0e-13); m3 m-2 s-1

example1.ChangePropertyInGroupToWhere( "granite",

"fluid volume source", source_term, PLAIN );

example1.ChangePropertyInGroupToWhere( "granite",

"fluid volume source", source_term, BOUNDARY );

// fixing pressure at perimeter of group

example1.ChangePropertyStatusInGroupToWhere( "granite",

"fluid pressure", DIRICH, BOUNDARY );

// restricting PDE_Integrator to group

total_pressure.RestrictApplicationTo("granite");

// recomputing fluid pressure in the granite taking into account the source term

example1.Pass( total_pressure );

printRangeOfVariable( example1, "granite", "fluid pressure" );

printRangeOfVariable( example1, "granite", "velocity" );

printRangeOfVariable( example1, "granite", "pore velocity" );

printRangeOfVariable( example1, "granite", "volume flux" );

printRangeOfVariable( example1, "granite", "nodal volume flux" );

jpg_output.OutputDataToJPG( example1, "fluid-pressure", "fluid pressure", 1 );

jpg_output.OutputDataToJPG( example1, "volume-flux",    "volume flux",    1 );

// output of the group "granite"

vtk_output.OutputDataToVTK( example1, "granite",

"fluid-pressure", "fluid pressure", 1 );

vtk_output.OutputDataToVTK( example1, "granite", "velocity", "velocity", 1 );

cout <<"\nmain: That's it..."<< endl;

return 0;

} // end example1 function

} // end namespace CSMP++

The example source code displays several key features, which are germane to any C++ object-oriented program:

- Comment lines start with a double slash (//).

- Functions typically have a return value which is output when the instructions in the body of the function enclosed in “{ }” have been executed.

- For all objects which are used in the main program, corresponding header files must be included into the source code file. This is done at the very beginning, before the function which uses these object begins. These header files contain necessary object declarations. The actual functionality of the objects is defined (implemented) inside the binary library files.

- Each object must be constructed by a constructor call before it can be used (for example the CSMP++_VTK_Interface object in program segment 5).

- Constructors without arguments are called default constructors, see for example the construction of the jpg_output object.

- Object construction implies that an instance of a C++ class is created. This instance has a unique name independent of the class name. Therefore, the construction process is also called instantiation.

- Before global objects or variables can be used in a function, C++ requires these to be declared using an extern statement. This statement just tells the compiler that there is global instance of a class x with the instance name y. Again, the class must be declared in a corresponding header file.

- Once an object has been instantiated, it can be manipulated by calling its interfaces. Interfaces are functions which only work in conjunction with that object. In C++, this is signified by the scope operator “.”. Hence, the statement dog.bark(loud), tells the instance dog of a class representing dogs to invoke its member function bark(). The function argument loud could be double giving a value in decibels.

- Objects may also be linked via pointers. If so, connections may be changed at runtime. The syntax “->” is used instead of the scope operator, i.e. dog->bark(loud) w.r.t. example above.

- Member functions are normally called methods to distinguish them from stand-alone functions.

- There are no destructor calls in main(). Unless an object is created dynamically (using the new operator, see creation of SAMG_Solver in example above) its destructor is called automatically at the end of the main program. Dynamic objects are destructed by calling the delete() function.

In the example program, the blocks 1 to 10 have the following roles:

- The (Model) model is constructed from an image with color-coded pixels defining its geometry. The mesh is created by CSMP++’s Triangulator mesher. This tool is described in more detail in the CSMP++ Model Building Guide. The user is then prompted to specify the physical dimensions of the model.

- The input data describe the geometry of the model and the material property “permeability” whose local values are determined by the the pixel colors. Initial values for the fluid source term etc. must still be assigned to the mesh. By default, all variables specified in the “...variables.txt” file are initialized to a status of PLAIN and a value of NAN (not a number). One can immediately tell if one forgot to initialize a variable, because the result of calculations involving such a variable will also be NAN.

- Fixed fluid-pressure values are assigned as Dirichlet boundary conditions to the right and left model boundaries.

- The interrelation ConductivityFromPerm is instantiated and applied to the model (calling the Pass() method of the Model class), to calculate the hydraulic conductivity from the permeability and the fluid viscosity of water at room temperature.

- The PDE (equation 1) is assembled and solved using an algebraic multigrid solver with the PDE_Integrator instance total_pressure. This includes post-processing of the flow velocities by the VelocityAndVolumeFlux class and the velocity magnitude is stored for later display as the scalar variable “volume flux”. To be able to visualize the volume flux with the visualization toolkit, this property is extrapolated to the nodes and stored in the node variable “nodal volume flux”.

- The result variables are output to screen to examine their minimum and maximum values.

- The computed “fluid pressure” and “volume flux” are output as JPEG images which associate colors with variable values and to VTK files for the MayaVi visualization tool. The latter files store the actual variable values and, in contrast to the JPEG format, the vector quantity (Darcy) “velocity” can be visualized.

- Finite elements with a permeability between 10-21 and 10-18 m2 are included in a group called “granite”.  A fluid volume source term is assigned to the interior and the marginal elements of this “granite” group and the variable fluid pressure is flagged Dirichlet at its boundary. Now the algorithm is restricted to apply only to this subregion of the model and fluid pressure is recomputed using the fluid pressure values at the group’s perimeter as Dirichlet boundary conditions. The results from this computation are output selectively for the granite group.

Fig 8: Cross-sectional model of a granite intrusive body in a folded and faulted rock sequence (file: granite_model1). The colors in the upper left image correspond to the log10 of permeability. Upper right: fluid pressure distribution ensuing from fixed upper and lower boundary pressures. Rainbow shading of variables (Red colours represent high and blue colours low values). Lower left:  volume flux is highest in the fault that breaches the (yellow) low permeability layers. Lower right: fluid pressure after a region-restricted computation in which a fluid source term was assigned to the (red) granite intrusion.

Summary. A CSMP model (Model) contains the finite-element mesh (MeshManager), a property database enlisting the physical variables in use (PropertyDatabase), an object that keeps track of the current finite element types (FiniteElementManager) as well as containers of the model regions and boundaries (RegonInterface etc.). Properties are discretized on such regions, their nodes and elements and their integration points. They are stored locally in LocalVariableStorage objects. Geological entities in the model are discerned as Region objects (Region->ModelSubmain).

Data access and modification occurs through the public methods of the Model (Fig. 6). Key steps are that the property database is queried for the specifications of each variable identified by a string (for instance "permeability"). The database then returns a struct csmp::Index that uniquely identifies the variable. This index is used for Read() / Store() / Status() operations overloaded to retrieve Scalar-, Vector-, Tensor and Array variables. Within the LocalVariable storage, the variables live in STL vectors whose size varies with variable placement. Typically, a triangular/tetrahedral mesh will contain many more elements than nodes. Memory to store variables is allocated upon demand according to the specifications in the property database that is usually initialized before a model is build using the specificatons of variables in the variables text input file. Even if it is not used in a calculation, every variable in the database requires storage.

Computations are carried out with PDE_Integrator that performs the assembly of partial differential equations and integrates them with Solver classes. The Model::Apply() interface applies PDE_Integrators and Interrelations to itself.  Interrelations compute local relationships between variables. Any PDE_Integrator, visitor or interrelation object can be selectively applied to model subregions or boundaries.

## 4. A Tour of CSMP++

The following subsections provide a first look at the high-level CSMP++ classes which a user or developer is likely to interact with. For documentation of their individual methods (=member functions), consult the CSMP++ DOxygen documentation.

### Region

The aforementioned Region class implements selective access to user-named subregions of a model including itself via a region named “Model”.

Region objects are accessed by name and  their elements via iterators.

Region<3>& target = model.Region(“upper sandstone”);

for ( auto it=target.ElementsBegin(); it!=target.ElementsEnd(); it++ )

(*it)->Out(); // writes state of element to console

Each region object consists of vectors of pointers to its Element and Node objects. Each of these vectors is sorted into two ranges. The first range contains the interior- and the second range the perimeter elements of the region. The same applies for the node vector. Perimeter elements and nodes make up the external boundary of the region (Fig. 9). Importantly, elements are considered perimeter elements only when they have at least on face (edge in 2d) on the region boundary.

Being able to distinguish the INTERIOR from the PERIMETER of a COMPLETE region via enums is useful because this permits quick determination of respective properties and the selective search for and assignment of property values. Important: Region objects have their own property value storage (LocalVariableStorage). This is used, for instance, to associate constitutive relationships with model regions to which they apply. Also uniform property values can be stored there, rather than on each individual element of the region.

Fig. 9: Region flags distinguishing different parts of the region. Elements that classify as located at the boundary=PERIMETER are shaded in gray. They must at least share one face on the boundary.

For each perimeter element, there is a record of the face(s) that is(are) located at the region boundary. It is stored in separate boundary face vector to be used in operations targeted on the faces of a region perimeter.

Regions do not have to represent contiguous model domains. They can also contain numerous disconnected entities with similar properties; for example, all the fractures in a block of rock, see Figure 10. Such discontiguous regions can be partitioned into contiguous subregions with Model methods like:

PartitionRegionIntoContiguousSubRegions( const char* );

Fig. 10: Region types in a CSMP++ model.

This method creates numbered subregions that are all contiguous.

Regions can be created flexibly on the basis of user-defined selection criteria. The smallest possible region contains at least one Element object. Thus, a region cannot consist only of nodes or integration points.

Normally, a region consists of equi-dimensional elements. There are special cases, however. For example, a volumetric region that contains surface mesh that sticks out of its boundary represents such a case. If so, all these exterior surface elements and their nodes are treated as perimeter entitities.

When a geomodel is input into CSMP++ its subdomains become non-overlapping = unique regions of the Model. However, the user also has the option to build regions on the fly using a wide variety of contruction criteria. The most widely used ways to build regions are:

- From a geometrical entity pre-defined in a CAD input model; this information is communicated to CSMP++ via a ModelTopology object automatically when the Model is constructed. To do it manually use FormRegionsFrom()).

- From elements within a certain property range; see example in the previous chapter (call FormAndAddRegion() or RegionsFromPropertyValues() to create multiple regions),

- From a combination of property values specified in a PropertyConstraints object,  see example below,

- As a rectangular / box shaped region with specific coordinates (use FormAndAddRectangularRegion()). This region will only have smooth boundaries if the mesh has appropriate element shapes.

- From a list of elements specified by the user (use FormFromElements()).

- By copying or carrying out Boolean operations on pre-existing regions: union, difference, intersection etc.

The use of a PropertyConstraint object delivers the highest degree of flexibility for region selection. This is illustrated in the following example where a region is build from elements that have a permeability below 10-18m2 and are subjected to a pressure and temperature above 100MPa and 400oC, respectively:

PropertyConstraints  constraints("permeability", 1e-21, 1e-18 );

constraints.AddConstraint( "fluid pressure", 1e8, 1e7 );

constraints.AddConstraint( "temperature",    800., 450. );

// relaxing the inclusion criterion from all to at least one node

constraints.SatisfyConstraintsForAtLeastOneNode( true );

model.FormAndAddRegion( "element group", constraints );

Due to the differences among the region creation methods and apart from region contiguity, two types of regions will result from creation: 1)  “unique” ones without any overlap that will, in most cases, cover the entire model volume in terms of subvolumes like in a CAD model; 2) “non-unique” ones that may overlap with one-another as in the case above where the region “element group” was created using the PropertyContraints object. CSMP keeps such regions in two separate containers allowing its users to access them selectively. This is important, for instance to avoid that the overlap of non-unique regions may create unexpected behaviour during REGION property value assignment.

Lower-dimensional regions can be unique although they share their nodes with higher dimensional ones.

The region “Model” always exists, but will be contained in the unique regions container if there is no other unique region.  If region “Model” is not alone, however, it will overlap any other unique region. In this case it is therefore stored in the non-unique regions container.

Starting with any type of region, additional ones can be created using boolean algebraic operations. This capability includes region intersection, difference, symmetric difference and union. Using this capability, one could, for instance, find a  subregion of a shaly layer which is faulted:

// form new group from the intersection of a shale and a fault group

intersection( “shale”, “diffuse faults”, “faulted shale” );

Regions are ideally suited for selective monitoring of variables over the course of a simulation. This is achieved with the RegionMonitor class:

string file_name("monitored-regions.txt");

list<string> integral_properties;  integral_properties.push_back("saturation oil");

list<string> range_properties;

range_properties.push_back("fluid pressure");

range_properties.push_back("saturation oil");

RegionMonitor<2>  monitor( model, integral_properties,

range_properties );

// monitor all regions at this particular time level

monitor.ScalarPropertyRanges( model, global_time );

monitor.ScalarPropertyIntegrals( model, global_time );

// writing the current level of chronologically ordered data to file

monitor.Out( file_name.c_str() );

As already shown in the steady-state fluid pressure example in the previous chapter, computations can be restricted to regions. Operations that can be carried out on regions include calculations of single variables (via Interrelation objects), multiple variables and constitutive relationships (via Visitor objects), and the integration of partial differential equations (via PDE_Integrator objects). The interface of the Regin class also permits the modification of distributed variables and their flags. This rich functionality of regions is further illustrated in the example application Region_Example (Region_Example.h).

A special flavor of the Region class is the NimbleRegion. This light-weight region class can be constructed more rapidly and is optimized for shape modification over time. It also supports the IntegrateOver() interface of the PDE_Integrator class.

### Boundaries

Model boundaries can be regarded as special regions that have a lower spatial dimension than the model as a whole. This comprises the side boundaries of the model, material interface in a volumetric model represented by surface elements, and or wells in three-dimensional models represented by a chain of line elements. Computations involving boundaries are possible. When a mesh is imported into CSMP++ from another application, CSMP++ has to be told which of the lower dimensional elements that this mesh contains should be treated as boundaries. This decision has to be made by the user or conveyed via region names that contain the string BOUNDARY.  When prompted this way, CSMP++ will convert a corresponding Region object into Boundary object, deleting the original region.

A Boundary is defined as a (dimension – 1) Region, sharing much of its functionality with the latter through the common base class ModelSubDomain (look at this base class for key Region or Boundary functionality). Thus, boundary objects also contain sorted ranges of elements and nodes and have a perimeter, which is an edge curve in 3D models, but reduces to the two end points of a line boundary in a two-dimensional model.

The other important difference between Boundary and Region is that it consists of Face objects as opposed to elements. FACE is a different variable placement than ELEMENT and element variables therefor are only available from the higher-dimensional neighbor Element objects of the Face objects. By constrast to the Element, however, the Face knows its higher-dimensional neighbors and neighboring Element objects are readily accessed (Element<dim>*  Parent( INTERFACE_SIDE ) const).

When a boundary is located on the outside of a model, each face only has a single higher dimensional neighbor element. This always is the INSIDE neighbor located on on the opposite side of the direction to which the normal of the Face points. This way everything is consistent: the Face normal and the outward pointing normals of the Element, which is covered by the Face.

For box-shaped models introduced earlier (see Box.h), Boundary objects can be constructed, extending the functionality of the BOX_BOUNDARY flags (Fig. 5). By contrast with flagged-only box models, one with Boundary objects will contain boundaries labeled TOP, LEFT…EDGE# so that computations can be performed on them. Configuration and boundary condition assignment to these models is explained in a later section on the InputDataManager class.

As discussed earlier, CSMP++ has a range of input interfaces which allow the construction of a csmp::Model from an  imported mesh, including region and boundary objects. The convention adopted in this context is that, beyond box-shaped models, during model construction, any (model dimension-1) region whose name contains the string “BOUNDARY” will be automatically converted into a Boundary object. In the “establish boundary” process, per default, the original Region object and all its elements will be removed. The Element objects get replaced by new (dimension-1) Face objects with the aforementioned extra functionality: they have a unit normal that points outside of the higher-dimensional Region that they surround. They also have connectors to the higher-dimensional “parent” Element objects on either side (method Face::Parent(INTERFACE)). However, when a Boundary is located on the perimeter of the “Model” region, the unit normals of its Face objects point into open space and the Face objects only have INSIDE parent elements.

Note that BOUNDARY and FACE constitute unique variable placements. If variables are placed there, storage for them is allocated only where boundary objects exist and their names ought be accordingly specific; for instance “basal heat flow” prescribed in W/m2. If the computation of boundary variable values involves variables placed on the Element, these are readily fetched from the parent elements of the faces via the aforementioned pointers:

csmp::Index element_property=model.Database().StorageKey(“permeability”);

for ( auto it=boundary.ElementsBegin(); it!= boundary.ElementsEnd(); it++ )

{

double k = (*it)->Parent(INSIDE)->Read( element_property );

...

Note that the Face class is not inherited from the Element. In stead, both share FiniteElementPolicy, which delivers their commonalities. This class design with switchable policies supports static polymorphism and has the advantage that there is no runtime nor memory overhead by contrast to the use of dynamic polymorphism  (virtual function, vtables, etc.).

Creation of boundaries.  The CSMP API currently supports three ways to create Boundary objects, implemented in the class BoundaryInterface accessible through the public interface of the Model object:

- Around volumetric regions:

  - bool AddFaces( const char* region );

  - InsertBoundary( BOX_BOUNDARY boxBoundary, const char* region );

- From (dimension-1) regions:

CreateInternalBoundaryFrom();

- Between volumetric regions:

  - bool InsertBoundary( const char* region1, const char* region2 );

Naming conventions for boundaries. Apart from the predefined names for the boundaries of box-shaped models (LEFT, RIGHT…), boundary names always contain the string “BOUNDARY.”  Unique names for internal boundaries (located between volumetric regions) are achieved by concatenating this keyword with the names of the adjacent volumetric regions; for instance,

“BOUNDARY_SAND_SHALE”

The order of these region names matters: the first region (SAND) marks the inside of the boundary; the second (SHALE) refers to the outside, i.e. the side that the Face normal vectors all point toward. Such directions are imposed by the meshing tools honouring the model’s original topology.

When internal boundaries are created from (dimension – 1) regions, the name of these parent regions is added to the boundary name. Extending the example from above and assuming that the parent region was called “FAULT”, this yields the name

“FAULT_BOUNDARY_SAND_SHALE”

Such nametags are useful because they support a treatment of interface processes which is dependent on which rocktypes are juxtaposed agains one another.  The creation of boundaries from surface meshes that intersect multiple unique model regions therefore automatically triggers the creation of multiple boundary objects – as many as regions are juxtaposed against one-another, while the name prefix (FAULT) ascertains that the relationship to the former parent region from the (geo)model is recorded.

Fig. 11: Fault surface partitioned into Boundary patches on the basis of juxtaposition relationships. Original surface region ‘NORMAL_FAULT’ intersected the 3 different layers LAYER_BOTTOM, RESERVOIR_LAYER, LAYER_TOP and was partitioned into NORMAL_FAULT_BOUNDARY0_LAYER_RESERVOIR_LAYER_TOP, NORMAL_FAULT_BOUNDARY1_LAYER_RESERVOIR_INTERSECTION, NORMAL_FAULT_BOUNDARY2_LAYER_BOTTOM_INTERSECTION, NORMAL_FAULT_BOUNDARY3_LAYER_BOTTOM_LAYER_TOP, NORMAL_FAULT_BOUNDARY4_LAYER_BOTTOM_LAYER_RESERVOIR,

NORMAL_FAULT_BOUNDARY5_LAYER_TOP_INTERSECTION.

The label INTERSECTION refers to patches where the same rocktype is present on either side of the fault. This code example is from ‘BoundaryInterface_Test’.

### Split Boundaries

In some cases the introduction of Boundary/Face objects is not sufficient to achieve computational goals. For instance when jump discontinuities in the computed variables are expected to arise at material interfaces (e.g., Nick and Matthai, 2011, VZJ). In this case, node-matched mesh patches can be imported or a contiguous mesh can be split (=“exploded” into separate mesh objects) in a 2-step procedure: 1) creation of boundary objects from lower dimensional regions at where the splitting shall occur; 2) converting existing Boundary- into SplitBoundary objects. Mesh splitting is achieved by replacing the lower-dimensional Face objects that made up the boundary by so-called InterFace objects with additional nodes. Each Face node is replaced by as many InterFace nodes as materials come together at this point.

The new mesh contains many additional nodes = degrees of freedom, but remains node matched, at least initially. Like the Face, the InterFace knows its higher-dimensional neighbor Element objects – exactly 2 – one on each side. Its inside is identified as opposite to the outward pointing normal.

InterFace objects are the building blocks of SplitBoundary objects. It follows that these can exist only on the inside of a model and always have higher-dimensional neighbors on their inside and outside.

If a node-matched set of mesh patches is supplied to CSMP++, SplitBoundary objects can be created to connect them by calling

bool DetectAndCreateSplitBoundaries();

This is a costly process, but may be justifiable when the model is subsequently saved to a CSMP native binary file for further use, which is always desirable.

There is only one method to create SplitBoundary objects within CSMP++:

bool CreateFrom(Boundary);

SplitBoundary creation transforms an existing model so that it will have a range of properties that are very different from the original model with Boundary objects only:

- Split boundaries decouple computational domains from one another; if nothing else is done, either side becomes a “natural boundary.”

- If split boundaries completely surround a part of a model, this part becomes a stand-alone mesh that needs extra essential conditions as integration constants for the PDE that will be solved.

- New nodes with property storage will be created and property values will be mapped to them from the original nodes

By contrast with Boundary, SplitBoundary objects can only be created in the interior of a model. Notwithstanding, one might use similar functionality to couple separate Model instances together.

### ErrorHandler

The CSMP++ API contains the Singleton (class that can only have a single instance) ErrorHandler that ascertains that error messages are reported in a consistent way and appropriately for the current computational platform. The easiest way ErrorHandler can be used to track and report a notable incident is:

ErrorHandler&  incident( ErrorHandler::Instance() );

incident.notice( WARNING, incident_source, message );

The first line of code provides a local reference to the global singleton initialized elsewhere. Here incident_source refers to the class::method name from where the incident originated. The message describes what happened.

ErrorHandler handles how incidents are reported and processed by CSMP++ and does so in a platform specific way. Since it takes control of the entire program it can only exist once and has therefore been implemented as singleton. Any CSMP++ program contains one instance of the ErrorHander, which also contains the licensing mechanism.

Normally, the error handler only prints the messages to a log file or the console / screen using either buffered (cout) or unbuffered (cerr) streams (Tip: make console memory big enough to scroll up to see the entire output of a simulation to see when and where WARNING or ERROR messages started to occur). The behaviour of the ErrorHandler depends on whether a program is run in DEBUG or RELEASE mode. In DEBUG mode, the program halts allowing the user to intervene manually. To recover from problems the user has to implement exception handling facilities that are part of CSMP-based applications. CSMP++ only contains rudimentary exception handling, complementing the ErrorHandler mechanism (see csmp::Exception).

The enumeration CSMP_MESSAGE is defined in “Exception.h” and is used by the error handler to categorize what happened and to control how it is reported and handled on the current computer platform.

enum CSMP_MESSAGE { INFO=0, EXCEPTION=-1, WARNING=-2, ERROR=-3,   FATAL_ERROR=-4 };

Important here is that when the ErrorHandler issues INFO and WARNING messages, these will not interrupt program execution. A maximum number of ERRORs is permitted to occur before the handler terminates a simulation. By contrast, a FATAL_ERROR will lead to the immediate termination of a program.

The ErrorHandler logs information as strings that are printed to 'cerr' and to the text file 'Runtime.log'. To be able to trace a message to its source, a string that consists of the class and method name from which the message originated is supplied as first message argument. In case of an overloaded method, additional information in braces should be included into the output to identify the called method uniquely, for example:

Error: Region::FormAndAddGroup(from PropertyConstraints): Supplied element list is empty.

Messages logged to file, will be preceded automatically by a time string. This tells the user when the incident prompting the message occurred. Since messages may also contain numerical data in the description of the error message the ErrorHandler interface is overloaded to accept std::string arguments. To handle top level (C++) exceptions the ErrorHandler employs the standard exception base classes exception, logic_error, and runtime_error. For CSMP++-specific error types instances of the class csmp::Exception can be thrown. A goal achieved with this exception handling is to store the state of a run just before it got aborted due to a FATAL_ERROR. This permits the restart of a simulation at a later time, for diagnosis and elimination of errors. To save a computation to a binary file, the model is first exported into a csmp::VSet container and then stored permanently in a binary file. To make use of these capabilities, a program segment that may throw exceptions has to be surrounded by try and catch statements, for example:

try {

cout <<"\nmain: Advecting non-wetting phase..."<< endl;

// here goes the code that does this

}

catch(Exception& csmp_excep )	{

cout <<"\nCSMP++ Exception: Raised by: "<< csmp_excep.What() << endl;

VSet<3> crashed_model;

cout << "\nmain: Saving current model to CSMP++_VSet... " << endl;

model.OutputTo( crashed_model );

const csmp::Index time_key(model.Database().StorageKey(“time”));

crashed_model.OutputTo( model.Name(), model.Read( time_key ) );

// any other code here…

return 1;

}

Alternatively, the high-level output of a complete Model object to binary file may be preferred:

model.OutputToBinaryFile( model.Name() );

ErrorHandler is only used inside CSMP++ specific larger classes like Model. Small generic single-purpose classes report errors directly to the 'cerr' stream to prevent a loss of generality by making them dependent on the error handler. Naturally, classes do throw standard exceptions.

### PropertyHandle

The class PropertyHandle was implemented to write top-level mathematical expressions involving distributed variables. Due to the inelegance of the current implementation it is likely to be refactored in the near future.  Lambda functions already provide a viable alternative.

PropertyHandle objects associate themselves with physical variables registered in the PropertyDatabase or prompt the latter to create a new variable at runtime according to specifications supplied via custom constructor. The new variable has local scope and will be destroyed when the destructor of the handle is called. PropertyHandle currently includes C++11 standard mathematical functions. In the following example, a PropertyHandle is used to calculate the natural logarithm of the variable “porosity” times a multiplication factor of 0.9. Afterwards the new variable "log porosity" is output to a text file. Note that in the example, the handle is created dynamically such that the corresponding variable can be removed at a user-defined time:

// existing variables

PropertyHandle  pf( current_Model, "fluid pressure”); // node scalar

PropertyHandle  phi( current_Model, "porosity", SCALAR, ELEMENT );

// new variable only needed for logarithmic output

PropertyHandle* log_phi = new PropertyHandle( current_Model,

"log porosity",

SCALAR, ELEMENT );

// assign the porosity to the new variable

double64  factor(23.4);

*log_phi  = 0.0;

*log_phi += phi * factor; // a+=b is C++ shorthand for a=a+b

// logarithmitize values

log_phi->Log();

log_phi->Out( logarithmitized_porosity );

// delete the temporary variable

delete log_phi;

It is also possible, to write expressions involving variables with different placements, which involves interpolation or extrapolations, supported by the finite element basis functions. However, this functionality is still in its infancy.

In summary, PropertyHandle provides direct access to- and mathematical manipulation options for distributed physical variables. This facilitates a math software-like notebook style of setting up calculations.

### Interrelations

Model

As seen in the steady-state fluid-pressure example, CSMP++ calculations, which can be carried out on an element-by-element basis because they do not involve the inversion of a global solution matrix, are done by subclasses inherited from the Interrelation base class. When writing the source code for such subclasses, one associates required global physical variable(s) with Operand class objects and writes intended mathematical expressions inside the virtual Calculate() method of the new subclass. A decisive rule for interrelation objects is that a single interrelation can only alter the value of a single physical variable. This restriction allows to execute interrelations in a sequence that takes mutual interdependencies into account. While the output from interrelations is checked against the property database, the input is not. It is the responsibility of the interrelation implementer to perform such range checks to ascertain that the intended calculations are only performed for input values for which the interrelation has been tested.

Fig. 10: Sequence diagram illustrating the use of an interrelation subclass (e.g., Conductivity) in a CSMP++ simulation.

As an example of a calculation, which could be carried out using an Interrelation object, the calculation of the element property “hydraulic conductivity” by dividing permeability by fluid viscosity is presented in Figure 10. In a main() programme an instance of this interrelation would be constructed and used in the following way:

#include “ConductivityFromPerm.h”

const PropertyDatabase& pref(model.Database());

const double            viscosity(1.6e-3);

ConductivityFromPerm< 3U>  K( pref, viscosity );

K.RestrictApplicationTo(“saturated zone”);

Model.Pass( K );

The code for this interrelation is in the header file of the class 'ConductivityFromPerm' and is listed below. To apply this interrelation to a model, one simple creates an instance of ConductivityFromPerm and supplies it as argument to the Pass() interface of the Model.

#ifndef CONDUCTIVITY_FROM_PERM_H

#define CONDUCTIVITY_FROM_PERM_H

#include "CSMP_definitions.h"

#include "Interrelation.h"

namespace csmp {

The interrelation ConductivityFromPerm is inherited from the base class Interrelation:

template<size_t dim>

class ConductivityFromPerm : public Interrelation<dim> {

Operand< dim>&  k;

Operand< dim>&  K;

double64        viscosity;

public:

ConductivityFromPerm( const PropertyDatabase& p, double visc );

~ConductivityFromPerm() {};

void Calculate();

};

When the interrelation ConductivityFromPerm is constructed, the Operands, k and K are associated with the target distributed variables “permeability” and “conductivity”, respectively. The viscosity is initialized to a fixed value. Also the VARIABLE_FLAG flag that permits the output of the result to the Model is set (INIT_COND) and the result property is identified as the “conductivity”.

// constructor

template<size_t dim>

ConductivityFromPerm<dim>::ConductivityFromPerm( const

PropertyDatabase& p,

double64 visc )

: Interrelation<dim>(p),

k( Interrelation<dim>::GlobalProperty("permeability") ),

K( Interrelation<dim>::GlobalProperty("conductivity") ),

viscosity(visc)

{

Interrelation<dim>::Name(" ConductivityFromPerm");

// setting up the variables for the formula

Interrelation<dim>::OutputCondition( K, INIT_COND );

Interrelation<dim>::ResultProperty("conductivity");

// test whether the input viscosity is within a physical meaningful

// range

ErrorHandler& csmp_error(ErrorHandler::Instance());

if ( viscosity < 1.0e-5 or viscosity > 1e6 )

error.notice( ERROR,

“ConductivityFromPerm<dim>(constructor)”,

“input fluid viscosity out of range”);

}

The calculation that is done when the interrelation ConductivityFromPerm is passed to the Model is defined in the Calculate() method:

template<size_t dim>

void ConductivityFromPerm<dim>::Calculate()

{

// range testing of input against PropertyDatabase

k.IsWithinRange();

// calculation

K = 1.0 / viscosity;

K *= k;

// testing

K.PrintValue(); // turn off once tested

} // end specification of interrelation

} // end namespace CSMP++

#endif

The source code listing illustrates that a new Interrelation subclass can be created in a few simple steps:

1. Test the formula that describes the interdependencies of the physical variables of interest. This may involve graphing and designing this Interrelation as a formula in Matlab or Mathematica or doing a curve fit through some experimental data. One must also define a value range for each of the involved input variables as the range in which the mathematical formulation produces meaningful results. Ideally, this range should be at least as wide as the value range specified in the property-database text file from which the CSMP++ program initializes the property storage. The new formulae must be tested for all possible input parameter combinations within the prescribed ranges before it is used inside CSMP++ computations: Using the global instance skm_err of the SkmErrorHandler, range violations can be reported as was described earlier. This approach is the only way of ascertaining that a program produces correct results once a model becomes complex and contains several interrelations.

2. Define a notation and make each symbol a reference to an Operand (Operand&). These references will be private data members of the new interrelation subclass declared by public derivation from the Interrelation class. In the example above, k and K are made to refer to the distributed variables “permeability” and “conductivity”, respectively.

3. Register the Operand references in the constructor of the derived Interrelation subclass. In C++ references must be initialized in the constructor. First, one constructs the base class, then the base class method GlobalProperty() is used to associate the Operands with the distributed variables. Finally, one defines which of these variables will store the result of the calculation using the base class method DefineResultProperty(). One must also define an output condition flag (OutputCondition()) for the result property. The use of this flag prevents Interrelations from overwriting variable values with a different flag.

4. Override the pure virtual member function Calculate() of the Interrelation base class. The user defined new method Calculate() will contain the mathematical formulation of the interrelation written as equation, using the Operand reference notation and involving * / + -  < > <= >= and other mathematical symbols which are interpreted as overloaded operators of the Operand class. If many temporary variables are needed they should be defined as private data of the Interrelation subclass to prevent their destruction when the Calculate() method goes out of scope. In the example above, the viscosity is such a private variable.

To assign the value of a distributed variable represented by an Operand to a scalar, vector or tensor variable, the AssignTo() method of the Operand should be used:

TensorVariable<CSMP++_float,3>  perm;

k.AssignTo( perm );

Note that if one writes something like

K += b + c - d * 24.5;

this will automatically create four temporary Operands (why ? - do C++ exercise). If speed is critical because interrelations are applied again and again in a transient calculation, avoid the creation of temporaries by converting such code as  follows:

K -= d * 24.5;

K += b;

K += c;

The product of d and the decimal number will be a floating point number; this temporary is not costly, as compared with a new Operand.

5. Apply the new Interrelation by passing it to the Model (Pass()). This will invoke the Interrelation method Apply(), which computes the formula and performs a range check on the result values.

6. By default, the new interrelation is applied to the entire model. If the new interrelation shall be applied only to a group of elements the method RestrictApplicationTo() facilitates this.

Calculations one can perform with Interrelation subclasses are limited to result variables that are on the same or a higher level in the Model mesh hierarchy as the input variables.  For instance, one can calculate an element property from node- or integration point properties but not the other way around because this would require information from more than one finite element. If one calculates an element property from node property values, the interrelation algorithm will use a nodal average rather than interpolated values within Calculate(). If this is too limiting, one may consider to do the calculation via a Visitor object (see next section). As another alternative one can extrapolate element properties to the nodes with:

void Model<dim>::ExtrapolateElementPropertyToNodeProperty(

const char* eprop, const char* nprop, bool by_distance=true );

Do not forget to create the necessary node variable first. Also, there are different ways of weighting the contributions from elements surrounding the node in this calculation: Weighting by the inverse distance of the element barycenter to the node is the default. Weighting by element area (by_distance=false) gives  quite different, arguably less physically meaningful results.

### Operands

As shown above, Operand objects support calculations involving variables with a diferent type or placement inside of Interrelation classes.

It is a yet-to-be-realised design objective of CSMP++ to support the writing of mathematical expressions involving physical variables with different placements and types so that they read like standard math with a concise notation. Since C++ meta-template programming permits compile-time assembly of such expressions this should be possible and has already been demonstrated elsewhere by domain-specific languages (DSL). Such a design could also implement, compile time error checking, and the detection of run-time range violations incurred in resulting calculations, including the detection of results that are mathematically correct, but not physically meaningful. This functionality is still missing from the CSMP++ API.

A small subset of such capabilities are made available through Operand objects used by the Interrelation class hierarchy. Apart from facilitating to write mathematical expressions which are applied to the entire Model as is done using PropertyHandle, the Operand class meets the need to use distributed instances of properties in calculations inside code modules, making distributed variables with a different placement interoperable.  The current implementatio requires the user to (1) define a mathematical notation for the variables used in mathematical expressions, and (2) associate csmp::Index objects with the physical variable using this notation, see example below. This way the PropertyDatabase neeeds to be queried only once when an Operand is initialised. Finally (3), temporary storage is needed to hold the result value of a calculation for range-checking before it is written back to the Model.

These capabilities are implemented in the Operand class, which is a simple composite holding a std::string with its name, a csmp::Index for the represented variable, any one of the basic CSMP++ variables, their physically meaningful range, and an output condition flag (VARIABLE_FLAG), which identifies distributed property values, which the variable is permitted to overwrite if the calculation yields a result that is within the permitted range.

The Operand has the overloaded operators +, -, *, /, +=, -=, *=, /= such that Operand objects can interact with each other, double64 data, and CSMP++ basic variables. In C++ the combined assignment operators are shorthands for expressions like:

a = a * b;      is equivalent to     a  *= b; // C++

One should not write expressions like A = C  because this means, following the logic of the C++ assignment operator, that the Operand A now is an exact copy of the variable B. Thus, if Operand A represents a property stored in the Model, the corresponding variable will no longer exist following this assignment.

It is also important to remember that scalar-, vector-, and tensor variables differ from their mathematical counterparts because they contain STATUS flags which specify how their entries are treated during the assembly of finite-element equations. This requires rules for the assignment of flags to the result of Operand-based calculations. For example, the scalar variable expression

A = C + B;

needs to resolve the flag of the temporary variable that is created before the assigment is made. The rule adopted here is that a temporary Operand like that created by addition will always be flagged like the result Operand.

If one compares an Operand with a VARIABLE_FLAG enumeration, the comparison will be carried out using the flag(s) of the underlying variable. Thus, if a vector variable in a 2D calculation has two different flags (for instance, a DIRICH flag for the x displacement and a PLAIN flag for the y displacement), any comparison will evaluate as false. In this case, one should use the method VectorVariable::Flag( int i ) to carry out a more specific comparison. Any comparisons with uninitialized Operands or Operands that store NaN (not a number) values will evaluate to false.

By default, Operands provide storage for a single variable value of arbitrary type. However, Operands can only be created if the variable that they associate with already exists in the PropertyDatabase. This differs from PropertyHandle objets, which can spawn the generation of a new variable in the PropertyDatabase.

### Visitors

The visitor scheme, as described in Gamma et al. (Reusable object-oriented software patterns), provides access to the public interfaces of classes inside of a class hierarchy. It thus permits the implementation of new functionality without modification of existing classes. In the visitor pattern, each target class has an interface Accept( visitor& ). This interface calls the visitor’s member function  Visit(target_class*) giving the visitor access to the public interface of the target class. With this simple technique a wide variety of operations are instrumentalized in CSMP++ (Fig. 11). All Visitor subclasses derived from the Visitor base class have access to the public methods of the visited classes and can differentiate the treatment of sub-targets in the Model hierarchy. This is achieved by overloading the method Visit() for regions, boundaries, elements, faces, interfaces, integration points and nodes, i.e. Visit( Node* ), Visit( Region* ). Like interrelations and PDE_Integrators, visitors can be restricted to regions or boundaries. Since visitors have access to the functionality of elements, the visitation path can also employ their connectivity, moving from an element to its neighbor. Any kind of distributed data can be retrieved from the finite-element mesh during its visitation. A visitation starts with the submission of a specific visitor to a target object through the Accept( Visitor& ) method (Fig. 11), defined for all classes in the Model hierarchy. As a source code example of an inherited visitor, the ID_Visitor collects ID numbers of target objects. Its declaration is in “ID_Visitor.h” (including inline functions that show how the visitor accumulates the desired data).

Region

Model

Visitor

Fig. 11: Application of visitor object to the Model hierarchy of objects.

The application of a visitor involves recursive dispatching by target objects: When the visitor is submitted to the Model (model.Accept(vtor)) it determines whether to pass it on to a Region or directly to the collection of elements. Subsequently, either the Model or the Region, allows the visitor element by element access to its members. The visitor may also choose to travel directly from an element to its neighbor, tracking a fluid flow path or conducting a mesh sweep such as a “flood fill” (protected method Model::FloodFill()). To control the entry and the target level of a visitation of the model class hierarchy, the user defines the APPLICATION_LEVEL (start of the top-down visitation) and the APPLICATION_TARGET (the lowest level member of the class hierarchy that shall be visited. Defaults are MODEL and ELEMENT, respectively.

### PDE_Integrators

PDE_Integrators, in CSMP++ terminology, were designed for calculations that require the inversion of a global solution matrix A. One or several second-order partial differential equations are discretized as a set of linear algebraic equations and solved in the form Ax = r, where x is the solution vector and r is the right-hand vector. A, x, and r are implemented as SparseMatrix and double precision STL vector templates (see applied_math library).

PDE_Integrator

Region

Model

Fig. 12: Sequence diagram for the application of a PDE_Integrator to the Model. Key steps are (1) construction of MathOperator objects added to the integrator to compose the integral form of the PDE which shall be solved; (2) application of PDE_Integrator to Model, and (3) solution of the global sparse matrix, A, by a Solver object. (4) writing the results back from the solution vector to the model while performing range checks uses the ranges stored in the PropertyDatabase.

The implementation utilises several design patterns (Bridge, Strategy etc.) to make the solving of partial differential equations on global and local domains relatively simple. The non-iterative solution process is implemented in the PDE_Integrator base class utilising MathOperatorLHS and MathOperatorRHS base classes, in the composition of differential equations. This process is the same for any type of finite element(s) used in the integration. The user has to pick the appropriate order of finite-element interpolation to obtain the desired continuity and accuracy of the solution.

The solution of the matrix equation is carried out by subclasses of the Solver base class. This polymorphism is implemented in the frame of the strategy design pattern offering the user a variety of solution methods (Fig. 13).

PDE_Integrator

Fig. 13: Strategy pattern employed to interface different solvers with the PDE_Integrator. Illustration for SAMG_Solver class, which receives its settings from the SAMG_Settings object, passed by reference to the constructor.

One can choose among algebraic multigrid (commercial SAMG_Solver, see Ruge and Stüben, 1987 and Stüben, 2000), conjugate gradient (CG_Solver) and Gaussian elimination (Gauss_Solver) schemes. These solver objects are configured with subclasses of the Settings base class (SAMG_Settings etc.). Dependent on the type of computation, solver parameters may have to be set manually, see documentation of third party tools in corresponding directory of API library. As an example, the following code configures the SAMG_Solver for the computation of displacements, see LinearElasticy example:

SAMG_Settings settings;

// sort rhs vector so that [x1, y1, x2, y2, ..., xn, yn]

settings.Set_napproach(2);

PDE_Integrator<3,Region>  deformation;

deformation.SetSolver(new SAMG_Solver(&settings));

Detailed descriptions of the SAMG solver parameters are given also in their user manual (SCAI Fraunhofer) and the HTML Reference manual in the documentation of the Solver class.

Apart from integration in space, PDEs may have to be integrated in time. Explicit, implicit, and semi-implicit schemes can be configured according to needs. The following 1D example computes the evolution of fluid pressure with time using a fully implicit (Backward-Euler) finite-difference time-stepping scheme and a lumped formulation for the capacitance (mass) matrices on the left and righthand sides:

// PDE_Integrator computes: ([C] + dt[K]){p}t+dt = [C]{p}t + dt {Q}t+dt

// ---------------------------------------------------------------

PDE_Integrator<1U,Region>  transient_pressure;

transient_pressure.SetSolver(new Gauss_Solver());

PropertyDatase&  p_ref = model.Database();

// [K] conductance matrix

Integral_dNT_op_dN_dV<1>

conductance1( p_ref, "conductivity", "fluid pressure", "fluid pressure" );

conductance1.MultiplyWithTimeIncrement();

// [C] capacitance matrix lefthandside

Integral_NT_lhsop_N_dV<1>

capacitance_lhs( p_ref, "storativity", "fluid pressure", "fluid pressure" );

// make lefthandside matrix equivalent to right-hand vector

capacitance_lhs.LumpedFormulation(true);

// [C] capacitance vector righthandside

Integral_NT_op_N_dV<1>

capacitance_rhs( p_ref, "storativity", "fluid pressure" );

capacitance_rhs.LumpedFormulation(true);

// [Q] fluid volume source

Integral_NT_op_N_dV<1>

source1( p_ref, "fluid volume source", "fluid pressure" );

source1.MultiplyWithTimeIncrement(true);

// add to right-hand side after its multiplication with the current pressure

source1.AddAccumulateLater();

source1.LumpedFormulation(true);

transient_pressure.Add( &conductance1 );

transient_pressure.Add( &capacitance_lhs );

transient_pressure.Add( &capacitance_rhs );

transient_pressure.Add( &source1 );

...

while (global_time < max_time && timestep <=100 ) {

global_time += time_increment;

transient_pressure.TimeIncrement( time_increment );

model.Pass( transient_pressure );

printRangeOfVariable( "fluid pressure" );

time_increment *= 1.2;

timestep++;

}

To use a PDE_Integrator in a transient (timestepping) calculation one has to communicate the TimeIncrement() to it as shown in the code example  above.

Apart from the PDE_Integrator base class, derived Picard-, Newton- and Upwind- PDE_Integrator classes are available in the /algorithm library. As the names suggest, these facilitate iterative solution procedures for non-linear PDEs. The PDE_Integrator classes also supports the solution of coupled PDEs, i.e. equations with several dependent variables. In this case, cross terms (“Jacobians”) are needed to create the coupling between dependent variables. If the coupling is non-linear such equations must be solved iteratively (see the advanced examples 13 and 14).

### PDE Operators

For the standard type of finite element integrals, suitable PDE operators have been inherited from the MathOperatorLHS and MathOperatorRHS base classes, supporting the PDE_Integrator design pattern. The syntax of the method arguments (see constructors in the examples above) prescribes the position of integral coefficients in the integral:

MathOperatorLHS( const PropertyDatabase<dim>&,

const char* oper,

const char* basic,

const char* test );

The 'oper' typically is a material property matrix Operand and ‘test’ stands for test functions (=element interpolation functions) or weighting function Operand objects. An Operand will never be an output variable. Since element-interpolation functions are used to approximate the value of the dependent variable, the physical variable that one puts as a placeholder of ‘test’ will be the result and output property of the finite-element computation. In the code library the following standard and many other differential operators are available:

MathOperator subclass	Placement in Gx=rh		Description

Integral_NT_lhsop_N_dV		left-hand vector operator		mass matrix, volume integral

Integral_NT_op_N_dV            	right-hand vector operator 		mass matrix, volume integral

Integral_dNT_op_dN_dV 		left-hand  		              conductance, volume integral

Integral_dNT_rhsop_dN_dV	right-hand		              conductance, volume integral

Integral_NT_op_dNi_dV	 	right-hand 		              volume integral in direction i

Integral_dNT_op_dN_NT_v_dN_dV right-hand 			advection - dispersion

Integral_NT_op_dS	 	right-hand 			surface integral

Integral_op_NT_dN_orthogonal_dV	left-hand				streamfunction operator

Integral_dNT_rhsop_dN2_dV	right-hand 			Laplacian volume integral

the same operators as above but for equations with vector type solution variables, e.g.,

Integral_BT_op_B_dV		left-hand				stiffness, volume integral etc.

the same operators but using numerical integration in conjunction with isoparametric FEs, e.g.,

NumIntegral_BT_op_B_dV		left-hand				stiffness, volume integral etc.

Inside of the PDE_Integrator, the PDE operators are stored in std::maps. Each integral contributes to the equation to be solved and contains information on the place that the Operand has in the equation. Operands are shared among equations. The PDE_Integrator object encapsulates the whole process of (cf., Fig. 12):

- Definition of a finite-element equation

- Collection of distributed variable values from the Model object

- Calculation of the finite-element matrices (contributions)

- Assembly of the solution matrix

- Solution of the linear algebraic system

- Transfer of results back to Model after range checking of dependent variables.

PDE_Integrator are applied by passing them to the Model:

model.Apply( evolve_pressure );

or passing the model or a region thereof to them:

pressure_integrator.IntegrateOver( model.Region(“fracture set 1”) );

Inside the Model, the public methods of the PDE_Integrator class are called in the execution sequence shown in Figure 11. Only in time-dependent calculations, initial conditions are assembled, usually into the right-hand vector. By default, PDE_Integrators objects compute a steady state.

For large problems or transient calculations it is a good practice to re-use PDE_Integrator objects to minimize memory allocation for global matrices. Note that if one re-uses a steady-state PDE_Integrator in a Newton-Raphson iteration, global matrices will be reset automatically.

## 5. Defining & Solving Finite-Element Equations

When the finite-element method (FEM, e.g., Huyakorn and Pinder, 1983; Istock 1989; Zienkiewicz and Taylor, 1994; Bathe, 1996) is used to solve a partial differential equation (PDE) that describes the distribution of a field variable (e.g., fluid pressure, solute concentration) across a domain of interest, this domain is discretized into a finite number of chunks, each of which is represented by a single finite element. All finite elements together form the finite element mesh. To see how a dependent variable varies in space and time as a function of material properties and independent / constitutive variables, one needs to integrate the PDE, obtaining the variable distribution. In the FEM, field variables are integrated element by element such that the incremental change across the mesh yields the variation across the model. As there is no exact solution to the PDE of interest, one integrates a piece-wise continuous approximation of which one hopes that it satisfies the PDE. For this approach to be successful, the approximating function must have a range of properties, which are discussed in detail in the CSMP++ Developer Guide. An FEM user needs to know that the functions must be differentiable to at least one order higher than the order of the derivative of the solution which shall be exact. This requirement reflects a side effect of the finite-element discretization: dependent on the order of the approximating functions, discontinuities will exist across element boundaries of the function derivative. In the presence of material property variations in the computational domain one cannot find the derivative of a solution at a node (the derivatives computed for each of the elements which share the node will differ from one another). Implications of this for calculating fluxes, stresses and strains are discussed in the Developer Guide and in the description of the Finite Volume library. In CSMP’s hybrid FE-FV discretization, the FVM represents a complementary numeric method, synergizing with the FEM.

This section explains the basics of the Galerkin finite-element method for a linear triangular element implemented in the LinearTriangle class in CSMP++, comprising matrices and vectors. The finite-element forms of second order partial derivatives are discussed approximating the physics of fluid flow through porous media and stress and strain in structural mechanics, respectively, in the following example.

A relatively small set of PDE operators representing volume, surface and time integrals over the change in the dependent variable, suffices to compose finite-element simulations of fluid flow, heat flow, and linear elasticity. The finite-element forms of such common PDE operators have a standard notation in the finite-element literature that is reviewed below. Subclasses of the MathOperatorLHS and MathOperatorRHS constitute the PDE operator library to be used together with the PDE_Integrator and derived classes.

| Notation | PDE Operator | Application |
| --- | --- | --- |
| basic operators |  |  |
| A |  | area of 2D element, volume in 3D |
| M |  | material property matrix |
| D |  | elasticity matrix |
| J |  | Jacobian matrix |
| N |  | interpolation functions (i.f.) |
| DN, dN |  | i.f. derivative matrix (Nx, Ny) |
| B |  | i.f. deriv.m. 2dof per node (dim x n) |
| r |  | scalar property value |
| composite operators |  |  |
| [K] | A BTDB | stiffness matrix |
| [C] | A DNTKDN | conductance matrix |
| [S] | A/3 N r N | lumped capacitance matrix |
| {Q} |  | source-sink terms |
| [V] |  | advection-dispersion matrix |
| {u} |  | flux, displacement |

Table 2: Notation used in the description of finite-element method.

In the following, these operators are explained for a simple two-dimensional steady-state fluid pressure computation that was already discussed in Chapter 2. Ni are the element interpolation functions which are used to model variations of the dependent variable, φ, inside of each element, such that

f(x,y) is the approximated value of at the point x, y inside of the element, calculated as the sum of the products of φ values at the nodes with the corresponding interpolation functions for these nodes. Index n refers to the number of nodes, and basal indices denote node numbers local to the element. Since the discussion focuses on individual finite-elements, the (e) superscript is omitted. It usually distinguishes element- from global matrices.

A, the area of a two-dimensional element is equivalent to the space integral over the interpolation functions:

DN is the matrix of the first spatial derivatives of the element interpolation functions and has the general form (in 2 dimensions):

MTRL is a symmetrical material property matrix with a size of space dimensions squared. It can be used to represent scalar, vector, or tensor properties. For example, the anisotropic hydraulic conductivity, K, in a two-dimensional element is expressed as:

J is the Jacobian matrix for the translation of the local coordinate system (ε,η,ζ) into global coordinates (x,y,z). Inside of the source code of CSMP++, local coordinates are represented by the variables r, s, and t  because the Greek letters do not form part of the ASCII character set that is available for C++ programming.

Matrix contributions calculated for a straight-sided reference element are translated to the global coordinate system using the inverse of the Jacobian, the transposed inverse of J, and J’s determinant. In CSMP++, this procedure is applied in all higher-order elements. This is further discussed in the Developer Guide.

N is a 1 by n matrix (vector) with element interpolation function values at point x, y:

Test function products of the form NTN are required when properties are integrated over an element.

### Example: Steady-state fluid pressure

The variation of fluid pressure in a model that was computed by the program described in Chapter 2, can be regarded as the sum of local fluid pressure variations in the x,y,z directions and their perturbations by local fluid sources and sinks:

If one works with hydraulic conductivities, K, treated as constant over the finite elements (ignoring fluid viscosity variations), the steady-state pressure equation simplifies to the elliptic PDE

also known as Laplace equation and expressed more concisely using the nabla operator

A global matrix expression is assembled as the sum of the element contributions:

where K is a global matrix of the combined element conductances, p, is the solution vector which will hold the nodal values of the dependent variable “fluid pressure”, and Q is an elemental fluid source or sink. The exact form of the conductance matrix is dependent on the dimensionality of the model and the type of finite element, which is used in the computation. When global element coordinates are used, the operator Integral_dNT_op_dN_dV, i.e., the two-dimensional conductance matrix has the form

This finite-element integral is solved by (associative but not commutative) matrix multiplication. This integral is just equal to the area, A, of the finite element. Thus,

2A

For straight-sided triangular elements there is a simple formula for the integration of element matrices taking into account the order of the element interpolation functions:

a, b, c are exponents of the interpolation functions, and the factor 2 captures the spatial dimension of the finite element.

If local coordinates are used, which allows to represent curved element boundaries of higher-order elements in physical (global) space, these are specified as constants for a reference element and transformed into global coordinates by multiplication with the inverse of the Jacobian matrix and its determinant.

Using this Jacobian transformation, the conductance matrix for a two-dimensional element with local coordinates becomes:

This integral NumIntegral_dNT_op_dN_dV is solved by numerical integration:

This calculation amounts to a summation of element matrices that arise at each integration point. However, at each integration point, the interpolation function derivatives and J, J-1 and det J must be newly evaluated (although for the simplex elements line, triangle, and tetrahedron the Jacobian matrix is constant). Wi,j represents weighting function values for each of the n integration points inside each element. They ascertain that the n contributions add up to the length, area or volume of the element in physical space when a field variable with a value of one is integrated. The location of integration points inside the reference element is chosen according to theoretical considerations, normally Gaussian quadrature rules.

In summary, interpolation-function partial derivatives at each integration point and with respect to the local coordinate system, are multiplied with the inverse of the Jacobian matrix at this integration point, and then summed, performing the numerical integration. All this happens behind the scenes, when isoparametric finite elements are chosen for a CSMP++ model (this is the default for poly-element type meshes).

Each of the integrals defined above has a corresponding PDE operator that is derived either from MathOperatorLHS or MathOperatorRHS, see above. To compose the partial differential equation using an PDE_Integrator object, references to these math operators are passed to the PDE_Integrator before it is applied on the Model object, see the code in the examples:

// 2. Steady-state fluid pressure computation [K]{p} = {Q}

// -------------------------------------------------------

PDE_Integrator<2>  steady_state_pressure;

steady_state_pressure.SetSolver(new SAMG_Solver());

Integral_dNT_op_dN_dV<2>

conductance( p_ref, "conductivity", "fluid pressure", "fluid pressure" );

Integral_NT_op_N_dV<2>

source( p_ref, "fluid volume source", "fluid pressure" );

VelocityAndVolumeFlux<2>

postpro( model, "conductivity", "porosity", "fluid pressure", true );

steady_state_pressure.Add( &conductance );

steady_state_pressure.Add( &source );

steady_state_pressure.AddPostProcess( &postpro );

model.Apply( steady_state_pressure );

Note that this code and the math operator objects are identical for two and three dimensional models and different types finite-element types. The MathOperatorLHS subclass Integral_dNT_op_dN_dV represents the exact volume integral over the derivatives of the element interpolation functions modeling the fluid-pressure distribution described by the PDE based on Darcy’s Law: The MathOperatorRHS subclass Integral_NT_op_N_dV represents a fluid source term per specific volume that needs to be integrated over each element. It enters the right-hand side of the matrix equation.

PDE_Integrator

Fig. 16: Collaboration diagram describing the accumulation process of added and product terms in the partial  differential equation solved by the PDE_Integrator (this figure is out of date because CSMP++ now uses the csmp::DenseMatrix and std::vector instead of Meschach IVec, Vec, and Mat structures.

When an PDE_Integrator is passed to a Model or Region object it accumulates the contributions from the Elements it loops over, calling the math-operator subclasses sequentially (Fig. 16, see also Fig. 11). The MathOperator methods

void  AddAccumulate();

void  MultiplyAccumulate();

void  AddAccumulateLater();

void  MultiplyWithTimeIncrement( bool multiply );

allow the user to specify the type of accumulation and when this is done within the global matrix and vector assembly process. Each PDE operator executes its methods

virtual void GetOperands();

virtual void ComputeContribution();

AssignToGlobal();

on each element that participates in the accumulation. The information that is specific to the type of finite-element that the element represents, is provided by the corresponding subclass of the FiniteElement. This FEM functionality is made available through the interface of the Element class through Policy-Based Class Design (cf., Alexandrescu). For this purpose, the Element class inherits the FiniteElementPolicy class, which contains a pointer to the FiniteElement. This is essentially the Bridge design pattern (Fig. 17).

NumIntegral_dNT_op_dN_dV

PDE_Integrator

Update needed !

Fig. 17: Policy-based class design of the Element class with Bridge design pattern connecting it to the FiniteElement which is polymorphic, allowing its behaviour to vary with finite element type. Although not shown here, for simplicity, the Element also contains the other policies: FiniteVolumePolicy and LocalVariableStorage policy.

### Transient Fluid Pressure

The transient fluid pressure distribution in a fluid-saturated domain is described  by:

This equation is integrated incrementally breaking total time up into steps. Time-stepping is usually implemented by a fully-implicit (Backward Euler) finite-difference scheme:

As already discussed above, S and K are the storage capacity and conductance matrices, and p and Q are the fluid pressure (dependent variable) and fluid production/consumption vectors, respectively. The fluid-pressure distribution at the next timestep t+1 is computed from the present fluid-pressure distribution at t, and potential pressure or flux changes at Dirichlet or Neumann nodes, respectively.  Fluid sinks or source rates are also taken into account.

The storage capacity matrix represented by Integral_NT_op_N_dV and Integral_NT_rhsop_N_dV has the form:

If this integral is solved by numerical integration, the storage capacity matrix is computed as:

### Buoyancy-Driven Flow

The buoyancy force Fb which acts on a volume of fluid, V with a density ρ, diverging from a reference density ρ0, is

where g is the acceleration of gravity (9.81 m s-2). The corresponding vertical “fluid pressure” contribution of the element is equivalent to the integral over the changing fluid density. The buoyancy-related volumetric flow in the element, q, due to this gradient is calculated from Darcy’s law:

Since the hydraulic conductivity refers to a volume flux not a mass flux the latter  is reduced as a greater flow volume passes through the rock when the fluid density is less than the reference density. The effect of buoyancy forces on the flow, requires the accumulation of an Integral_NT_op_dNi_dV operator to represent the vertical pressure gradient term in the equation. This term is non-zero only where fluid density deviates from the reference density and is accumulated into the right-hand side of the matrix equation.

We recommend against the accumulation of the full density term because this would skew the distribution of the residual across the model.

### Lumping

The diagonalization of element matrices is called “lumping”. In this process, the “off-diagonal” elements of a matrix are set to zero by multiplying these terms into the diagonal. This is equivalent to defining the interpolation functions differently such that

where n is the number of nodes in the element. In a “consistent” matrix for a two-dimensional triangular element (left matrix below), the area factor A, is divided by 12. This is required to obtain an exact integral over the interpolation functions according to the integration formula presented earlier.

In the “lumped” matrix on the right, lumping has been achieved by multiplying the identity matrix with a third of the area. This denominator is equivalent to the number of nodes = interpolation functions used. The determinant  remains the same (4A/12 vs. A/3). Lumping may also be achieved by finding the Eigenvalues of a matrix. In many cases, this is the most physically meaningful lumping procedure. Relative to the consistent formulation, lumping can introduce errors since off-diagonal matrix terms are effectively ignored. Lumping must therefore be used with caution, especially for linear elasticity problems where off-diagonal terms describe shear stresses etc.

#### LinearElasticity

The elastic deformation of a discretized rock volume in response to (1) boundary displacements, (2) point forces applied to the nodes, (3) stresses applied at the boundaries, (4) body forces, and (5) volume changes (thermal expansion, contraction, or compaction), can be simulated with a specially configured PDE_Integrator (example12.cpp).  Also quadratic finite elements are used, this computation is only first-order accurate. This means that while the displacements in the model are computed fairly realistically (however only if variations in the material properties are small), the strains and stresses computed in post-processing operations, are somewhat noisy. Example 12 solves the matrix equation:

[K]{u} = {F}

where K is the global stiffness matrix, u is the nodal displacement vector and F represents boundary conditions in the form of displacements, stresses, body forces, and volumetric source or sink terms. Element contributions to the stiffness matrix have the form:

[t A [B]T [D][B]]{u} = {F}

t, is a model thickness multiplier, A is the area of the element, and B is the interpolation-function derivative matrix (since a Galerkin method is used, the test functions are also used as weighting functions). D is the material property matrix for a plane stress computation (stresses act only in the model plane):

In D, E refers to Young’s modulus and μ to Poisson’s ratio. After the displacements have been computed, the strain that is constant in each element is calculated as:

{ ε } = {B}{U}

From the strain, the stress is calculated as:

{σ} = [D]{ε}

The right-hand side vector {F} of the global matrix equation can include either one or a combination of fixed displacements, body forces, stresses, or volume strains as essential conditions.

Volume strain can be incorporated into {F}, by defining fictitious point forces at the nodes of elements undergoing the volume change. The resulting element contribution to {F} is:

The actual volume change vector ε0, has no shear components, and its form, in the case of thermal expansion is:

where α is the thermal expansivity and T is the temperature. The resulting vector has six components since each node has two degrees of freedom, determining its displacement in two dimensions. The directional components of point forces are assigned individually to desired nodes of the finite element. In the equation below, fx and fy are point forces which act on node 1 of the element in horizontal and in vertical direction, respectively.

Fixed displacements are added to the matrix equation after all other right-hand terms have been applied to {F}. They are inserted as Dirichlet boundary conditions without reducing the size of the global solution matrix. This procedure is documented in the CSMP++3D Reference guide for the method AssignEssentialConditions() of the CSMP++_PDE_Integrator.

Stresses. Stresses acting on element faces at model boundaries are applied by distributing these as nodal point forces, but only on the boundary nodes of each element. This requires a knowledge of the length of each boundary segment (area of a face), L, and the x, y components of the stresses as forces per unit area. Since the stress is distributed on 2 nodes, a and b, the area-thickness product (tL) is divided by 2.

Body forces are accumulated into {F}, by distributing the total force among the nodes of an element:

where x and y are the directional components of the body force per unit volume and the indices denote nodes 1 to 3 of the triangular element (again t refers to the element thickness and A to its area).

### References Cited

Bathe, K.J., 1996, Finite Element Procedures, Prentice Hall, 1037 p.

Istock, J., 1989, Groundwater Modeling by the Finite Element Method, AGU Water Resources Monograph, Vol. 13, 495 p.

Huyakorn, P.S. and Pinder, G.F., 1983, Computational Methods on Subsurface Flow, Academic Press, 473 p.

Liu, Y. and Vinokur, M, 1997, Exact Integrations of Polynomials and Symmetric Quadrature Formulas over Arbitrary Polyhedral Grids, NASA Technical Memorandum 112202

Schwarz, H.R., 1991, Methoden der Finiten Elemente, Teubner Studien Bücher Mathematik, 435 p.

Weatherburn, C.E., 1927, Differential Geometry of Three Dimensions. Cambridge University Press

Zienkiewicz, O.C. and Taylor, R.L., 1994, The Finite Element Method Volume 1, McGraw-Hill, 648 p.

## 6. Initial and Essential Conditions

The boundary of a Model or Region subregion thereof, is defined by those finite-elements which share at least one face (edge in 2D) with this boundary. To such elements, their faces, or nodes a range of essential conditions can be applied.

The simplest way of constraining a model is to fix values of dependent variables at boundary nodes or some nodes inside. These “first order” constraints which must be satisfied by the solution are called “Dirichlet” conditions and are incorporated into the resulting system of linear algebraic equations by simple substitution, to the effect that the number of degrees of freedom is reduced. Since nodal values can be fixed inside of a model one speaks of “essential” rather than boundary conditions. Dirichlet conditions can be assigned with a variety of Model methods (for details see HTML documentation of the Model class):

void AssignBoundaryFlags(  SG_BOUNDARY, const char*, VARIABLE_FLAG );

void AssignBoundaryValues( SG_BOUNDARY, const char*, VARIABLE_FLAG,

double, double );

void AssignBoundaryValues( SG_BOUNDARY, const char*,

const VectorVariable&,

const VectorVariable& );

void AssignConstraint( const char*, long, const ScalarVariable& );

void AssignConstraint( const char*, long, const VectorVariable& );

void AssignConstraint( const char*, long, const TensorVariable& );

void AddBoundaryNormalFluxes( SG_BOUNDARY, const char*,

double, double ); // influxes are positive

bool AssignBoundaryValues( SG_BOUNDARY, const char*, VARIABLE_FLAG,

double, double, double, double );

bool AssignBoundaryValues( SG_BOUNDARY, const char*,

const VectorVariable&, const VectorVariable&,

const VectorVariable&, const VectorVariable& );

void ChangePropertyStatusInGroupToWhere( const char*, const char*,

VARIABLE_FLAG, VARIABLE_FLAG );

void ChangePropertyStatusInGroupToWhere( const char*, const char*,

VARIABLE_FLAG, double, double );

One must set a Dirichlet value at least for one Node, otherwise there will be an infinite number of correct solutions to the integration problem and the solver will produce an arbitrary result. The Dirichlet value has the same role as the integration constant, C, picked up in the integration of a standard indefinite integral of a function,

This is the reason for why such conditions are essential.

To assign constraints like a fixed flow rates across interfaces or fixed chemical source rates etc. a special approach is needed, since these involve first or higher-order spatial derivatives of the dependent variable. First, one projects the gradients onto the normal of the boundary and ends up with a scalar quantity. A little digression is warranted here in order to explain the meaning of this type of condition in an integral formulation:

Fig. 15: Boundary flagging in a CSMP++ model. Only the elements which share a face with the model boundary are considered as boundary elements.

Think for a moment about the difference between specific properties such as the density of inhabitants per km2, and overall properties, such as the total number of inhabitants of a village. A finite element represents a certain volume of rock with specific properties defined per unit volume such as the heat capacity or a fluid volume source term (m3 m-3 s-1). Specific quantities are integrated over the element volume by the FEM. One calculates the specific value of quartz which is precipitated per unit volume of rock due to gradient reactions (assuming a quartz-saturated fluid) by multiplying the fluid mass flux with time and the solubility-gradient in the direction of flow. However, if one is interested in the total amount of quartz precipitated within an element, this quantity must be multiplied with the element’s volume. This amounts to an integration. Similarly, essential conditions can be applied to CSMP++ models by specifying specific quantities or total values. Taking the specific approach has the advantage that one does not have to perform the integrations a priori.

Neumann conditions. The area of the face of an element (Fig. 15) comes into play when essential conditions are distributed as sources or sinks to node or element variables. To apply such NEUMANN boundary conditions (also called non-essential conditions because they are not critical), one has three alternatives:

(1) One does nothing. This implies that the derivative of the dependent variable across the boundary has to be zero. This Neumann condition is called natural boundary condition and is equivalent to a no-flow condition in a flow model (fluid will flow down a pressure gradient, i.e. parallel and not perpendicular to this kind of a boundary).

(2) One can distribute the specific face-normal quantities as point sources to the boundary nodes of the boundary element via a PointSource_rhsop PDE operator. However, this necessitates prior integration of these quantities and, because the boundary face area is rarely unity is a fairly intricate calculation. For example, for a boundary flux, q, per unit area, the nodal contribution to a linear triangular element and the righthand side of the global equation has the form

where t is the element thickness, L is the boundary segment length (facial area), A is the element area (volume in 3D), and Nb is the number of nodes at the boundary. This type of boundary conditions is also assigned with AssignBoundaryValues()and requires a separate variable to hold the prescribed boundary in- or outfluxes specified as boundary-normal components. These are added in the global matrix accumulation procedure using the differential operators as follows:

// 2D triangular finite elements only

model.AssignBoundaryValues( BOTTOM, "influx", NEUMANN, 1.0, 1.0 );

model.AddBoundaryNormalFluxes( BOTTOM, "heat flow", 1.0, 1.0e-12 );

The caveat of this method is that it only works for triangular finite elements in 2D.

(3) Alternatively, the boundary normal flux can be assigned as an face source term at the boundary of the mesh. This term is integrated with a suitable PDE operator, such as Integral_NT_op_N_dS which performs a surface integration of face variables where they have been flagged NEUMANN.

If  the model uses a combination of finite elements and finite volumes, the most accurate way of assigning boundary fluxes is via the finite volume technology:

FiniteVolumeTransport*  advector(0);

advector = new FiniteVolumeTransport( /* args */ );

model.AssignBoundaryValues( LEFT, "nodal source", 1., 1., 1., 1. );

advector->AssignScalarBoundaryValues( model, LEFT, "nodal source" );

...

// when the fluid pressure algorithm is defined include the following

PointSource_rhsop<CSMP++_float,3>

nodal_source( p_ref, "nodal fluid source", "fluid pressure" );

fluid_pressure_algorithm.Add( &nodal_source );

Initial conditions such as a fluid pressure distribution or initial stresses at the onset of a transient computation have to be specified by the user. CSMP++ initializes any physical variable specified in the variables “*.txt” file with a default value of NAN (not a number) and a VARIABLE_FLAG flag of PLAIN. With these settings, one cannot compute anything, because the NaN value will be the result, highlighting any not initialized variables as was the design intention.

To initialize physical variables or material properties one must set them in the configuration file or write a statements like

model.InputPropertyValue( "fluid pressure", makeScalar(ANY,0.) );

VectorVariable<1> vc(PLAIN,2.);

model. InputPropertyValue( “velocity”, vc );

The condition flags of initial values, can be changed in Groups or the entire Model, using the methods:

VARIABLE_FLAG  global_to(PLAIN), group_flag(BOUNDARY);

model.Region(“region”).ChangePropertyStatus(

“fluid pressure”, global_to, group_flag );

model.ChangeConditionFromTo("fluid pressure", DIRICH, PLAIN );

This example will remove Dirichlet boundary conditions assigned to the distributed variable “fluid pressure”.

Box-shaped or rectangular models. CSMP++ allows to assign essential conditions anywhere within a Model model. However, this process is greatly simplified for rectangular or box-shaped models essential conditions using the overloaded method AssignBoundaryValues().

moab.AssignBoundaryValues( LEFT,  "fluid pressure", DIRICH,  1.0e+7, 0.0 );

moab.AssignBoundaryValues( RIGHT, "fluid pressure", DIRICH,  1.0e+7, 0.0 );

In this example, scalar Dirichlet boundary conditions are assigned to the variable “fluid pressure”. The two input values allow to specify linear fluid-pressure gradients along the left and right vertical model boundaries and four values permit the assignment of bilinear variations to the sides of box-shaped models. The floating-point values supplied as method arguments specify variable values which shall be fixed. In the example code, values of the Node variable “fluid pressure” will be varied linearly from 107 to 0 between the first and the last Node situated at the left and right model boundaries, respectively. A few important conventions apply with regard to the boundary labeling and condition assignment to box-shaped models:

- A righthand-rule coordinate system is used always. In 2D, the TOP boundary is the one with the highest y-coordinate values since Y points upward reflecting altitude.

- In 3D, X points to the right, Y to the top, and Z to the front, adhering to a right-hand coordinate system.

- In 3D, all faces of the model are numbered counterclockwise, looking down the Z-axis from the front into the model.

- These faces are labeled BOTTOM, LEFT, FRONT, RIGHT, BACK and TOP.

- Face BOTTOM has the corners  	CNR1, CNR5, CNR6, CNR2.

- Face LEFT has the corners  	CNR1, CNR4, CNR8, CNR5.

- Face FRONT has the corners  	CNR5, CNR6, CNR7, CNR8.

- Face RIGHT has the corners  	CNR2, CNR6, CNR7, CNR3.

- Face BACK has the corners  	CNR1, CNR2, CNR3, CNR4.

- Face TOP has the corners  	CNR3, CNR7, CNR8, CNR4.

- Edges 1-12, 3D models only (see Fig. 5).

Corner locations and boundary gradients are only meaningful if the model is rectangular. If the boundary faces are tilted more than 45o relative to an orthogonal-normal orientation, CSMP++ will wrongly identify boundaries if one attempts to treat such models as box-shaped. However, in this case the boundary may be identified and flagged accordingly in the input data. This way the user can specify named boundary entities already in the graphics program in which the geometry is build. Essential conditions are then assigned with:

void Model::AssignEssentialValuesToGroup( const char* group,

const char* property,

VARIABLE_FLAG econd, fT val );

## 7. Input to & Output from CSMP++

This chapter describes how models are configured with ASCII configuration files and it introduces a very simple way to build rectangular two-dimensional models from bitmap images color-coded for permeability with arbitrary drawing programs. All the more sophisticated model building methods which require third-party software tools are documented in the CSMP++ Model Building & Interface Guide. It is a stand-alone manual and contains step-by-step instructions for model building with Rhinoceros (McNeel  & Associates), and meshing with the TETRA & PRISM family of meshing tools from ICEM CFD Engineering (now part of ANSYS). The model building guide describes how two-dimensional, adaptively refined models are constructed from line drawings using the 2D meshing tool Triangle and it introduces CSMP++’s GeometryPreprocessor (furnished upon request). A further section is dedicated to model building with GoCAD and SKUA (its commercial pendant). However, Rhinoceros and ANSYS TETRA are the key SOFTWARE tools relied on in CSMP++’s professional modeling workflow. They are recommended for state-of-the-art modeling.

We strongly recommend to test newly created CSMP++ simulation programs with simplified and coarsely meshed models first, before building and using memory-intensive elaborate models. This is especially true for transient simulations. There are several classes such as CSMP++_MeshDiagnostics which help the user to test whether a model is fit for purpose before a simulation is started.

### Model Configuration from File

It is not practical to recompile programs everytime input variables have to be changed are the timing when output is written shall be changed. For this reason, CSMP++ simulations can be configured from file using the CSMP++_InputDataManager class:

CSMP++_InputDataManager<3>  model_configuration;

ComputationalSettings       run_settings;

const bool groups_from_perm(false);

const bool set_prop_defaults(true);

const bool set_prop_of_groups(true);

const bool set_essential_conds(true);

const bool set_group_flags(false);

model_configuration.ConfigureFromFile( model,

file_name,

groups_from_perm,

et_prop_defaults,

set_prop_of_groups,

set_essential_conds,

set_group_flags,

run_settings );

Typically, simulation programme are configured with two text files which select the target regions of the mesh and initialize the model with material properties, initial, and essential conditions. The names of these files must include the name of the model and the extensions:

-regions.txt

-configuration.txt

The regions file is needed only if a three-dimensional mesh from ICEM's tetra mesher is used as an input. Mesh files generated by TETRA are recognized by their file extensions (*.asc) and (*.dat) and contain subregion information and mesh-connectivity data, respectively. Following the headline and a property specification line, the regions file lists a selection of model subregions which shall be used inside the simulation, e.g.:

‘ICEMexample-regions.txt’ created by SKM 2/3/01

no properties

FRACTURE1

FRACTURE2

CARBONATE_MATRIX

SANDSTONE_MATRIX

All other discretized regions in the (.asc) input file are ignored. The user has to make sure that the model is still contiguous and retains a box shape if respective methods of the CSMP++_DataInputManager shall be called. The “-regions.txt” file can also be used to assign initial property values to the selected regions (see documentation of interface library).

The “-configuration.txt” file has the following format (listing all possible data blocks). Comments are preceded with a pound (#) sign:

Reservoir simulation testcase (1): reservoir with fractures 30/5/2001 # file header

# 1. subregions of model identified from the permeability values assigned to

#    individual finite elements by the meshing tool

#    (NB: This datablock does NOT assign properties to elements but it will create

#     Region objects with the names ‘joints’ and ‘oil pool’ which contain elements

#     within the specified permeability ranges)

#    This data block is not needed in ICEM models because model subregions are

#    already identified by their family name in the CAD model.

# ----------------------------------------------------------------------------------

joints		permeability	1.0e-13	1.0e-1

oil pool	permeability	1.0e-14	1.0e-14

# 2. default material properties and initial variable values

porosity					0.3

total system compressibility			1.0e-9

pore radius					1.0e-4

pore throat radius				0.25e-4

fluid volume source				0.0

capillary pressure oil				0.0

density oil					800.0

density water					1000.0

viscosity oil					1.0e-4

viscosity water				1.0e-4

saturation oil					0.0

saturation water				1.0

fluid pressure					1.0

# 3. properties which shall be assigned to model subregions =Groups

#    subregion properties can be assigned to the subregion boundary, interior,

#    or the complete subregion

# ----------------------------------------------------------------------------------

joints		complete	porosity		1.0

joints		complete	pore radius		1.0e-3 # joint aperture

joints		complete	pore throat radius	1.0e-3

oil pool	complete	permeability		1.0e-13

oil pool	complete	saturation oil		0.9

oil pool	complete	saturation water	0.1 # well parameters

# 4. essential conditions which shall be assigned to model boundaries

# ----------------------------------------------------------------------------------

# here as many values are given as there are corner points that shall be interpolated

# between (2 in 2D models with line boundaries and 4 in 3D models with rectangular

# boundary surfaces

TOP	Dirichlet	fluid pressure		1.0E+07	1.0E+07

LEFT	Dirichlet	fluid pressure		1.0E+06	1.0E+06

RIGHT	Dirichlet	fluid pressure		0.0E+00	0.0E+00

TOP	Dirichlet	saturation oil		0.0	0.0

TOP	Dirichlet	saturation water	1.0	1.0

BOTTOM	Dirichlet	saturation oil		0.0	0.0

BOTTOM	Dirichlet	saturation water	1.0	1.0

RIGHT	Dirichlet	saturation water	1.0	1.0

RIGHT	Dirichlet	saturation oil		0.0	0.0

LEFT	Dirichlet	saturation water	1.0	1.0

LEFT	Dirichlet	saturation oil		0.0	0.0

# 5. (optional) essential condition flags for model subregions

# ----------------------------------------------------------------------------------

joints		complete	capillary pressure oil		Dirichlet

joints		interior	saturation oil			Dirichlet

joints		interior	saturation water		Dirichlet

Datablocks (including the file header) must be separated by an empty line. A problem that users frequently have with the configuration files relates to the use of tab and blank characters: Since, region and variable names are allowed to contain white space “ “, columns must be separated by tab characters (this feature exists as an output option for Excel spreadsheets which can be used to generate the configuration files). Also, end of line characters must not be preceded by whitespace.

When the configuration file is read by by the DataInputManager it checks variables against the PropertyDatabase and reports range violations or if a variable does not exist. If a range is exceeded, the variable value will be set to the nearest range bound specified in the global_variables_text file. If the variables are vectors or tensors, a corresponding number of values and flags need to be set. This number depends on the dimensionality of the model (for details see the documentation of the CSMP++_DataInputManager in the HTML reference guide).

### Regular Meshes from Pixel Data

A fast and simple way to create rectangular two-dimensional models for CSMP++ is to use 8-bit-color-coded bitmap images to resolve geologic features in models. These representations are converted by a triangular finite-element-mesh generator (Triangulator class) into evenly refined meshes. Input geometries are drawn either directly in “pixel paint” programs like GIMP or Scion Image or with standard drawing programs like Freehand, Illustrator, Canvas and so forth. In the drawings, structures and horizons are color-coded for permeability and regions of interest for the modeling are clipped. Scion Image (formerly NIH Image) permits the output of selected image regions as 256-color pixel dumps in text file format.  These ASCII files can be read by CSMP++. On unix / linux systems other drawing programs are available in the public domain. Below, the regular-mesh-input generation procedure is described in detail for the freeware software tool Scion Image and tips are given for drawing and manipulating geometries.

The following steps are required for the production of a color-coded text input file:

- Draw the Geologic cross section

- Import the section into Scion Image

- Color-code the section using a 256-value (8-bit) rainbow scheme

- Output the section as an ASCII text file

Step 1 can be  done with any drawing program that supports bitmap output.

#### 1. Drawing the Geologic Cross Section

As the first step towards the two-dimensional CSMP++ model, the geologic cross-section needs to be prepared in which each geological unit or structure has a unique color of the rainbow color scheme (In Scion Image: Options -> Color Tables -> Rainbow). The horizons must be filled with a unique color and must not be framed with black lines as these would be interpreted as separate low-permeability units (see below). Care has to be taken to avoid blank areas between the horizons. The most convenient way of drawing a horizon with polygon tools is to use the same line color as the fill color of the polygon.  Smoothing of the lines is recommended. However one needs to check afterwards with the magnification tool whether the horizons still overlap.

The generated drawing can be labeled further as a useful documentation of the model (publishing, presentation etc.).  Since one may want to modify the section in the future or even build an adaptively refined version of it using procedure (2) from above, it is advisable to keep a copy in the format of the original drawing program before proceeding with the next steps.

#### 2. Importing the Section into an Image Editing Program

Most image editing programs can import bitmap, PICT- and TIFF files. If you did prepare the model in another drawing program, save a copy of your model file (with a new name) as TIFF file. Now open this file with the image editing program like NIH image or GIMP and choose a Rainbow color palette. Once the file appears on the screen, use the selection tool (stippled quadrangle in the upper right of the tools menu) to extract the region of the model (geologic cross section), which you intend to input into your CSMP++ model (or use the whole section). If the section is too large to fit on the screen use "Scale to fit window" from the Options menu before using the selection tool. One can also scale drawings with NIH Image. Since the model is build from pixel data, the screen size of the geological cross-section controls model resolution. For test runs, the model should not be larger than 100 by 100 pixels. This will lead to a computation size of about 30-50 MB RAM in CSMP++ depending on how many physical variables the model has. A 1000 x 1000 image will translate into a 2000 x 2000 triangle mesh (4 millions elements), requiring about 1.5-2.3 GB RAM (Fig. 13).

#### 3. Color Coding the Image

The constructor for regular-gridded CSMP++ models (Model::Model( Triangulator& )) prompts the user for the name of the input text file and expects the input of property values of choice in the form of color-coded pixels. Use the paint bucket and pen tools to change colors in the image according to the 256-rainbow color vs. property value scheme. With this scheme, permeability values are assigned ranging from 0.1 nano-Darcy ( 10-21 m2) to 1.0 Darcy (10-12 m2):

10-21 		255		BLACK

10-20		0		WHITE

10-19 		236-254		RED

10-18 		216-236		ORANGE

10-17		206-215		PEACH

10-16 		183-205		YELLOW

10-15 		126-182		GREEN

10-14 		75-125		AQUAMARINE

10-13    		45-74		BLUE

10-12		1-44		AZURE

The desired integer color values (second column) are set by calibrating the pipette on the color bar in Scion Image. The color value chosen with the pipette tool appears in the “Info” window in the lower left hand corner underneath the color bar. Values with decimal places which may be output when Scion Image tracks color saturation (which is optional) are not permitted. One can vary values later in CSMP++ once a mesh has been created and Region objects can be build for each uniformly colored region.

Ideally the extracted region should now be complete for output as text-formatted CSMP++ input file.  This is achieved using "File -> Export -> text" from the file menu.  Before you output the file, test whether the color values are unique. The cross hair tool ( + symbol) will indicate the 256-color value when pointed at a horizon (Again, the tool should not indicate decimal color values, otherwise the text file cannot be interpreted by CSMP++). With the magnifying tool individual pixels of the drawing can be checked which normally are only poorly visible in the default view. This is necessary when converting pictures from drawing-program formats to TIFF since minor changes may have occurred in the image. Units must not have black outlines (color value 255) ! - If one finds any errors, the color picker (pipette tool) and pencil tools can be used to replace erroneous colors. In many cases, one can also changes whole areas with the paint-bucket tool (the color for this tool is also chosen with the pipette; choose it either from the image or from the palette=color bar). Like in Photoshop or GIMP, the paint bucket will color a mono-chromatic region bounded by other colors. Such regions will be filled-in completely with the new color.

a)          b)

Fig. 14: Correct (a) and incorrect (b) representation of a fracture in a 2D pixel input image for the Triangulator mesher inside of CSMP++.

One should give individual structures or units enough thickness to achieve contiguity in the finite-element models (Fig. 14). In a steady-state model, oblique fractures must be interconnected pixel chains to be represented as continuous high-permeability pathways. In models with fluid sources and sinks (e.g., wells), these units must be at least 3 elements across to get meaningful computation results for linear finite elements.

#### 4. Output of the section as color-coded text file

If the image satisfies the listed guidelines it can be exported as pixel dump in text format: Choose "Export" from the File menu and then click the text box. Keep a TIFF version of the geometry because it may need to be modified later. We recommend against using selected region output (“Export Selection as...text'') as input files for CSMP++ since region size almost always differs between manual selections or they are slightly offset relative to one-another. This can render work useless if one later wants to compare simulation results. Instead, one extracts only one selection from the original geometry file and make copies with alternate color coding. This assures that all output files have the same size and the same reference frame.

When the Model constructor reads the input file, it asks the user to specify the actual dimensions of the model. This is necessary because the pixel-data lack this information. In order to get optimal finite-element shapes, one should keep the aspect ratio, when specifying model dimensions.

A Model model is build from the ASCII text file in the following code segment:

Triangulator  linear_triangle_mesher;

Model<2>      model( linear_triangle_mesher );

This specific Model constructor will prompt the user to enter the name of the input file and to specify the actual dimensions of the model as this information is not conveyed by the image.

### Output via the VTK Interface

An effective way to visualize CSMP++ simulation results is instrumentalized by the CSMP++_VTK_Interface object for the visualization toolkit (VTK) supported by kitware.com. VTK is a very powerful C++ library of visualization algorithms and tools for structured and unstructured data, albeit with somewhat limited interactivity. Normally one will use VTK with the MayaVi GUI from sourceforge.net but one can also write visualization scripts in Tcl, Python, Java or C++. Apart from an impressive sample suite which comes with VTK, a suite of SKM-generated Tcl scripts demonstrating many features relevant to CSMP++-related visualization issues, is provided with the CSMP++ Distributed Example set.

As the perhaps most severe limitation as seen from a CSMP++ perspective, VTK does not directly support the display of scalar element properties – oddly – as this is easily done in OpenGL by using flat shading. However, VTK’s data model does not support this. Due to this limitation, we have chosen to output CSMP++ element properties as point data to VTK. These can therein be connected by means of a Delauney triangulation if continuous scalar fields are to be displayed. However, material boundaries will be blurred using this approach. Alternatively, element properties can be extrapolated to finite-element nodes or visualized with circular VTK glyphs.

The use of the interface to VTK is simple. The interface object is instantiated and its interfaces are called as is shown in the code example below for an instance of a Model named “box” and for a Region identified by the string “fault”:

CSMP++_VTK_Interface<3>  vtk_output;

//                          S.G.  filename        variable name   timestep

vtk_output.OutputDataToVTK( box, "nodal_test",      "nodal_test", 0 );

vtk_output.OutputDataToVTK( box, "fluid-pressure",  "fluid pressure", 0 );

//                     S.G.  groupname  filename   variable name  timestep

vtk_output.OutputDataToVTK( box,"fault","nodal_test","nodal_test",tstep );

### Output via the JPEG Interface

For convienience and to save space when monitoring runs, scalar variables from two-dimensional models can be output directly to JPEG image files. There are two classes for this purpose: CSMP++_JPEG_Interface and CSMP++_JPEG_GroupInterface. As the names suggest, the first outputs entire two-dimensional models and the second outputs subregions identified as groups. Output is generated as follows:

CSMP++_JPEG_Interface  jpg_output;

//                                  file name         property name

jpg_output.OutputDataToJPG( model, "fluid-pressure", "fluid pressure" 0 );

In this operation, CSMP++ maps the variable values to a regular grid corresponding to the raster image and it associates a greyscale- or rainbow color scheme with these data. By default, the actual value range determines the color-mapping. Red (black) generally marks the highest values, and blue (white) marks the lowest.

Alternatively, a fixed value range can be set by the user. This is useful when images are to be strung together as animations.

A caveat of the JPG interface lies in the number of pixels of the auxiliary grid. Pixel size is automatically set to 1.6 of the inner radius of the smallest finite element in the mesh. If a model locally contains very small elements, the memory required for the grid will be excessive causing CSMP++ to terminate with an error message.

## 8. A Comprehensive Simulation Example: Carbonate Dissolution

This chapter is a step by step description how to carry out a reactive transport simulation with a feedback between carbonate dissolution and permeability, which (after thousands of iterations) will lead to wormholing. The source code for this example is provided in the examples directory on the distribution CD (example18.cpp). To achieve a certain degree of independence and completeness justified in review of the material covered thus far,  this chapter repeats some of the concepts presented earlier and is written like a checklist to ascertain a safe landing of the CSMP++ “pilot”.

### 0. Input Geometry

Build a geometrical model which will serve as an input geometry for the calculation; for instance:

- Use NIH Image (current version >1.66) to create a rainbow color-coded representation of the model geometry and save it as TIFF file. Export an additional copy as text file from which a CSMP++ mesher will automatically generate a regular triangular mesh with dimensions 2*pixel rows x 2*pixel columns. For simple runs limit the image dimensions to <50 x 100. The color values will be translated into permeabilities using the factors defined in “CSMP++_DataInterpreter.h.” (see table in Input/Output chapter).

- Build a PSLG (planar straight line graph) of the geometry. It will consist of closed polylines with a material point to which its properties are attached. The PSLG is output to a dxf file and converted by the GeometryPreprocessor.exe to an input file for the Triangle mesher. This mesher produces a “*.poly” file with a constrained, conforming Delauney triangulation (see Model Building & Interface Guide for step by step instructions regarding this procedure).

### 1. Physical Variables

The desired physical variables must be communicated to CSMP++. This is accomplished via a text file which initializes the CSMP++ variable database (as managed by the Property class object). This file is named “CSMP++_carbonate_variables.txt”. Recall from Chapter 2 that notation in the file’s headline refers to symbols used for writing equations with the variable; units are SI units, and the index tells CSMP++ what type of variable shall be constructed (scalar (index=1), a vector (2) or a tensor (3)). Min. max. defines a physically meaningful range for each new variable. All variables which appear in the reaction file must also be represented in the physical-variables database.

### 2. Material Properties, Initial values, and Boundary Conditions

For high-level interactions with CSMP++ one most commonly uses the interfaces (functions) of the Model object. All of these are documented in the HTML documentation which can be browsed with any internet tool. For instance, one may set a fluid-volume source term to zero on all elements and assign Dirichlet boundary pressures on the left side of the geometry. This is done by the following code:

//    Model interface, condition, your variable,  its desired value

model.InputPropertyValue("fluid volume source", makeScalar(ANY,0.) );

model.InputPropertyValue ("fluid pressure", makeScalar(DIRCH,1.0e+6) );

One must always initialize variables and specify at least one Dirichlet boundary value, before one carries out a computation.

The model geometry, as expressed in terms of spatial variations in the permeability, is perhaps the most important initial condition for the carbonate dissolution model. It ultimately determines where “wormholing” and “sealing” will occur by dissolution and precipitation, respectively. While one can try the carbonate-dissolution program with many different input geometries, the input file set “fractures.1” is recommended as a default. After starting the program, one enters this filename, when prompted for it by the program.

### 3. The Interrelations

To compute the fluid-pressure distribution using the partial differential equation based on Darcy's law, one must first calculate hydraulic conductivity values from the permeability values. This is done with an Interrelation object from a library of already implemented Interrelations.  In summary, to use an existing Interrelation one must:

1. Include its definition into the program file before main():

#include "ConductivityFromPerm.h"

2. Create an instance of the Interrelation object:

//                                        Pa.s

ConductivityFromPerm<2>  relation( p_ref, 1.6 );

Here, the viscosity of the fluid is supplied as second argument to the interrelation subclass constructor.

3. Execute the Interrelation by passing it to the Model object:

model.Apply( relation );

After this step, the variable "conductivity" will have been assigned the desired value. Alternatively one may derive own Interrelations from the Interrelation base class. The code for this is as follows:

1. declare class name for the preprocessor

2. include necessary declarations of base class and CSMP++ variables

3. derive the subclass 'ConductivityFromPerm'

4. construct the Interrelation object

5. specify how the result property is calculated from the input properties. Here one makes use of the overloaded capabilities of Operand objects:

Always close a preprocessor ‘ifdef’ statement from above with #endif (omitting this may send the compiler into an infinite loop if this file is used in several source code files).

### 4. The calculation

How to compute a steady-state fluid pressure distribution in a model, for spatial variations in the permeability and Dirichlet (=fixed pressure) boundary conditions was already discussed in Chapter 2 for  the code example "example1.cpp".

In summary, one first constructs a Model representing the discretized version of the physical model. Secondly, one specifies initial and boundary conditions. Thirdly, one builds an PDE_Integrator with PDE operators and perhaps interrelations among the physical variables used. Finally, the computation is carried out and the results are output for display. The computed interstitial flow velocity field is used further by the FiniteVolumeAdvection class to advect the concentrations of the aqueous solutes on the finite volume mesh. After each advection step, speciation calculations are carried out with the AlterationVisitor to equilibrate the fluid with the rock.

### 5. The Regions

In most geologically-oriented models one will discover the need to address specific model regions selectively to obtain local results, assign regional properties, and/or restrict the application of Interrelations and/or PDE_Integrators to specific regions of the model. Fir this purpose, group objects are defined by:

model.FormAndAddGroup( "high perm", "permeability", 1.0e-12, 1.0e-5 );

This line of code will include the model region where permeability ranges between 1.0e-12 and 1.0e-5 m2, in a group with the name “high perm” (names are allowed to contain white-space). The following statements assign new porosity values to the elements in the new group and along the group boundary, respectively

sg.ChangePropertyInGroupToWhere( "high-perm", "porosity", 0.4, PLAIN );

sg.ChangePropertyInGroupToWhere( "high-perm", "porosity", 0.4, BOUNDARY );

### 6. The Visitors

Remember that the term visitor comes from the object-oriented software design pattern named Visitor. It facilitates a traversal of the Model hierarchy of Groups, Elements, Nodes, and ConstraintPoints. A visitor in CSMP++ can therefore visit all Groups, Elements, ConstraintPoints, and Nodes of a Model object while accessing these hierarchical objects via their public interfaces. The example at hand, introduces the usage of the “TransportVisitor” and the “AlterationVisitor”.

### 7. FiniteDifferenceGrids

Apart from unstructured-mesh finite-element capabilities, CSMP++ also supports structured, regular gridded data. Here they are used in the output of the results to JPEG images. Regular grids can be mapped to finite-elements and vice versa, permitting mesh-assignment of geophysical data from regular-gridded arrays and the output of mesh variables to structured-array visualization tools. If animations shall be produced, pixel-type regular-gridded data can be output to JPEG files or pixmaps, using the finite-element mesh interpolation functions. Regular grids are represented in CSMP++ by “FiniteDifferenceGrid” objects and can also be used for finite-difference calculations, kernel type operations, image processing etc. To build an instance of a finite-difference grid inside a CSMP++ main() program one writes

FiniteDifferenceGrid  mygrid;

mygrid.Initialize( x0, x1, y0, y1, RESOLUTION_x, RESOLUTION_y,

framewidth_x, framewidth_y );

An advantage of grids is that they require much less storage than a comparably refined finite-element mesh. For this reason the carbonate-dissolution example uses a FiniteDifferenceGrid for the advection of aqueous species.

### 8. Speciation Calculations

In the carbonate dissolution example, reactions are treated as rate laws using a forward rate and the inverse of the equilibrium constant. Via the law of mass action, the typical equilibrium expressions are replaced by ordinary differential equations (ODE's) which are written in the form:

A + B = C 	is replaced by  	dA/dt = -kf [A][B] + kf [C] / K

where A and B are the reactants, C is the reaction product, kf is the forward (far from equilibrium reaction rate and K is the equilibrium constant. Because equilibrium is defined such that

kf [A][B] = kb [C] 	and 	K = [C] / [A][B]

it follows that

K = kf/kb

at equilibrium. Therefore, one can express kb in terms of K as is done in the equation above. This kinetic approach is outlined in detail in Carl Steefel’s article in Reviews in Mineralogy 34, p. 93ff. For the carbonate-dissolution example, the system of ODE's is:

//  rate of formation is equal to –forward reaction rate + backward reaction rate

dOH_1m/dt   = -KF H_1p OH_1m    +  KB1 H2O_aq

dCO2_aq/dt  = -KF CO2_aq H2O_aq +  KB2 HCO3_1m H_1p

dCC/dt      = -KF CC H_1p       +  KB3 HCO3_1m Ca_2p

dH_1p/dt    = -KF H_1p OH_1m + KB1 H2O_aq +

KF CO2_aq H2O_aq + -KB2 HCO3_1m H_1p +

-KF CC H_1p + KB3 HCO3_1m Ca_2p +

dH2O_aq/dt  =  KF H_1p OH_1m    + -KB1 H2O_aq + -KF CO2_aq H2O_aq +

KB2 HCO3_1m H_1p

dHCO3_1m/dt =  KF CO2_aq H2O_aq + -KB2 HCO3_1m H_1p +

KF CC H_1p + -KB3 HCO3_1m Ca_2p

dCa_2p/dt   =  KF CC H_1p       + -KB3 HCO3_1m Ca_2p

where KBx is the inverse of K. The products in the equations which produce the left-hand species always have a rate coefficient with a negative sign. Also, as follows from the law of mass action, stoichiometric factors in the conventional equations like 2A+B=C translate into dependent variable exponents (2A -> A2).  Stoichiometric factors must also be taken into account in the rate coefficients. Thus, if a reaction produces 2A, this implies for the total rate at which A is produced or consumed that this reaction contributes twice the amount of A. Consequently, both, the forward and the backward rate must be multiplied by 2 in any expression involving A. This shall be illustrated using a wollastonite producing reaction:

WOL + 2 H_1p  = Ca_2p + SiO2_aq + H2O  (1/equilibrium constant = KB1)

The contribution of this reaction to an ODE which describes the H+ concentration is:

d H_1p/d t = 2 -KF WOL H_1p^2  +  2 KB1 Ca_2p SiO2_aq H2O_aq + ....

since, during every reactive exchange, 2 moles of H+ are produced or consumed. Stoichiometric factors must be multiplied into forward and backward rates, since the equation parser in the ODE solver cannot deal with 2 coefficients in a single product term. Alternatively, the number 2 is defined as an additional independent variable. This, however, is more costly than just generating

KB1x2 = 2 * KB1

The reaction ODEs are integrated over the reaction time interval with a Burlisch-Stoer solution scheme for stiff ODE's (Bader and Deuflhard 1986). This scheme is implemented as a “StiffODESolver” object which has a method that reads the example text file "calciteOH_25.txt" with the specification of the ODEs as above. The equilibrium constants and forward reaction rates for the corresponding reactions at 25oC are also defined in this file.

### 9. Initial Conditions

Before the simulation of reactive transport is started, the fluid on the grid is equilibrated with the rock via a visitation of each finite element by the alteration visitor. Before this visit, the grids are initialized to zero and the equilibration time given to the visitor is set to a very large value (in seconds) to ascertain that the fluid fully equilibrates with the rock. The speciation calculations are carried out element by element as the AlterationVisitor visits them. The code which specifies how this is done is contained in the method,

virtual void AlterationVisitor::Visit( Element* e );

a purely virtual function of the base class CSMP++_FEM_Visitor.  The Visit() method does the following:

1. Read the speciation of the fluid from the last timestep from the Node and assign it as initial condition to the ODE solver object.

2. Obtain the new speciation in the considered rock volume for 1 kg of fluid.

// maxtime is just the time needed for full equilibration (no other meaning)

//             maxtime        init.t-step  save-steps  desired tolerance

solver.Solve( time_increment,  1.0e-17,       0,       fmol*tolerance );

3. Calculate how much calcite was dissolved from the difference of the Ca+2 molality between the old and the new speciation in 1 kg of fluid. Convert this amount to a unit volume of the fluid-rock assembly. Store the result back to the node.

// dissolved calcite is derived from change in Ca+2 concentration

// result is converted into kg

// information is stored in the basis species vectors

n->Store( pmem, cc_key.index,

(sc=BASIS[2]+(BASIS[6]-solver(7))*100.08*1.0e-3*fmass) );

### 10. The Overall Model

In the main program, reactive transport is simulated via a sequential iteration approach (SIA) as discussed by Yeh and Tripathi (1989, 1991) and summarized in Steefel (1996, see reference above). This technique means that transport and speciation steps are decoupled. An additional complexity arises as calcite-dissolution increases porosity which, in turn, increases the permeability. This is implemented via two Interrelation subclasses 'PorosityChange' and 'PermeabilityFromPorosity'. They are passed to the Model after each transport- and equilibration- pair of steps. Subsequently, the Darcy flow velocity distribution at t+1 is recomputed as was discussed  in Chapter 2.

The carbonate dissolution alters the previous fluid-velocity distribution to the model state at t = previous time + time_increment.

To drive the chemical reactions, after each advection step, an acid but charge-balanced fluid composition is specified on the left margin of the grids. This acid fluid is advected to the right into the cross-sectional model where it dissolves carbonate until it is neutralized by the rate-controlled reaction with calcite.

The model is run while the H+ concentration on the first of the grids and the “porosity” are output from time to time to DXF and VTK files, respectively, to monitor the progress of carbonate dissolution.

## 9. The CSMP++ Libraries

The CSMP++ libraries are written in layers which are increasingly high-level and specific in terms of the functionality which they provide. Thus, while the standard CSMP++ library contains the base classes for most objects, as are listed in the CSMP++ Distribution Section of Appendix A, additional libraries contain  subclasses derived from base classes in the main library, such as the PDE_Integrator, CSMP++_FEM_Visitor, Interrelation, MathOperatorLHS, MathOperatorRHS, FiniteElement, etc. classes. CSMP++ ships with the following libraries:

- algorithms

- applied_math

- computational_geometry

- computer_science

- finite_elements

- generic_finite_volumes

- geometry_preprocessor

- hydrothermal

- main_library

- parallel

- pde_operators

- solver

- triangular_finite_volumes

- interfaces

- interrelations

- unit_test

- visitors

These libraries are continuously supplemented by additional objects from CSMP++ developers. These publicly generated extensions are made available to other CSMP++ developers and users such that they can tap this growing reservoir to build increasingly realistic simulations.

All CSMP++ libraries depend on the main library. Clearly, as the CSMP++ examples code is meant to demonstrate the capabilities of CSMP++, it must be linked with all of the CSMP++ libraries in order to function. The examples code combines a choice of functions in a main program. These functions perform a wide variety of computations, including the examples referred to earlier in this User Guide.

Any CSMP++ class consists of a header file which contains the declaration (*.h) and a source code file (*.cpp) that contains the definition of the class. Only developers have access to the definitions.

### /source_code/algorithms/ Library

IterativeIntegrator

NewtonIntegrator

PicardIntegrator

UpwindIntegrator

SteadyStateDiffusionAlgorithm

TransientDiffusionAlgorithm

PDE_Integrators can be assembled for forward, central, and backward difference time stepping. In matrix notation these different procedures as illustrated for fluid pressure (p) diffusion (see chapter 2) are:

Forward Difference (=explicit)

[C]{p}t+t = ([C] – t[K]){p}t + t {Q}t

Central Difference (semi-implicit = Crank-Nicholson)

([C] + t/2[K]){p}t+t = ([C]- t/2[K]){p}t + t/2 ({Q}t+ {Q}t+t)

Backward Difference (=implicit = Backward-Euler)

([C] + t[K]){p}t+t = [C]{p}t + t {Q}t+dt

Here C, K, and Q refer to the capacitance, conductance and fluid source matrices.

### /source_code/applied_math/ Library

Array

CSMP++_ANSI_ISO_patches

CSMP++_DenseMatrix

CSMP++_ErrorFunction

CSMP++_LoopUnroller

CSMP++_Matrix

CSMP++_matrixMethods

CSMP++_SparseMatrix

ODE_StiffSolver

EigenFinder

Log10Property

MaximumResidual

RandomFloat2

SparseMatrixCondensedRowStorage

SumOfProducts

SumOfProductsWithExponents

Tolerance

### /source_code/computational_geometry/ Library

MJL_Edge

MJL_Edge3D

MJL_Geometry

MJL_Point

MJL_Point3D

MJL_Polygon

MJL_Triangle3D

Triangulator

### /source_code/computer_science/ Library

KD_BinaryTree

MBST_Node

SK_DataObject

### /source_code/eos/ (Equation of State) Library

eos_h2o/decl

iaps

meta

more

steam

eos_nacl_h2o/

Brine

Fluidproperties

MetastableWater

Vapour

CriticalCurve

HaliteLiquidus

NaCl

Viscosity

Fluid

LVTwophase

ThreephaseHLV

Water

H2OPropertiesLookUpTable

IAPWS_H2O_HeatTransferVelocity

IAPWS_H2O_Storativity

IAPWS_H2OPropertiesVisitor

IAPWS_H2O_dVdT_Source

IAPWS_H2O_Conductivity

IAPWS_H2O_Density

NaClH2O_PropertiesVisitor

NaClH2O_TotalHeatCapacity

NaClH2O_TransportPropsVisitor

SalinityFromMassFractionSalt

The library {CSMP++3D-eos contains the implementations of the equation of state for pure H2O and NaCl-H2O, as well as interfaces to them, {Visitor and Interrelation classes for fluid property calculations, and a class to generate a lookup table for the pure H2O fluid properties (Fig. 3). Using the CSMP++3D-eos library, fluid and transport properties for single-phase thermal and thermohaline convection simulations can be calculated.

The C-library PROST (Bauer, 2002), which has implemented the NBS/NRC Steam Tables (Haar et al., 1984), is included in the CSMP++3D-eos library to compute fluid properties of pure H2O. The H2OPropertiesLookUpTable class employs PROST to generate a 105 MB lookup table for the pure H2O properties density, enthalpy,

viscosity, compressibility, isobaric heat capacity, and thermal expansivity. The lookup table is stored as a binary file generated from a FiniteDifferenceGrid object. It is valid from 5 to 800ºC and 1 to 4000 bar pressure. Bi-linear interpolation is used to extract the fluid properties. The grid spacing is 1.0ºC and 2.5 bar away from the critical point of H2O and 0.1ºC and 0.1 bar in its vicinity. In addition, the properties for the liquid and steam phase can be obtained along the saturation curve of pure H2O for a given temperature or pressure value. The error of the interpolated fluid properties is below 0.01 % close to the critical point and below 0.0001 % away from it.

The IAPWS_H2O_Density class is an interface to PROST and may be used to obtain fluid properties as a function of temperature and pressure. The IAPWS_H2OPropertiesVisitor class employs PROST to compute nodal fluid properties as a function of nodal pressure and temperature values. The object terminates a simulation if non-physical, i.e. negative pressure or temperature values are encountered. It does not make a distinction between liquid and vapor. Hence it cannot be used to accurately model two-phase liquid-steam flow. The Interrelation classes IAPWS_H2O_Storativity and IAPWS_H2O_dVdT_Source compute the element variables 'storativity', respectively 'thermal expansion', as a function of the

fluid and rock compressibilities, respectively expansivities. The Interrelation object

IAPWS_H2O_Conductivity calculates the element variable 'conductivity' from the element permeability and the nodal fluid viscosities. The Interrelation object IAPWS_H2O_HeatTransferVelocity calculates the element variable 'heat transfer velocity' from the relation (v cp )/((1-)cpr  r +  cp ) using the element Darcy velocity, rock density, and rock heat capacity as well as the nodal fluid density and fluid viscosity.

The NaCl-H2O equation of state can be accessed through the interface Fluid. The phase state of the fluid and the properties density, enthalpy, viscosity, saturation, heat capacity, mass fraction salt, and compressibility are returned for the brine, vapor, and halite phase as a function of pressure, temperature, and salinity. If a phase does not exist, its values are set to zero. The NaCl-H2O equation of state employs a H2OpropertiesLookUpTable object to retrieve the properties of pure H2O, because pure H2O properties must be computed repeatedly at small temperature, pressure, and salinity increments away from the values of interest. The object NaClH2O_PropertiesVisitor calculates nodal fluid properties as a function of nodal pressure, temperature, and salinity. It terminates a simulation if non-physical, i.e. negative, pressure or temperature values are encountered. A NaClH2O_PropertiesVisitor object does not return properties of a vapor phase. Instead, the simulation is terminated if vapor is present. The object NaClH2O_TransportPropsVisitor computes the element variables 'conductivity, 'storativity', and 'total source' as a function of nodal fluid and element rock properties. The Interrelation object SalinityFromMassFractionSalt converts the mass fraction salt into salinity, accounting for the possible presence of halite.

Fig. 9.1:. UML class diagram for the calculation of thermodynamic fluid properties. Associations between classes are denoted by a bold line. The arrow notes the direction of flow of the associations. The diamond denotes a composition.

### /source_code/finite_elements/ Library

IsoparametricLinearHexahedron

IsoparametricLinearLineElement

IsoparametricLinearPrism

IsoparametricLinearPyramid

IsoparametricLinearQuadrilateral

IsoparametricLinearTetrahedron

IsoparametricLinearTriangle

IsoparametricQuadraticHexahedron

IsoparametricQuadraticLineElement

IsoparametricQuadraticQuadrilateral

IsoparametricQuadraticPrism

IsoparametricQuadraticPyramid

IsoparametricQuadraticTetrahedron

IsoparametricQuadraticTriangle

LinearLineElement

LinearTetrahedron

LinearTriangle3D

LinearTriangle

This documentation only comprises the triangular-tetrahedral family of finite elements. Other types are documented in the developer’s guide. Triangular-tetrahedral elements comprise the types, the LinearTriangle, LinearTriangle2D (3-nodes), the IsoparametricQuadraticTriangle (6-nodes), the IsoparametricQuadraticBarycentricTriangle (7-nodes), the LinearTetrahedron (4-nodes), and the IsoparametricQuadraticTetrahedron (10-nodes). Linear triangular and tetrahedral input meshes can be transformed for the quadratic element types with the CSMP++_VSetConverter. Hybride meshes with both triangular and tetrahedral elements have been used to model fluid flow in highly permeable but narrow fault zones.

#### The Linear Triangular Element: LinearTriangle

Linear interpolation functions, L, for the triangular element are equivalent to the area- or so-called natural  coordinates of a straight-sided triangle

where a,b,c are linear coefficients and x,y are the Cartesian coordinates of the nodes of the triangle. The sums in L are divided by two-times-the-area, A of the triangle to achieve that L1 to L3 sum up to 1. The area coordinates of a point x,y in a triangle are linearly related to its Cartesian coordinates. Thus

Here, the indexing of the Cartesian coordinates in the matrix refers to the node numbering in the triangle (Fig. 1).

Fig. 9.2: Counterclockwise node numbering in linear triangular element.

The coefficients of the area-coordinate functions are calculated from the Cartesian coordinates of the nodes n1 to n3

The area of the triangle is

The spatial derivatives of the interpolation functions (N=L) in generic matrix form are

[dN]

This matrix is expanded for linear elasticity problems, where each node has 2 degrees of freedom (x,y displacements). This expanded form is also known as the ‘B’ matrix:

For the linear triangle, the interpolation functions can be integrated analytically over the area of the element, using the general formula

where l, j, k represent exponents of the area-coordinate functions. These exponents are equivalent to the degree of the interpolation functions and their products as they arise during matrix multiplication. To illustrate the use of the integration formula for a single entry into the finite-element matrix integral consider

As the partial derivative of the interpolation function with respect to x is a constant, it can be pulled out of the integral, and, since the interpolation functions N2 and N3  do not appear in the equation, their exponents are zero.

For the conductance matrix K integration in this manner for all terms gives

#### The LinearTetrahedron

The interpolation functions for the LinearTetrahedron are implemented similar to those of the linear triangle and the isoparametric barycentric triangular element. The only difference is that a third spatial coordinate is incorporated. This turns the interpolation functions into volume coordinates instead of area coordinates

Where ij (Kronecker-Delta) simply means that the interpolation functions are orthogonal to the weighting functions. Again, the normalization by 6V ascertains that the four orthonormal interpolation functions L sum up to 1 anywhere in the tetrahedron

Cartesian (x,y,z) and volume (L) coordinates have the relation

The Cartesian coordinate indices correspond to the four nodes of the tetrahedron. The determinant of the square (44) matrix is equivalent to 6V, if the nodes are numbered so that the sequence 1-2-3 runs counterclockwise when viewed from node 4 (Fig. 19).

Fig. 9.3: Four-noded tetrahedral finite element in an right-handed coordinate system.

The coefficients for the basis functions are computed from the global node coordinates

The remaining coefficients are obtained by cyclic permutations of the indices with a sign change for each permutation.

The volume of the tetrahedron is

Differentiation of the interpolation functions gives

The general analytical formula for the integration is

where k, l, m, n are equivalent to the exponents of the interpolation functions L1 to L4. Each exponent is unity for linear, 2 for quadratic, and 3 for cubic interpolation functions. The factor 3 in the denominator, is equal to the space dimension of the element. When an interpolation function integral only contains first order terms, the integral reduces to

(0! = 1). Evaluating, the volume integral ∫NT r N dV, one arrives at

Individual product terms are integrated as follows

and so forth, until one arrives at the usual integrals like that over the interpolation function products

Integration exponents for interpolation function derivatives are 0 since these functions are linear for the LinearTetrahedron.

#### The IsoparametricQuadraticTriangle with 6 Nodes

Quadratic triangular elements (Fig. 19) are used very often for elasticity problems. The quadratic order of the interpolation functions of this element makes displacement derivatives continuous across element boundaries (not across corner nodes) and special ways of numerical integration can rule out zero-energy modes of deformation, which lead to unrealistic results. The element can also be constructed for three-dimensional computations in which case 3D coordinates are transformed into the plane of the 2D local reference element. In linear-elasticity applications, mesh convergence can usually be obtained in contrast with linear elements, which give appaling results. The quadratic interpolation implies that displacement derivatives are represented as bilinear fields inside each element.

In the following, the steps are discussed of how one solves a linear elasticity problem using quadratic triangular finite elements, including the post-processing calculation of strains and stresses at the elements integration points.

Fig. 9.4: Reference (unit) quadratic triangle showing node numbering and local coordinates. The black midside nodes are used as integration points.

Given the reference triangle shown in Figure 20, interpolation functions Ni, (= shape functions since the isoparametric approach is used) in the local coordinate system, can be defined as:

N(3) = 4. * r * (1. - r - s)

N(4) = 4. * r * s

N(5) = 4. * s * (1. - r - s)

N(0) = 1. - r - s - N(3)/2. - N(5)/2.

N(1) = r - N(3)/2. - N(4)/2.

N(2) = s - N(4)/2. - N(5)/2.

and their spatial derivatives dN/dr, dN/ds, with regard to the local reference frame:

DNS(0) = -3. + 4.*r + 4.*s

DNS(1) =  0.

DNS(2) = -1. + 4.*s

DNS(3) = -4.*r

DNS(4) =  4.*r

DNS(5) =  4. - 4.*r - 8.*s

DNR(0) = -3. + 4.*r + 4.*s

DNR(1) = -1. + 4.*r

DNR(2) =  0.

DNR(3) =  4. - 8.*r - 4.*s

DNR(4) =  4.*s

DNR(5) = -4.*s

Jacobian Transformation. The use of a local coordinate system makes it necessary to transform element contributions, which are calculated on the reference element, into the global coordinate system. This is achieved via a Jacobian coordinate transformation. For this purpose one needs to form a Jacobian matrix, J, with the size of spatial dimensions x spatial dimensions (2 x 2 in 2D):

J  can be calculated as the product of the shape-function derivative matrix, DN, calculated at the point (r,s) in the local coordinate system, and the global node-coordinate matrix, XY (cf. Zienkiewicz, Vol I, (4th ed.) p. 162). With the inverse of J one can carry out the desired transformation of the local to the global shape-function derivative matrix, DN(x,y), but J-1 is only valid at the transformation point (x,y) of the local point (r,s) where the local shape-function derivatives were calculated. Therefore, J-1 must be calculated again at each point where derivatives shall be transformed into the global coordinate frame.

In summary, to calculate the global shape-function derivative matrix DN(x,y), at a point (x,y) representing the mapping of point (r,s), the following steps must be performed:

1. Form DN(r,s) and XY,

2. Form J = DN(r,s) * XY

3. Invert J

4. Premultiply J-1 with DN(r,s):  DN(x,y) = J-1 * DN(r,s)

As a next step, depending on the degrees of freedom per node of the problem which shall be solved, DN(x,y) must be expanded into a correspondingly larger matrix, B. Taking a linear elasticity problem in 2D, the nodal degree of freedom is 2, and the the often so-called B matrix is:

This matrix is built by substitution of the elements of  DN(x,y) into the corresponding locations. Since there is no analytical integration formula for the curved (deformed) isoparametric triangular element, numerical integration has to be used. The numerical integration formula presented below, however, is only exact for an undeformed (straight-sided) triangle.

Element Stiffness Matrix. The element’s contribution to the global stiffness matrix, K, given by the spatial integral

over the matrix product of B-transposed D B, where D is the 3 x 3 material property matrix, is approximated numerically as the sum

of such products computed at nip integration points in the triangle.  In this example, the midside nodes are chosen as integration points (dark nodes in Fig. 1, nip=3). |Ji| refers to the determinant of the Jacobian matrix which must be non-singular and wi, is a weight that is applied to the matrix contribution from each integration point. This weighting depends on the area of the reference element and the number of integration points chosen. Thus, with the reference element’s area being ½ and nip=3, it follows wi = 1/6 such that the integral terms span the area of 0.5. Due to the location of the integration points, all weights can be treated equally.

For the case of plane strain (no strain normal to the plane of the two-dimensional model) which is considered here, the D matrix incorporates the properties Young’s modulus, E, and Poisson’s ratio, nu:

Matrix Inversion. A common linear-elasticity problem where the dependent variable is the nodal displacement, is defined by the global stiffness matrix, K on the lefthand side and a prescribed displacement boundary-condition vector uv on the righthand side

K d = uv

The solution is returned into the nodal displacement vector d, with dimensions nodes x degrees of freedom per node. Initial stresses and strains, boundary stresses and body forces are ignored.

Strains & Stresses. Among others Barlow (1976, Int. J. Num. Meth. Eng. 10, 243-251) has shown that the best locations for strain (change of length and shape) and stress computations in the quadratic (and cubic) triangular element are the midside nodes. He recommends to linearily interpolate or extrapolate these to the element center or to the nodes, respectively, for the later usual visualization (linear interpolation is recommended, because derivatives of quadratic- are of course linear functions). If strains or stresses are interpolated to the nodes, an averaging of nodal values computed by the elements sharing the node is often applied since the stresses are not continuous from element to element when quadratic interpolation functions are used.

From the nodal displacements in the global coordinate system, dxi, dyi  where i refers to node i, the strains at each integration point are calculated as

Where j refers to the integration point j. This multiplication is equivalent to multiplying Bi, the expanded shape function x,y-derivative matrix (see above) at the integration point i (a midside node) with the nodal displacement vector, d(e), for the element:

The Bi are computed before the nodal displacements are applied to change the elements shape in global coordinates. The stresses at the integration points are finally computed from the strains as

Principal Strain- & Stresses Axes. The principal strain / stress axis are found as the Eigenvectors of the calculated strain and stress tensors

in which the off-diagonal elements are the shear strains / stresses. The Eigenvalues represent the principal strain /stress magnitudes (in the directions defined by the Eigenvectors). The Eigenvector matrix EIG contains the principal axis 1, 2, 3 in its corresponding rows.

Dilatation. The sum of the Eigenvalues of the strain tensor.

Mean Stress. The average of the principal stresses.

Testing. A series of tests can be performed to detect errors in the implementation of newly implemented elements like the discussed one. Thus, for any point inside the implemented element (including its boundary) and with reference to the global coordinate system:

- The interpolation functions must sum up to 1 (down to numerical precion).

- Their derivatives must sum up to zero.

- The sum of the products of interpolation function derivatives with nodal scalar variables must represen the gradient of a scalar variable prescribed on the nodes anywhere inside the triangle.

- The element volume as calculated by integrating the shape functioons over all integration points in the element must be correct.

- In an even-sided symmetric transformation of an elements shape, for instance  an equal-sided triangle, the length of the vectors defined by the interpolation functions derivatives at the nodes and integration points,  must be the same at opposite nodes. Thus, a transformation into the highest possible symmetry makes the testing easier.

Global to Local. Interpolation of property values at a point specified in global coordinates for mapped elements.

Property Value Gradients. If the gradient of a property value is desired at a global point (x,y), the global interpolation-function derivatives at the element’s nodes can be interpolated to this point using the elements interpolation functions. For this purpose, the global x,y-derivatives of shape function 1 at node 1, shape function 2 at node 2 and so forth are assembled into a derivative matrix DN (size spatial dimensions x nodes). These data are now interpolated with the element’s interpolation functions to the point of interest, DN(x,y), just as one would interpolate any nodal property value. The desired property-gradient is found by summing the products of the interpolated derivatives with the corresponding nodal property values:

where p,x(x,y) is the property gradient in the x-coordinate direction at the global point (x,y).

Inter-Element Continuity. Once an element has been verified in isolation, further tests can be devised for element patches in order to see whether the nth order derivatives of the shape functions are continuous across the sides of the elements and their corner nodes. For the quadratic triangle they are not continuous across the corner nodes.

#### The IsoparametricBarycentricQuadraticTriangle with 7 Nodes

The barycentric triangular finite element has quadratic interpolation functions involving an extra node in the element’s center. This particularity makes the element well suited for buoyancy-driven fluid flow and elastic-viscous deformation simulations. Since this element can be dramatically over integrated using a 7 or 13-point numerical integration scheme it is capable of very accurately representing integrated field variables like varying fluid density or body forces. The 13-point integration scheme is only recommended for calculations where buoyancy forces vary dramatically over short distances but the overall scale of the model dictates that the mesh has to be relatively coarse. An example of such a calculation would be a model of dike swarm intrusion. The node-numbering in a right-handed coordinate system and the splitting of the elemental area in areal coordinates is shown in Figure 9.5.

Fig. 9.5: The triangular barycentric element. (Left): Node-numbering scheme; (center): Natural (area) coordinate breakdown of the reference element; (right): Coordinate convention for the local coordinate system.

The origin of the local coordinate system is located in node 1 of the barycentric triangle and the limbs are stretched such that node 2 and 3 have the local two-dimensional coordinates (1,0) and (0,1), respectively.

##### Element Interpolation Functions in Global Coordinates

The BarycentricQuadraticTriangle is implemented as an isoparametric element using a local coordinate system and Jacobian transformations. However, element interpolation functions in global coordinates are still needed to interpolate property values at points whose coordinates are known only in the global coordinate system. Interpolation using this approach is correct only as long as the triangle remains straight sided.

As for the LinearTriangle element, interpolation functions in global coordinates are composed using area coordinates, L. The seven interpolation functions N1 to N123 are now expressed in terms of the area coordinate functions:

N123 is sometimes called the bubble function and is unique in that the corresponding node is not shared among elements. The first partial derivatives of these functions with regard to the node coordinates x are:

Again the analytical integration formula already presented for the triangular element could be used but this would become a fairly complicated expression:

##### Local Coordinates

Figure 9.5 illustrates a local coordinate reference frame for the barycentric element. Since local coordinates are used, local shape function derivatives required at the integration points are constants for the reference element. At the corresponding nodes the derivatives are:

| node | 1 | 2 | 3 | 12 | 23 | 31 | 123 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| i | -3 | 3 | 0 | 0 | 5 | -5 | 0 |
| i | -3 | 0 | 3 | -5 | 5 | 0 | 0 |

The use of a local reference element has the advantage that the sides of the triangle can deform into curvilinear shapes without compromising on the accuracy of the numerically evaluated integrals. The Jacobian matrix for each integration point is formed from the shape (=interpolation) function derivatives at this point and the elements global coordinates:

##### Numerical Integration

The integration points and their weights are defined in terms of the local coordinates of the reference element. The location of the points (IP7 or IP13, in L coordinates) and their weights (W7 or W13) are computed as follows:

Matrix IP7(7,3), IP13(13,3);

double g1, g2, g3, g4;

g1 = (6.0+sqrt(15.0))/21.0;

g2 = (6.0-sqrt(15.0))/21.0;

g3 = (9.0-2.0*sqrt(15.0))/21.0;

g4 = (9.0+2.0*sqrt(15.0))/21.0;

IP7(0,0)=1./3.;  IP7(0,1)=1./3.;

IP7(1,0)=g1;     IP7(1,1)=g1;

IP7(2,0)=g3;     IP7(2,1)=g1;

IP7(3,0)=g1;     IP7(3,1)=g3;

IP7(4,0)=g2;     IP7(4,1)=g2;

IP7(5,0)=g4;     IP7(5,1)=g2;

IP7(6,0)=g2;     IP7(6,1)=g4;

for ( i=0; i<7; i++ ) IP7(i,2) = 1.0 - IP7(i,1) - IP7(i,0);

IP13(0,0)=0.3333333;  IP13(0,1)=0.3333333;  IP13(0,2)=0.3333333;

IP13(1,0)=0.4793079;  IP13(1,1)=0.2603459;  IP13(1,2)=0.2603459;

IP13(2,0)=0.2603459;  IP13(2,1)=0.4793079;  IP13(2,2)=0.2603459;

IP13(3,0)=0.2603459;  IP13(3,1)=0.2603459;  IP13(3,2)=0.4793079;

IP13(4,0)=0.8697397;  IP13(4,1)=0.0651301;  IP13(4,2)=0.0651301;

IP13(5,0)=0.0651301;  IP13(5,1)=0.0651301;  IP13(5,2)=0.8697397;

IP13(6,0)=0.0651301;  IP13(6,1)=0.8697397;  IP13(6,2)=0.0651301;

IP13(7,0)=0.6384441;  IP13(7,1)=0.3128654;  IP13(7,2)=0.0486903;

IP13(8,0)=0.3128654;  IP13(8,1)=0.6384441;  IP13(8,2)=0.0486903;

IP13(9,0)=0.3128654;  IP13(9,1)=0.0486903;  IP13(9,2)=0.6384441;

IP13(10,0)=0.6384441; IP13(10,1)=0.0486903; IP13(10,2)=0.3128654;

IP13(11,0)=0.0486903; IP13(11,1)=0.6384441; IP13(11,2)=0.3128654;

IP13(12,0)=0.0486903; IP13(12,1)=0.3128654; IP13(12,2)=0.6384441;

Vector W7(7), W13(13);

W13(0) = -0.14957;

W13(1) =  0.1756152;

W13(2) =  0.1756152;

W13(3) =  0.1756152;

W13(4) =  0.0533472;

W13(5) =  0.0533472;

W13(6) =  0.0533472;

W13(7) =  0.0771136;

W13(8) =  0.0771136;

W13(9) =  0.0771136;

W13(10) = 0.0771136;

W13(11) = 0.0771136;

W13(12) = 0.0771136;

W7(0) = 0.225;

W7(1) = (155.0+sqrt(15.0))/1200.0;

W7(2) = (155.0+sqrt(15.0))/1200.0;

W7(3) = (155.0+sqrt(15.0))/1200.0;

W7(4) = (155.0-sqrt(15.0))/1200.0;

W7(5) = (155.0-sqrt(15.0))/1200.0;

W7(6) = (155.0-sqrt(15.0))/1200.0;

#### The IsoparametricQuadraticTetrahedron with 10 Nodes

Fig 9.6 show the node-numbering for the QuadraticTetradron finite element. This element is not seen as such during visualization because it is partitioned into subtetrahedra when meshes are output to VTK or other OpenGL based visualization tools as OpenGL shading can only represent linear property variations.

Fig. 9.6: The QuadraticTetrahedron with coordinate reference frame and node numbering scheme (NB: for consistency with the ICEM node numbering, nodes 8-10, now have a different placement, see HTML documentation of this element).

Like the other higher-order elements in CSMP++, the IsoparametricQuadraticTetrahedron is isoparametric and can therefore be used in models of progressive deformation where element boundaries become curved after the first displacement increment. The interpolation functions in local coordinates are

L1 = 1.0 - L2 - L3 - L4;

N(0) = L1 * ( 2.0 * L1 - 1.0 );

N(1) = L2 * ( 2.0 * L2 - 1.0 );

N(2) = L3 * ( 2.0 * L3 - 1.0 );

N(3) = L4 * ( 2.0 * L4 - 1.0 );

N(4) = 4.0 * L1 * L2;

N(5) = 4.0 * L2 * L3;

N(6) = 4.0 * L3 * L1;

N(7) = 4.0 * L2 * L4;

N(8) = 4.0 * L3 * L4;

N(9) = 4.0 * L1 * L4;

Note that L1 = 1 - r - s - t, L2 = r, L3 = s, L4 = t, see Bathe, p. 256 for midside nodes and Huyakorn & Pinder (1983) p. 91 for corner node interpolation function specifications, respectively.

The local partial derivatives of the interpolation functions at arbitrary points inside of the tetrahedron are with respect to the r local coordinate

DNR(0) =  1. - 4. * (1. - r - s - t);

DNR(1) = -1. + 4. * r;

DNR(2) =  0.;

DNR(3) =  0.;

DNR(4) = -4. * r + 4. * (1. - r - s - t);

DNR(5) =  4. * s;

DNR(6) = -4. * s;

DNR(7) =  4. * t;

DNR(8) =  0.;

DNR(9) = -4. * t;

and with respect to the s local coordinate

DNS(0) =  1. - 4. * (1. - r - s - t);

DNS(1) =  0.;

DNS(2) = -1. + 4. * s;

DNS(3) =  0.;

DNS(4) = -4. * r;

DNS(5) =  4. * r;

DNS(6) = -4. * s + 4. * (1. - r - s - t);

DNS(7) =  0.;

DNS(8) =  4. * t;

DNS(9) = -4. * t;

as well as with respect to the t local coordinate

DNT(0) =  1. - 4. * (1. - r - s - t);

DNT(1) =  0.;

DNT(2) =  0.;

DNT(3) = -1. + 4. * t;

DNT(4) = -4. * r;

DNT(5) =  0.;

DNT(6) = -4. * s;

DNT(7) =  4. * r;

DNT(8) =  4. * s;

DNT(9) =  4. * (1. - r - s - t) - 4. * t;

The local partial derivatives at the node points are

| node | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r=i | -3 | 3 | 0 | 0 | 0 | 2 | -2 | 2 | 0 | -2 |
| s=i | -3 | 0 | 3 | 0 | -2 | 2 | 0 | 0 | 2 | -2 |
| t=ξi | -3 | 0 | 0 | 3 | -2 | 0 | -2 | 2 | 2 | 0 |

All four integration points have a weight of ¼. Their locations in local coordinates are

IP(0,0) = 0.13819660, IP(0,1) = 0.13819660, IP(0,2) = 0.13819660;

IP(1,0) = 0.58541020, IP(1,1) = 0.13819660, IP(1,2) = 0.13819660;

IP(2,0) = 0.13819660, IP(2,1) = 0.58541020, IP(2,2) = 0.13819660;

IP(3,0) = 0.13819660, IP(3,1) = 0.13819660, IP(3,2) = 0.58541020;

The volume of the tetrahedron is found by numerically integrating the element interpolation functions over the tetrahedron.

The IsoparametricQuadraticTetrahedron is documented in more detail in the HTML Reference Guide.

### /source_code/generic_finite_volumes/ Library

BoundaryFiniteVolumeConnectivity

ElementFiniteVolumeTraits

ExplicitFiniteVolumeAlgorithm

ExplicitFiniteVolumeProcessor

ExplicitFiniteVolumeTransport

ExplicitNodeCenteredFiniteVolumeTransport

FV_Parameter

FiniteVolumeProcessor

FiniteVolumeAlgorithm

FiniteVolumeMonitor

FiniteVolumeConnectivity

FiniteVolumeTransport

FEM_CV_BijectiveMapping

finiteVolumeAuxiliaryFunctions

NodeCenteredFiniteVolumeMonitor

NodeCenteredFiniteVolumeInterface

NodeCenteredFiniteVolumeProcessor

NodeCenteredFiniteVolumeTransport

NodeCenteredFiniteVolumeAlgorithm

QuadrilateralSurfaceNormal

### /source_code/geometry_preprocessor/ Library

geometryPreprocessor

DXFObject

DXFPolyline

GH_String

Line

OpeningModeFracture

Polyline

SKM_FileHandler

Vertex

### /source_code/hydrothermal/ Library

AqueousDiffusivityVisitor

HeatTransferVelocityVisitor

H2OLiquidSteamPropertiesVisitor

H2OLiquidSteamTransportPropsVisitor

H2OLiquidSteamVelocityVisitor

LiquidSteamRelativeDensityVisitor

NaClH2OLiquidSteamPropertiesVisitor

NaClH2OMassFractionToFlowingMassVisitor

NaClH2OMassFractionVisitor

NaClTwoPhaseFVAdvection

PoreDiffusivityVisitor

RelativePressureToFluidPressure

TotalHeatCapacity

TotalHeatCapacityVisitor

TotalMassFractionToSalinity

TotalFluidVolumeSource

The library CSMP++3D-hydrothermal provides a collection of  Visitor and Interrelation classes for more complex simulations of heat and salt transport at single-phase and two-phase conditions such as in magmatic hydrothermal systems.

The H2OliquidSteamPropertiesVisitor object calculates nodal properties of pure H2O liquid and steam as a function of nodal pressure and temperature values. If the saturation curve of pure H2O is crossed at subcritical conditions, the energy balance between fluid and rock is calculated to compute the mass of liquid and steam present. This object can employ the PROST library or a H2OPropertiesLookUpTable object to calculate fluid properties. Calculations with PROST are computationally more costly than a direct retrieval from a H2OPropertiesLookUpTable object. On the other hand, a H2OpropertiesLookUpTable object requires and additional 105 MB memory. Sophisticated error checking is used in the H2OLiquidSteamPropertiesVisitor object, because PROST often fails at temperatures close to the critical point or pure H2O saturation curve. If a failure of PROST is encountered, the object attempts to recompute fluid properties at some small temperature increment away from the current temperature. Only if this method fails repeatedly, the simulation is terminated.

The NaClH2OliquidSteamPropertiesVisitor object calculates nodal brine, vapor, and halite properties for the NaCl-H2O system as a function of nodal pressure, temperature, and salinity values (Fig. 3). It uses a H2OPropertiesLookUpTable object to calculate the pure H2O properties and a Fluid object to obtain the NaCl-H2O properties. If the salinity is below 10-15 Wt. % NaCl, the object treats fluid as pure H2O. By setting the boolean in the NaClH2OLiquidSteamPropertiesVisitor constructor to true, thermal and chemical expansivities are retrieved from lookup tables as well. Each expansivity lookup table requires 65 MB of additional memory. Only one third of the computation time, however, is needed, because fluid properties do not have to be evaluated at temperature and salinity increments away from the values of interest. The NaClH2OliquidSteamPropertiesVisitor object uses sophisticated error checking as well, attempting to recompute fluid properties at some small temperature increment away from the temperature at which property calculations failed.

Both fluid property Visitor object terminate if non-physical, i.e., negative pressure, temperature, or salinity values are encountered. If the pressure exceeds the maximum valid pressure of 4000 bar, a warning is issued, because properties can only be computed up to 4000 bar.

The H2OLiquidSteamTransportPropsVisitor object calculates the element transport properties 'storativity', 'expansivity', 'mobility liquid', 'mobility vapor', 'total mobility', 'mass mobility liquid', 'mass mobility vapor', and 'total mass mobility' for two-phase fluids from nodal fluid and element rock properties. The mobility variables refer to the property kkri/i and the mass mobility variables to kkri/i. The expansivity includes thermal and chemical expansivity. If two-phase conditions are encountered, nodal fluid properties are averaged to the element considering only the nodes at which the phase is present.

The H2OLiquidSteamVelocityVisitor object solves Darcy's law for each element to compute fluid velocities for the liquid and vapor phase from nodal fluid pressures, element mobilities, and nodal relative fluid densities. The latter can be computed

for each phase using a LiquidSteamRelativeDensityVisitor object. Relative fluid pressures, which result if buoyancy terms are evaluated using relative fluid densities, can be converted into the actual fluid pressure using a RelativePressureToFluidPressure object. This object adds the relative pressure to a reference pressure, which must be computed once at the beginning of a simulation, to obtain the actual fluid pressure.

Fluid phase velocities may be transformed into a 'heat transfer velocity' using a

HeatTransverVelocityVisitor object. This object calculates the heat transfer velocity for two fluid phases on each element using nodal fluid and element rock properties. At two-phase conditions, fluid properties are averaged to the element only for the nodes where the phase is present.

The NaClH2OMassFractionVisitor object computes the mass of salt in the liquid and vapor phase such that the term div • (vi Xi) can be solved. The Interrelation object TotalMassFractionToSalinity converts mass fraction back to salinity considering that brine, vapor, and halite may be present.

Advection of salt in brine and vapor can be modeled using the NaClTwoPhaseFVAdvection object. This object employs a transport algorithm object and uses pre- and post processing steps to calculate fluid mass and salt transport in each phase. It automatically converts mass fractions back to salinity considering that brine, vapor, and halite may be present. If sub-stepping is used, i.e. if multiple advection steps are calculated during a single time-step, salt mass fractions are automatically updated using a NaClH2OMassFractionToFlowingMassVisitor object. Range checking at the end of each transport step returns error messages if salinity values exceed their prescribed ranges.

The Interrelation object TotalHeatCapacity and the Visitor object HeatCapactiyVisitor both calculate the term [(1-)r cpr +  (Sl l cpl + S v v cpv)] needed for the solution of the heat conduction equation. The Interrelation class TotalFluidVolumeSource can be used to add the source/sink term arising from the thermal and chemical expansivity to any additional fluid sources/sinks.

The aqueous diffusivity of NaCl in H2O can be computed as a function of temperature, pressure, and salinity using an AqueousDiffusivityVisitor object. This object evaluates the Stokes-Einstein relation (e.g., Oelkers, 1996) at each node. The diffusivity of the dissolved particle within the pore-space can be computed from a PoreDiffusivityVisitor object. This object calculates an element pore diffusivity from the nodal aqueous diffusivity and the element porosity (e.g., Oelkers, 1996) For large scale convection models, however, it is not necessary to evaluate the diffusivities, because their changes are negligibly small in comparison to the numerical diffusion, even when using a higher order transport scheme. For simulations in which only diffusion of heat and salt is considered, it is important though to accurately calculate the diffusivities.

For single-phase fluid property calculations of pure H2O or NaCl-H2O, the objects provided by the CSMP++3D-eos library always should be used, since they require memory allocation for only a single fluid phase.

### /source_code/interfaces/ Library

BinaryFileHandler

CSMP++_BinaryFileInterface

CSMP++_GoCadInterface

CSMP++_FRED_Interface

CSMP++_InputDataManager

CSMP++_JPEG_GroupInterface

CSMP++_JPEG_Interface

CSMP++_ICEM_Interface

CSMP++_Output

CSMP++_Tecplot_Interface

CSMP++_TextInterface

CSMP++_TRIANGLE_Interface

CSMP++_VTK_Interface

GocadHeader

GocadObject

GocadPropertyClassHeader

FRED_Fracture

ICEM_CSMP++_BoundaryConditions

ICEM_ElementSpecifications

SKM_RhinoSurfaceReader

skmReadWrite

StochasticPermeabilityFieldInterface

### /source_code/interrelations/ Library

Concatenate

Conductivity

ConstantFactor

ConductivityFromPerm

CozenyCarman

Dilatancy

ExtractTensorVariableComponent

ExtractVectorVariableComponent

ExtractVectorVariableLength

EnthalpySource MaximumDifference

FluidExpansionSource

HeatTransfer

HydraulicHead

ModifyPropertyWhere

MohrCoulombFailure

NodeToElementProperty

PermeabilityFromThickness

PropertyDifference

QuartzPrecipitation

QuartzSolubility

QuartzSolubilityNaCl

ScalarMassFlux

StreamingPotential

StokesEinsteinDiffusivity

ThermalDiffusivity

TransportStepSize

Local interdependencies among physical variables in a model are expressed by subclasses inherited from the base class Interrelation. Their declarations are included in the main program file in standard C/C++ manner

#include “MyInterrelation.h”

Interrelation subclasses are executed when passed to the Model by calling its Pass() method:

my_Model.Apply( inherited_interrelation );

Interrelation “Seed” library means that only a few interrelation subclasses are supplied with the CSMP++ distribution to illustrate what interrelations can do and how to write them. These interrelations are briefly described below in an alphabetical order.

All interrelations expect specific names and placements of physical variables (in italics) which they interrelate. These properties must be defined in the variable database input file. The provided interrelations either completely replace or add to or subtract from the value of the target variable. This important behavioral difference is indicated in brackets after the subclass name.

Concatenate. Combine variables with mathematical operators, see examples 13-14.

Conductivity. (replace) Calculates the hydraulic conductivity “conductivity” [m s-1] of a permeable medium from its “permeability” [m2] and the physical variable “fluid viscosity” [Pa s-1]. These variables are expected to be element variables. If the fluid viscosity is a node variable, the average value over the nodes of the element will be used in the calculation.

ConductivityFromPerm. (replace) Simplified version of previous interrelation. Calculates the hydraulic conductivity “conductivity” [m s-1] of a permeable medium from its “permeability” [m2] using a fixed value for the viscosity of the fluid [Pa s-1].

ConstantFactor.

CozenyCarman.

Dilatancy. (replace) This 2D interrelation calculates the physical element variable “dilatancy” [m2 m-2] not from the variable “strain”, but as the difference of the element “area” before and after displacements have been applied (“new area” [m2]). “Dilatancy” can be applied to any 2D element type to obtain an independent measure how well a strain calculation performed (as is discussed in the description of the finite element library, strains tend to be most accurate at the integration points inside of elements and often erratic if they are calculated directly at the nodes.

EnthalpySource.

ExtractTensorVariableComponent. (replace) Permits to extract specific components of tensor variables as identified by their index pair (i,j), and place them into a scalar variable of choice. Thus, diagonal and off-diagonal components can be extracted if needed.

ExtractVectorVariableLength. (replace) Computes the length of  vector variables assuming that the vectors originate in the coordinate origin. The computed length can be assigned to a scalar variable of choice.

ExtractVectorVariableComponent. (replace) As for tensor variables (see above), vector variable components identified by an index integer, can be mapped to user-specified scalar variables.

FluidExpansionSource. (add/subtract) Calculates a “fluid volume source” [m3 m-3] source or sink term which arises due to the expansion or contraction of the pore fluid in the rock from one to the next timestep. The fluid-volume change is corrected for the thermal expansion or contraction of the country rock. If this value is not set by the user, a value for concrete at 20oC is applied (12.5e-6 K-1). A volume change usually accompanies a change in fluid pressure or temperature and is monitored by taking the difference between the element variables “previous fluid density” and “fluid density” [kg m-3] multiplied by the “porosity” [X].  The temperature change is monitored by the variables “previous temperature” and “temperature” [°C] and  is used in conjunction with a user-specified thermal expansivity of the rock [m3 m-3 K-1], to calculate the thermal expansion of the pore space in the rock (“porosity”, [X]).  Since a source term in a transient calculation is always expressed as a rate [m3 m-3 s-1], the calculated “fluid volume source” is divided by the time increment over which the change occurred.

HeatTransfer. (replace) The variable “heat transfer velocity” [m s-1], vh, is calculated from the Darcy flow velocity “velocity”, v, [m s-1] the heat  capacities of the fluid and the country rock, hf, hc (“heat capacity fluid”, “heat capacity”) [J kg-1 K-1], and their densities, rf, and r (“fluid density” and “density”) [kg m-3], respectively, according to the formula:

vh = (v hf rf) / (hr r)

HydraulicHead. (replace) Calculates the nodal variable “hydraulic head” from the "fluid pressure" and the "elevation", which refers to the height above the lowest topographical point in a model.  The head, h, is calculated according to the formula

h = pf / (rho0 g) + elevation

g is the acceleration of gravity (9.80665 m N) and rho0 is the fluid reference density which is assumed to be 1000 kg m-3.

ModifyPropertyWhere. (replace) For a user-specified physical variable, this generic interrelation searches for specific scalar values and replaces them with a fixed scalar value.

NodeToElementProperty. (replace) Takes the average value of a user-specified nodal physical variable of all the nodes of an element. The result is returned into a user-specified element variable.

QuartzSolubility. (replace) Calculates the solubility of quartz in pure water “quartz solubility” [kg m-3] in an element from the nodal variables “temperature” [°C] and “nodal fluid density” [kg m-3]. The calculation is based on Craig Manning's fit of quartz-solubility data. The result is returned in grams per kg H2O.

QuartzSolubilityNaCl. (replace) As above, but calculation of aqueous solubility according to the empirical formula of Fournier & Potter, GCA 46, p. 1969-73, with a correction for the presence sodium chloride according to Fournier 1983, GCA 47, p. 579-586. The fluid salinity is specified by the nodal variable "mass fraction NaCl".

QuartzPrecipitation. (add/subtract) Integrates over a user-specified time-increment, how much quartz is precipitated or dissolved per unit volume of each finite element through which fluid flows at a Darcy velocity “velocity” [m s-1]. This is achieved by projecting the “quartz solubility gradient” vector onto the flow velocity vector. In this calculation, the physical variable “fluid density” [kg m-3] is used as element average density from which the time-integrated fluid mass flux [kg m-2 s-1] is derived. The result is added to or subtracted from the variable “quartz” [kg m-3].

ScalarMassFlux. (replace)  Calculates the length of the Darcy flow velocity vector [m s-1] and multiplies it by the element variable “fluid density” [kg m-3] in order to compute the physical variable “scalar mass flux” [kg m-2 s-1] in each finite element. If the flow model is three-dimensional, the scalar mass flux is calculated directly from the element variables “fluid pressure gradient” [Pa m-1],  “conductivity” [m s-1], and “fluid density” [kg m-3]. In this case, the variable fluid pressure gradient is computed conveniently, from the variable fluid pressure [Pa] via the Model method “CopyGradientOfProperty_A_To_B()”.

StokesEinsteinDiffusivity.

StreamingPotential.

ThermalDiffusivity.

TransportStepSize. (no variable is changed) Determines the maximum permitted advection-time-increment size for property advection on FiniteDifferenceGrid objects by the TransportVisitor or HeatTransportVisitor objects. This time increment is calculated on the basis of two vector variables that define nodal and elemental transport velocities, respectively, and a third variable which stores the radius of the inner circle of each triangular element. The stepsize is chosen such that the product of the velocity and the time increment is smaller than the diameter of the inner circle for all triangles. The physical variables are submitted to TransportStepSize as constructor arguments, and are named, in most cases “nodal velocity” [m s-1], “velocity” [m s-1], and “inner radius” [m]. When the transport-step-size interrelation is passed to the Model, it calculates the time-increment which can subsequently be retrieved using the method AdvectionTimeIncrement().

Many more  interrelations exist for a wide variety of purposes. Please contact the authors of this guide to find out more.

### /source_code/main_library/ Library

PDE_Integrator

BE_Time

BoundaryElementPair

BoundarySegment

CommandLineParser

ComputationalSettings

ConstraintPoint

ColorPalette

CSMP++_Box

CSMP++_FiniteElementManager

CSMP++_FEM_Data

CSMP++_Exception

CSMP++_FiniteVolumeStencilManager

CSMP++_high_level_utilities

CSMP++_Index

CSMP++_IndexMapper

CSMP++_MemoryManager

CSMP++_Mesher1D

CSMP++_MeshDiagnostics

CSMP++_MeshManager

CSMP++_ModelTopology

CSMP++_number_types

CSMP++_Operand

CSMP++_Standard_IO_Handler

CSMP++_StatisticalAnalyzer

CSMP++_String

CSMP++_TimeStepper

CSMP++_VData

CSMP++_VSet

CSMP++_VSetConverter

DataInterpreter

Element

ElementGrid

Face

FaceData

FEM_Visitor

FemToGridVisitor

FemFromGridVisitor

FiniteDifferenceGrid

FiniteElement

FiniteVolumeStencil

FV_IntegrationPointsAndWeights

GlobalTimeToInteger

GridPoint

Region

GroupMonitor

HashKey

IndexMapper

Interrelation

MathOperatorLHS

MathOperatorRHS

NodeMonitor

Node

Operand

Parameter

PostProcessor

PropertyConstraints

PropertyDatabase

ScalarVariable

SkmErrorHandler

Solver

Model

Model1

TensorVariable

TensorVariable1

TensorVariable2

VectorVariable

VectorVariable1

VectorVariable2

### /source_code/pde_operators/ Library

CSMP++_mechanics

Integral_NT_M_dV_Triangle

Integral_NT_N_dV

Integral_NT_lhsop_N_dV

Integral_NT_op_N_dV

Integral_NT_op_dNi_dV

Integral_NT_op_dS

Integral_NT_rhsop_N_dV

Integral_dNT_op_dN_NT_v_dN_dV

Integral_dNT_op_dN_dV

Integral_dNT_rhsop2_dN_dV

Integral_dNT_rhsop_dN2_dV

Integral_dNT_rhsop_dN_dV

Integral_op_NT_dN_orthogonal_dV

Integral_var_NT_lhsop_N_dV

Integral_var_NT_rhsop_N_dV

Jacobian_Integral_dNT_op_dN_dV

Jacobian_Upwind_Integral_dNT_op_dN_dV

NT_op

NumIntegral_BT_D_B_dV

NumIntegral_BT_D_op_dV

NumIntegral_BT_op_dV

NumIntegral_PT_op_dV

NumIntegral_dNT_mixed_op_dN_NT_v_dN_dV NumIntegral_dNT_mixed_op_dN_dV

NumIntegral_dNT_op_dN_NT_op_dop_dN_dV

NumIntegral_dNT_op_dN_dV

NumIntegral_dNT_op_dN_dV_NT_v_dN_dV.h

NumIntegral_DNT_rhsop_DN_dV

NumIntegral_op_NT_N_dV

NumIntegral_DNT_v_dV

NumIntegral_op_NT_dN_orthogonal_dV

NumIntegral_NT_lhsop_N_dV

NumIntegral_op_PT_P_dV

NumIntegral_NT_mixed_op_dNi_dV

NumIntegral_NT_op_N_dV

NumIntegral_NT_op_dS

NumIntegral_PT_lhsop_P_dV

NumIntegral_PT_op_P_dV

NumIntegral_PT_op_dS

NumIntegral_NT_op_dNi_dV

PointSource_rhsop

PT_op

StressesAndStrains

Upwind_Integral_dNT_op_dN_dV

Upwind_Integral_dNT_rhsop_dN_dV

Upwind_Integral_dNT_rhsop_g_dV

VelocityAndVolumeFlux

To solve PDEs with the finite-element method, they are converted into linear-algebraic matrix problems of the form

where, on the lefthand side, the square matrix A is called the global solution matrix, the vector x, is called the solution vector, and the vector b, is called the righthand vector. The size n of A is equivalent to the number of unknowns squared. For “scalar” problems where the dependent variable is a scalar, like “fluid pressure”, n is equivalent to the number of non-Dirichlet nodes. For problems where the dependent variable is a vector property, like displacement for instance, there are several degrees of freedom (DOF) per node, and the size of A is n DOF x n DOF. Since A holds the elements contributions to the global conductance or stiffness of the equation system it is sometimes also called the stiffness or conductance matrix. The volume or area integrals from the elements are accumulated into A one by one until one obtains a piecewise integration over the whole model. Integration constraints and constants are supplied via b. It will hold both, essential and initial conditions.

The PDE Operators in CSMP++ are grouped into those which compute (matrix) contributions to be assembled into the lefthand global solution matrix and are derived from the MathOperatorLHS base class, and into those which compute (vector) contributions to the righthand vector and are derived from the MathOperatorRHS base class. It was a design intent to completely decouple PDE operators from the type of finite elements used, and to allow for the integration of both element and node properties. Necessarily, there is a difference in the form of PDE operators which use analytical integration as is only possible for straight-sided elements, and PDE operators which employ numerical integration. This distinction is expressed in the PDE operator names:

Integral...	analytical integration (LinearTriangle or LinearTetrahedron)

NumIntegral...	numerical integration (all higher order elements)

Matrix contributions from the elements are called element “stencils”. These are again square matrices with a size equal to the number of nodes per element times the DOF per node. Further naming conventions used for the PDE operators are (with reference to the number of nodes per element).

| Notation | Meaning |
| --- | --- |
| N, NT | interpolation function vector and its transposed with n=nodes entries |
| N2 | duplicated interpolation function block matrix |
| dN, dNT | interpolation function derivatives matrix (dimensions x nodes) |
| P, PT | interpolation function matrix and its transposed for vector properties |
| B, BT | dN expanded for multiplication with 3 x 3 (2D) or 6 x 6 (3D) matrix |
| op | the material property(ies) which enter the MTRL matrix in integral |
| lhsop | lefthandside op if there arises an ambiguity in names when same operator is used in the left and the righthandside of the equation |
| dV | area- or volume integral |
| dS | surface integral (as for boundary stresses or Neumann fluxes |

Table 9.2: Naming conventions for the MathOperatorRHS and MathOperatorLHS subclasses.

When numerical integration is used or the properties are not constant on the element, interpolation functions and their derivatives must be computed at the integration points just like the material properties. This is how property variations are interpolated over the element and then integrated. In some cases the operator implementations are restricted such that only scalar properties or only element properties can be integrated by the specific operators. In these cases one will get a FATAL_ERROR message if one tries to use incompatible variables.

To reiterate, in detail, what is stated in the table, the matrix forms used futher below are listed:

Element interpolation functions:

N must be computed at each point of interest; n is the number of nodes per element. If vector properties are interpolated, each vector component must be interpolated separetely. This is expressed by writing P instead of N (this example is for the interpolation of a two-dimensional vector variable):

Element-interpolation function derivatives:

The expansion for multiplication with tensor variables (as is used in mechanics) is (for two-dimensional applications):

For three dimensional applications, an entry for a single node (where the last three rows refer to the off-diagonal tensor components) is:

In the following, analytically integrated operators are described first and numerically integrated ones second.

#### PDE Operators for Analytical Integration

The available operators for analytical integration are essentially all those which are also available (and addressed via enumeration strings) in conjunction with the CSMP++_PDE_Integrator:

| Operator | Integration of PDE Term | Other Name |
| --- | --- | --- |
| Integral_NT_lhsop_N_dV |  | capacitance |
| Integral_dNT_op_dN_dV | , | conductance |
| Integral_dNT_op_dN_NT_v_dNT_dV |  | diffusion-advection |
| Integral_NT_op_N_dV | N | capacitance |
| Integral_N_op_dN_dV | e.g. | buoyancy |

Table 9.3: PDE operators for elements using analytical integration.

The actual form of these operators and the contributions arising from them are for 2D triangular elements:

##### Integral_dNT_lhsop_dN_dV (conductance matrix)

Analytical integration of this integral for 2D linear triangles yields

and for 3D linear tetrahedral

where B for 3D linear tetrahedra is

Note that for linear finite elements, shape function derivatives are constants.

##### Integral_NT_op_N_dV (mass- or capacitance matrix)

consistent formulation

lumped formulation

##### Integral_dNT_op_dN_NT_v_dNT_dV (diffusion-advection matrix)

For 2D triangular elements, integration yields

Additional decay and sorption terms also are based on “mass” matrices and therefore have the same consistent and lumped forms:

#### PDE Operators for Numerical Integration

| Operator | Integration of PDE Term | Other Name |
| --- | --- | --- |
| lefthand |  |  |
| NumIntegral_NT_lhsop_N_dV |  | capacitance |
| NumIntegral_dNT_op_dN_dV | see other form above | conductance |
| NumIntegral_NT_op_dN_dV | e.g. | buoyancy |
| NumIntegral_BT_D_B_dV | lhs force balance | stiffness |
| righthand |  |  |
| NumIntegral_NT_op_dV |  | fluid body forces |
| NumIntegral_NT_op_N_dV | N | capacitance |
| NumIntegral_BT_op_dV |  | body forces, initial stresses |
| NumIntegral_BT_D_op_dV |  | volume strain |
| NumIntegral_PT_op_dS |  | surface stresses |
| PT_op |  | nodal forces |

Table 9.6: PDE operators for elements using numerical integration. The operators in red have not been tested rigorously yet.

In order to carry out the numerical integration interpolation function values and their derivatives must be computed at all integration points. Derivates need to be computed also in the assembly of mass matrices, since the Jacobian must be formed as its determinant post multiplies each integration point contribution.

Lumping of the mass matrix (if so desired for time-dependent calculations), is often implemented the crude way: The off-diagonal elements of the mass matrix are summed into its diagonal after forming the consistent form of the mass matrix:

This adds computational effort rather than reducing it by a simpler form. An alternative approach circumventing this problem is discussed in Lapidus & Pinder (1999, p. 328-329). They advocate Simpson’s rule-based integration to arrive naturally at a diagonal form of the mass matrix. For the isoparametric elements in CSMP++ lumping is implemented by multiplying the operand with the elements volume and dividing each of these nodal contributions by the number of nodes per element:

##### NumIntegral_dNT_op_dN_dV

This integral arises for elliptic partial differential equations, such a pressure diffusion or heat conduction. When the operand represents the hydraulic conductivity a conductance matrix results where K values are interpolated to the integration points using N. Numerical integration amounts to the following summation over the contributions from each integration point:

##### Integral_NT_op_N_dV

The capacitance or mass matrix computed by summing the contributions at the integration points has the form (3D):

The storativity is inserted as a scalar operand.

##### NumIntegral_dNT_D_dN_NT3_v_dN

NT3 is a matrix consisting whose 3 columns consist of interpolation function vectors. The diffusion-advection (dispersion) operator (not implemented yet) is based on the dispersivity tensor and the velocity vector. The accumulation of this term will lead to a non-symmetric solution matrix. If the matrix becomes strongly assymmetric the AMG solver can no longer be used and Gaussian Elimination may be the only method capable of inverting the solution matrix.

Radioactive decay can be incorporated into this element matrix by adding a NumIntegral_NT_op_N_dV term in which the scalar operand is composed from the decay constant, the porosity, fluid density and the KD value

and the (ad)sorption within the rock represented by the element is described by a similar NumIntegral_NT_op_N_dV term in which the operand contains the porosity, fluid density and the KD value

Volume integrals for mechanical calculations differ from the previous ones in that the dependent variable is no longer a scalar but a vector or tensor variable. These operators describe the elements stiffness, body forces, and volume strains. Surface integrals describe boundary tractions, stresses etc.

##### NumIntegral_BT_D_B_dV (stiffness)

##### NumIntegral_BT_op_dV (initial stresses)

##### NumIntegral_PT_op_dV (body forces)

##### NumIntegral_PT_op_dS (boundary tractions)

##### NumIntegral_BT_D_op_dV (volume strain)

##### PT_op (nodal forces)

### /source_code/solver/ Library

AMG_Solver

CG_Solver

Gauss_Solver

Meschach_Solver

SAMG_Configurator

SAMG_Exception

SAMG_Settings

SAMG_Solver

### /source_code/triangular_finite_volumes/ Library

AdvectionAlgorithm_Triangle2D

AdvectionAlgorithm_Triangle3D

CSMP++_FEM_FV_Transporter

FiniteVolume

FiniteVolumeAdvectionAlgorithms

FiniteVolumeAdvection

FiniteVolumeManager

FiniteVolumeMultiPhaseEnergyAndMassTransport

FiniteVolumeFluidPhasePropagation

FiniteVolumeSector_Triangle2D

FiniteVolumeSector_Triangle3D

FiniteVolumeToElementProperty

FiniteVolumeTransport_Triangle2D

FiniteVolumeTransport_Triangle3D

ExplicitFiniteVolumeAlgorithm_Triangle3D

ExplicitFiniteVolumeSector_Triangle3D

ExplicitFiniteVolumeTransport_Triangle3D

LinearTriangle3D_Advector

NodalFluidVolumeSource

NodePropertyGradient

NodePropertyGradientLimiter

ThetaLimitedAdvector

Triangle3DAdvectionAlgorithm

Triangle3DFiniteVolumeSector

A FiniteVolumeManager object generates, stores, and manages the finite volume grid. In this grid, finite volume cells are centered around the corner nodes of a triangular finite element mesh. Since fluid velocities are constant within each finite element, the flux n • v across the segment of such a node-centered finite volume is continuous. The finite elements can be of type LinearTriangle and IsoparametricQuadraticTriangle. Each finite volume cell is stored as a FiniteVolume object. This object contains the information about the size of the individual finite volume, its location, its neighbors, its parent elements, and its segments. A FiniteVolume objects can be accessed through the FiniteVolumeManager object.

Figure 1 shows the relationships, interfaces, and collaborations between the classes implemented in the CSMP++3D-triangular-finite-volumes library. The kernel of the explicit second-order accurate finite volume method is the FiniteVolumeAdvectionAlgorithms object. Its member functions allow to project fluid velocities onto the finite volume segments, compute the flux at each segment, and sum the segment fluxes to each individual FiniteVolume object. The calculation of the segment fluxes can be done with first or second order accuracy. If second order accuracy is desired, a

FiniteVolumeAdvectionAlgorithms object automatically obtains the nodal gradients of the transported variable from the NodePropertyGradient object. These are computed for each object. The corresponding MINMOD limiter values are calculated in the NodePropertyGradientLimiter object. As the finite volume method employs explicit time stepping, it is not necessary to construct a global solution matrix. Instead, the summation of the segment fluxes to the finite volumes is done sequentially by looping over all finite volume segments and adding or subtracting the flux at each segment to its two connected finite volumes.

Fig. 9.6. UML class diagram for parts of the CSMP++-triangular-finite-volumes library. Associations between classes are denoted by a bold line. The arrow notes the direction of flow of the associations. The diamond denotes a composition.

The FiniteVolumeAdvection and FiniteVolumeFluidPhasePropagation objects employ the FiniteVolumeAdvectionAlgorithms object. Each object carries out five identical steps to calculate advection (Fig. 2). First, the CFL criterion is determined. Second, fluid velocities are projected onto the finite volume segments. Buoyancy driven flow is automatically simulated if the pressure equation and fluid velocities include gravity terms. Third, if desired, the property gradients and limiter values are computed. Fourth, the segment fluxes are accumulated for each individual finite volume. Only those finite volumes are considered where the flag of the transported variable is not DIRICH. If its flag at the outflowing boundary is NEUMANN, it is assumed that the flux into the finite volume equals the flux leaving the finite volume. Fifth, the new values are mapped from the finite volume grid onto the finite element grid. During this step, each nodal value is compared to its prescribed variable range. An error is returned if this range is exceeded. If the global time-increment is larger than the CFL criterion, sub-stepping is employed until the global time-increment is met. In addition to these five steps, the respective objects use several pre- and post-processing operations to calculate a specific type of advection (e.g., transport in a compressible fluid).

The object FiniteVolumeAdvection solves the advection equation for transport of a scalar variable  i.e. div • (v ). For slightly compressible fluids, a first and second order accurate transport scheme is available. Transport in compressible fluids or a fluid with strong density variations can be calculated as well, i.e. div • (v ) can be solved for. In this case, the mass flux is computed at each finite volume segment. The latter methods are only available with second order accuracy.

Fig. 9.7. UML sequence diagram for a FiniteVolumeAdvection object. The individual steps necessary to calculate the higher order advection of  in a single-phase incompressible fluid using function CalculateSecondOderAdvection() are shown. All steps are repeatedly calculated as along as the CFL criterion is smaller than the external time-increment.

The object FiniteVolumeFluidPhasePropagation computes the immiscible displacement of two slightly compressible fluids (e.g., water and oil). Transport can be calculated with or without capillary effects for the wetting and non-wetting phase. The methods allow to chose from a variety of relative permeability models by setting the enum TWO_PHASE_METHOD in the constructor of a FiniteVolumeFluidPhasePropagation object to the desired value. All methods only work with second order accuracy to accurately resolve shock-fronts and rarefaction fans that occur during immiscible displacement of two fluids.

If the fluid is slightly compressible such that the divergence of the velocity field is non-zero, i.e. div • v ≠ 0, the object NodalFluidVolumeSource must be employed to calculate the divergence and correct the transport for it. Otherwise, under- and over-shoots of the transported variable are likely to occur.

### /source_code/unit_test/ Library

Algorithm_Test

CommandLineParser_Test

ConstantFactor_Test

Integral_var_NT_rhsop_N_dV_Test

Integral_NT_rhsop_N_dV_Test

Integral_var_NT_lhsop_N_dV_Test

Integral_dNT_rhsop2_dN_dV_Test

TotalDensity_Test

TotalMobilityInterpolation_Test

Upwind_Integral_dNT_op_dN_dV_Test

Upwind_Integral_dNT_rhsop_dN_dV_Test

Upwind_Integral_dNT_rhsop_g_dV_Test

### /source_code/visitors/ Library

AlterationVisitor & Reactant

BoundaryTransferVisitor

DispersivityVisitor

HydroFractureVisitor

InterfaceVisitor

NodePropertyToElementPropertyVisitor

TransportVisitor2D

VelocityAndVolumeFluxVisitor

Subclasses of the CSMP++_FEM_Visitor base class facilitate the solute advection on regular grids, the mapping of variables to- and from FiniteDifferenceGrid objects to a two-dimensional mesh and a way to simulate hydraulic fracture in models where overpressure is generated. There are also two visitor subclasses for the monitoring of fluxes across model- and sub-region boundaries. Example source code is provided to illustrate the application of visitor objects in general. This code defines the CSMP++_ID_Visitor and illustrates how to traverse the mesh and collect ID numbers from finite elements, constraint points or nodes.

The ID is only intended to illustrate how visitors work and not for any practical application. The visitor has a private integer vector into which it collects the ID numbers. These may either be Node, ConstraintPoint, Face or Element instances in the model. A key feature of the visitor pattern is an incomplete declaration in the base class CSMP++_FEM_Visitor, of the objects which shall be visited. This declaration allows to define access pointers to the visited objects as arguments of  the pure virtual method Visit(T* object). The interface of the objects pointed to by “T”, can later be accessed by the concrete Visit(T* object) methods in the derived visitor classes. These classes provide definitions of the visited objects in corresponding header files.

Remember, to use one of the visitor subclasses, you must first include its declaration into your program file:

#include “MyVisitor.h”

Now instantiate the visitor object and submit it to the Model or Region:

MyVisitor<CSMP++_float,2>  guest(...);

Model.Accept( guest );

Model.Region(“the garden”).Accept( guest );

// or alternatively

guest.RestrictApplicationTo( “the garden” );

Model.Accept( guest );

That’s it. This code example should persuade the user how easy it is to create and use original visitors. Feel free to contact CSMP++ authors for advice in writing new visitors to complement the library.

#### BoundaryTransferVisitor (2D linear triangles only)

The BoundaryTransferVisitor visits Groups determining fluxes across their boundaries. These fluxes are stored in STL maps separating the influx from the outflux and using the element ID numbers as keys. The maps are preserved after the visitation such that if the visitor is accepted a second time, only the flux values have to be changed. A specific call can be made to purge the maps before renewed application of the visitor. Boundary fluxes are always computed for the elements inside the target regions. The fluxes can also be integrated over time, if a time interval is supplied. A balance of the flux total (influx versus outflux) may also be obtained. Finally, the BoundaryTransferVisitor object can be used to assign NEUMANN (flux-type) boundary conditions.

#### InterfaceVisitor (2D linear triangles only)

The InterfaceVisitor visits the Model or Groups determining fluxes across model boundaries or interfaces between groups where groups terminate against eachother. The group boundary can be output from the visitor to a VTK text file for visual verification. An UpdateGeometry() function permits to track changing group interfaces. The InterfaceVisitor can also monitor fluxes across a Model boundary where it coincides with a group boundary. The model boundaries are identified by values of the enumeration SG_BOUNDARY.

Like for the BoundaryTransferVisitor, fluxes are stored in STL maps separating the influx from the outflux and using element ID numbers as keys. The maps are preserved after the visitation such that if the visitor is accepted a second time, only the flux values have to be changed. A specific call can be made to purge the maps before renewed application of the visitor. Fluxes are averaged between the elements inside and outside the interface. They can also be integrated over time, if a time interval is supplied. A balance of the flux total (influx versus outflux) may also be obtained.

#### AlterationVisitor

The AlterationVisitor computes the reactive exchange between the pore fluid and the rock in sequential node visitations. At the end of each visitation the new speciation is written back to the node property storage. The visitor can be used for reactive transport calculations where operator splitting is used to separate transport from reaction steps, see Steefel & MacQuarie (1996) for a discussion of this and other approaches. In a typical application advection/dispersion steps alternate with the invocation of the visitor, but the user must ascertain that no more than the entire pore-volue is exchanged before the next reaction step occurs. From this restriction, it is evident that countless reaction steps are required to change the mineralogy of a low-porosity rock appreciably. To improve its performance, the AlterationVisitor monitors reaction rates and concentrations at each timestep and carries out speciation calculations only when the pore-fluid is out of equilibrium or equilibrium was not attained in the previous visitation of a node. Nodes where the compulsory property ‘H2O_aq’ is flagged DIRICH are ignored. The visitor incorporates the ODE_StiffSolver object, the functionality of which is demonstrated in the distributed ‘example6.cpp’ program.

The AlterationVisitor reads the chemical reactions it is supposed to solve from a text input file. Another text file with the same name but with the extra extension “-prop.txt” provides the charges and atomic weights of the involved aqueous and gaseous species as well as the solids:

reactant properties for carbonate dissolution model

OH_1m		AQUEOUS     -1      17.0073

CO2_aq		AQUEOUS  	0	44.01

H_1p    	AQUEOUS  	1       1.0079

H2O_aq  	AQUEOUS  	0      18.02

HCO3_1m	AQUEOUS     -1     161.1043

Ca_2p		AQUEOUS	2   	40.078

HCl_aq  	AQUEOUS  	0      36.46

Cl_1m   	AQUEOUS     -1      35.4530

CC      	SOLID    2710.0    100.09

This is discussed again in the carbonate-dissolution example code (Chapter 7). The AlterationVisitor is documented in more detail in the CSMP++ Reference Guide.

Steefel, C. I. & MacQuarrie, K.T.B, 1996, Approaches to modeling reactive transport in porous media. (In) Reactive Transport in Porous Media, Rev. in Mineralogy 34, 83-125, American Mineralogical Society.

#### H2OPropertiesVisitor

Calculates the PVT properties of pure H2O at the Nodes of a mesh. This includes the physical variables "nodal fluid density" [kg m-3],  "nodal relative fluid density" [kg m-3] for a reference pressure, "nodal heat capacity liquid" [J kg K-1], "nodal enthalpy liquid" [J kg], "nodal fluid viscosity" [Pa s-1] and, optionally, also the same properties of the gas phase. These have the names "nodal heat capacity steam", "nodal enthalpy steam", as well as the "nodal saturation". The nodal variables are calculated from the “absolute fluid pressure” [Pa] and the “temperature” [°C].  The calculation is carried out using the IAPS equation of state for pure water which covers a temperature range from 0 to 1000oC and a pressure from atmospheric to 1 GPa.

The conditions for the reference density are set by a call to

void ReferenceDensity( double Tc, double Pa );

And the calculations of the properties of steam can be supressed by a call to

void LiquidPhaseProperties();

Since this eliminates an iteration loop, it greatly speeds up the visitations. However, it should be applied only after verifying that the PT conditions in the model are always outside of the critical region. With the

void Verbose();

option water properties can be output to screen directly after their computation. This enables a first visual examination if a new simulation is setup.

#### HydroFractureVisitor

The HydroFractureVisitor enhances the hydraulic “conductivity” [m s-1] within and beyond the boundary of regions in which the “absolute fluid pressure” [Pa] exceeds a fracture-threshold pressure given by the lithostatic load [Pa] augmented by the tensile strength [Pa] of the rock. Through this capability, the visitor permits just enough fluid to dissipate from the overpressured region to keep fluid pressure at the fracture threshold. This is achieved within the framework of an iteration loop. The visitation will terminate when the absolute fluid pressure in all the elements of the model is no higher than the confining pressure plus the tensile strength of the rock.

Conceptually, this treatment of hydraulic fracture is equivalent to creating an interconnected network of small fractures in the rock. This increases the effective hydraulic conductivity sufficiently, to bleed excess fluid out of the regions where fluid pressure would otherwise exceed the lithostatic pressure plus the tensile strength of  the rock. When the hydro-fracture visitor is accepted by the Model it will perform six different operations on each element it visits:

1. Test whether at least one element node is overpressured. Only where an overpressure exists, conductivity will be enhanced and diffused.

3. Using the hydraulic conductivity, K which shall be adjusted, the visitor calculates a “trial” fluid pressure gradient [Pa m-1] in overpressured elements.

The calculation employs the transport (Darcy) “velocity” [m s-1].

4. The hydraulic conductivity, K, of elements with at least one overpressured node is enhanced proportional to the quotient grad P over the “storativity”, βc, [m3 m-3 Pa-1] following the rationale that βc is the proportionality constant, which relates the change of pressure in an element to the amount of fluid leaving or entering it. K is adjusted to the value which will just valve off enough fluid to bring the pressure down to the confining pressure  plus the tensile strength of the rock.

5. Only in elements with an appreciable internal fluid-pressure gradient, an increased hydraulic conductivity fosters the dissipation of fluid. Therefore, if grad P is negligible the conductivity is not enhanced.

6. The new hydraulic conductivity is compared with the previous hydraulic conductivity. Only if the new conductivity is greater than the old one it is assigned to the element.

In the code segment where the HydroFractureVisitor gets applied, a decision must also be made whether regions that are no longer overpressured shall be kept at an enhanced conductivity. Pseudo code of such a hydrofracture visitor application loop might look like this:

- Calculate the hydrostatic pressure.

- Calculate the hydro-fracture threshold pressure as the “lithostatic pressure” plus the tensile strength of the rock, minus the hydrostatic pressure.

- Start the iteration procedure by computing the variable "effective pressure" (Pf -Phydrostatic) and modifying the hydraulic “conductivity” with the hydro-fracture visitor until the overpressure is dissipated.

- Recompute fluid properties and calculate fluid transport velocity.

- Compute new “absolute fluid pressure” = “effective pressure” + “hydrostatic pressure”.

The HydroFractureVisitor gets applied in the iteration loop (3) as follows:

while ( dP > 0.0 && step <= MAX_PF_ITERATIONS )

{

// 0. starting with fluid pressure before the erratic timestep

// -----------------------------------------------------------

sg.CopyReplace( "previous effective pressure", "effective pressure" );

// 1. diffusing new conductivity across border of hydrofracture

regions (letting fracs grow)

// -------------------------------

sg.Accept( hydrofrac_visitor );

// 2. re-compute fluid-pressure distribution (when hydrofractures

are propagated one must not consider convection)

// -------------------------------------------------------------

...

// 3. calculate new overpressures with an interration

// --------------------------------------------------

sg.CopyReplace(  "excess pressure", "previous excess pressure" );

sg.Pass( excess_pressure );

// 4. test for loop exit condition

// -------------------------------

sg.MinMaxOf( "excess pressure", dP_min, dP_max );

if ( dP_max <= 0.0 ) break;

step++;

} // END interation loop

Importantly, when a transient fluid pressure distribution is computed, a backup copy of the “effective pressure” from before the step must be kept and re-used as initial condition in each of the “tries” in the iteration loop.

#### TransportVisitor (2D only)

The TransportVisitor advects distributed variables on an array of regular grids using a two-dimensional velocity vector field defined by nodal and elemental velocities on the triangular element mesh. The velocity field must not contain discontinuities across element boundaries. These arise when the interstitial velocity is used and porosity changes occur from one element to the other. In this case, use a finite-volume approach to model transport (see code example5.cpp and following). To calculate incremental field displacements, the visitor uses a seven-point quadratic interpolation scheme (first order since no spatial derivatives of the advected property are considered). The grids are not managed by the visitor itself but have to be defined and initialized beforehand. This can be done by code similar to the following:

const CSMP++_float    RESOLUTION(0.1);

uint32             n_species = 5;

vector<CSMP++_float>  xy;

Model.Dimensions( xy );

frame_x = RESOLUTION * 10.0;

frame_y = RESOLUTION * 10.0;

vector<FiniteDifferenceGrid<CSMP++_float> >  aqueous_system(n_species);

for ( stl_index i=0; i<n_species; i++ )

aqueous_system[i].Initialize( -frame_x, xy(1)+frame_x,

-frame_y, xy(3)+frame_y,

RESOLUTION, RESOLUTION );

Once the grids are initialized, values must be assigned to them in one of three ways: (1) by setting selected rows, columns, or the entire grid to specific values (see interface of FiniteDifferenceGrid class); (2) by mapping properties from the finite-element mesh onto the grids (Model::WritePropertyToGrid()); (3) by assigning property distributions specified as regular-gridded data onto the grids (methods like In() of FiniteDifferenceGrid).

When the TransportVisitor is constructed one must specify the node and element variables which define  the velocity field:

TransportVisitor<CSMP++_float,2>

advector( Model_instance, "nodal velocity", "velocity",

aqueous_system, time_increment );

TransportStepSize<CSMP++_float,2>

step_size("nodal velocity", "velocity", ”inner radius”);

Model.Pass( step_size );

advector.AdjustTimeIncrement( step_size.AdvectionTimeIncrement() );

while ( time <= total_time )

{

...

advector.AdvectUntil( Model, time_interval );

...

}

In this code example, the advection time_increment is adjusted. This variable cannot assume an arbitrary value but geometrical arguments dictate that it must be less than the diameter of the inner circle of each triangular finite-element. The maximum increment which satisfies this conditions is calculated on an element-by-element basis from the velocity-vector field and the finite-element element size characteristics. This is done by the Interrelation TransportStepSize which uses the variables “inner radius”, “nodal velocity” and “velocity” to identify the maximum permissible advection time-increment for the current mesh and velocity distribution. The call to the method AdvectUntil() prompts the visitor to advect the property distributions on the grids in as many steps as it takes to span the time interval specified as method argument.

The TransportVisitor uses the velocity-vector field specified at the element centers as well as at the nodes to displace upstream solute concentrations on the transport grids. It also incorporates a scheme which interpolates concentrations advected into the model from outside the mesh. In order to achieve this, the TransportVisitor uses a five-point interpolation scheme which is fitted to solute concentrations along the flow path immediately adjacent to the grid boundary. This fit is used to extrapolate solute concentration outside of the grid. This scheme is capable of advecting linear gradients across boundaries. However, in some cases it fails to carry out such an extrapolation and the user receives warning messages. This may occur when (1) the transport grid resolution is very high, (2) the flow velocities are very high while the advection timespan is long, and (3) the concentration increases steeply behind an inflow boundary. Putting a frame around the transport grids with fixed concentrations or low permeabilities may help out of such a situation.

#### VelocityAndVolumeFluxVisitor

The VelocityAndVolumeFluxVisitor computes the flow velocities and the scalar volume flux from the fluid pressure distribution, the porosity, and the hydraulic conductivity of visited elements. The variable names "conductivity", "porosity", "fluid pressure", "velocity", "pore velocity", and  "volume flux" are hardwired into the VelocityAndVolumeFluxVisitor. The visitor is intended as a post-processor which is applied after a fluid pressure computation, but only if nodal velocities are desired for restricted model regions output to visualization tools. The application of this visitor restricted to surface representations of fractures or faults has the advantage that matrix element properties are not taken into account. The visitor will only use elements inside the target group, i.e. the surface, to compute nodal properties. In contrast, indiscriminate velocity post-processing of such models will lead to an averaging of volume (rock matrix) and surface (fracture plane) velocities.

# Appendix

### A: The Distribution

CSMP++ comes as a set of header and binary library files for the specific platforms and compilers with which the simulations shall be carried out. The files are stored in subdirectories with names corresponding to the libraries: main-library/, interrelations/, algorithms/, visitors/, finite-elements/, finite-volumes/, pde-operators/, and distributed-examples/. Unix-linux distributions are based on the GNU C++ compiler (version 2.95.2 or later) and have bin/ and lib/ directories in which executables, input, and variables files, and the binary libraries themselves are stored, respectively. Makefiles are provided for the example programs, and, on request, CodeWarrior Pro 6.0 (or later) project files for the Macintosh, PC, and the Linux PC, can be obtained.

The class objects which constitute the current distribution of CSMP++ are listed in alphabetical order in the tables below corresponding to the different CSMP++ libraries:

- main_library

- algorithms

- applied_math

- computational_geometry

- computer_science

- finite_elements

- generic_finite_volumes

- geometry_preprocessor

- hydrothermal

- interfaces

- interrelations

- pde_operators

- solver

- triangular_finite_volumes

- unit_test

- visitors

The Meschach AMG, SAMG, JPEG, and F2C libraries are documented elsewhere.

| CSMP++ Main Library |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Algorithm | solution of PDE’s using MathOps. | yes | html |
| BE_Time | measurement of runtime /processes | yes |  |
| BinaryFileHandler | template: IO of any class objects | yes |  |
| BoundaryElementPair | computes fluxes between elements |  |  |
| BoundarySegment | representation of line segments |  |  |
| ColorPalette | conversion of double to RGB or HSV rainbow or grayscale (modified from Per Kistler’s version) | yes |  |
| ConstraintPoint | point on finite-element edge | yes |  |
| CommandLineParser | as the name suggests | yes |  |
| CSMP++_Box | for boundary conditions | yes |  |
| CSMP++_DataContainer | storage of CSMP++_FEM_Data data sets | yes | html |
| CSMP++_Exception | information container for exceptions | yes |  |
| CSMP++_FiniteElementManager |  |  |  |
| CSMP++_FiniteVolumeStencil-Manager |  |  |  |
| CSMP++_FEM_Data | vector of scalar, vector, tensor variables | yes |  |
| CSMP++_high_level_utilities | text I/O for run documentation |  |  |
| CSMP++_Index | class equivalent of CSMP++_INDEX struct | yes | html UG |
| CSMP++_IndexMapper | re-indexing of ID-key maps |  |  |
| CSMP++_MemoryManager | property value storage | yes | UG |
| CSMP++_MeshManager | mesh connectivity | yes | UG |
| CSMP++_Mesher1D |  |  |  |
| CSMP++_ModelTopology |  |  |  |
| CSMP++_Operand | model-wide calculations | yes | html |
| CSMP++_Standard_IO_Handler | input queries etc. | yes |  |
| CSMP++_StatisticalAnalyzer | property value histograms | yes |  |
| CSMP++_String | operations with text strings | yes |  |
| CSMP++_STL_utilities | ways to print vectors etc. |  |  |
| CSMP++_VData | mesh-connectivity-data container | yes |  |
| CSMP++_VSet | Model model container, persistence | yes | html DG |
| CSMP++_VSetConverter | linear to quadratic mesh conversions | yes |  |
| DataInterpreter | 1...256 integer to permeability converter | yes | UG |
| Element | connectivity and coordinates of finite element | yes | DG |
| ElementGrid | vector of grid cells overlapped by element | yes |  |
| FEM_Visitor | base class for all visitors | yes | UG |
| FemToGridVisitor | unstructured to structured mapping | yes |  |
| FemFromGridVisitor | structured to unstructured mapping | yes |  |
| FiniteDifferenceGrid | interpolation, advection, JPEG | yes | UG |
| FiniteElement | base class of specific finite-element type | yes | DG |
| FV_IntegrationPointsAnd-Weights |  |  |  |
| GlobalTimeToInteger | CSMP++_float to integer conversion |  |  |
| GridPoint | storage array for xy-point data on several but corresponding grids | yes |  |
| Region | sub-region of model | yes | DG |
| GroupMonitor | value ranges and integrals for file output / runtime monitoring | yes | UG |
| HashKey | element neighbor identification | yes |  |
| IndexedMapper | renumber dis-contiguous lists |  |  |
| Interrelation | base class for interrelation objects | yes | html |
| MathOperatorLHS | base class for PDE operators for left-hand side of matrix equation | yes | DG |
| MathOperatorRHS | base class for PDE operators for right-hand side of matrix equation | yes | DG |
| Node | nodal point in finite element | yes | DG |
| NodeMonitor | for node properties | yes | UG |
| ODE_StiffSolver | stiff ordinary differential equation solver | yes | html |
| Operand | variable handle in computations | yes | html |
| Parameter | property specifications record used by PropertyDatabase | yes |  |
| PropertyDatabase | variable management in a model | yes | html |
| ScalarVariable | double plus VARIABLE_FLAG | yes | UG |
| SkmErrorHandler | global platform-independent error-handler | yes | html |
| skmReadWrite | binary File I/O functions | yes |  |
| Solver | sparse-matrix solver, AMG, conjugate gradient, Gaussian elimination etc. | yes | UG, DG html |
| Model | model representation of real-world system | yes | html |
| TensorVariable | space-dimensions squared double matrix with flags | yes | UG |
| VectorVariable | space-dimension-sized double vector with flags | yes | UG |

Table A.1: Class objects contained in the CSMP++ main_library. Doc. = documentation in html (HTML) format, in the User Guide (UG, this document), or the Developer’s Guide (DG).

| CSMP++ Algorithms |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Algorithm | buoyancy-driven fluid flow | yes | UG |
| IsoparametricAdvection-DispersionAlgorithm | advection and diffusion | yes | UG |
| IsoparametricDiffusion-Algorithm | simplified Algorithm for diffusion problems but for numerical integration | yes | UG |
| IterativeAlgorithm | general for all kind of iterative schemes | yes |  |
| LinearDiffusionAlgorithm | simplified Algorithm for diffusion problems | yes |  |
| NewtonAlgorithm | general Newton-Raphson, derived from IterativeAlgorithm | yes |  |
| PicardAlgorithm | general Picard iteration, derived from IterativeAlgorithm | yes |  |
| UpwindAlgorithm | 2-phase flow, Newton or Picard iteration | yes |  |

Table A.2: Class objects contained in the CSMP++ algorithms library.

| CSMP++ Applied Math |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Array | basic array class with operators |  |  |
| CSMP++_ANSI_ISO_patches |  | yes | UG |
| CSMP++_DenseMatrix |  | yes | UG |
| CSMP++_ErrorFunction |  | yes | UG |
| CSMP++_LoopUnroller |  |  | UG |
| CSMP++_Matrix |  |  |  |
| CSMP++_SparseMatrix |  |  |  |
| CSMP++_matrixMethods |  |  |  |
| EigenFinder |  |  |  |
| Log10Property |  |  |  |
| MaximumResidual |  |  |  |
| ODE_StiffSolver |  |  |  |
| RandomFloat2 |  |  |  |
| SparseMatrixCondensedRowStorage |  |  |  |
| SumOfProducts |  |  |  |
| SumOfProductsWithExponents |  |  |  |
| Tolerance |  |  |  |

Table A.3: Class objects contained in the CSMP++3D applied_math library.

| CSMP++ Computational Geometry |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |

| MJL_Point | topology: 2D point object |  | Lazl. |
| --- | --- | --- | --- |
| MJL_Edge | topology: 2D straight edge |  | Lazl. |
| MJL_Polygon | topology: 2D straight-edge polygon |  | Lazl. |
| MJL_Point3D | topology: 3D point object |  | Lazl. |
| MJL_Edge3D | topology: 3D straight edge |  | Lazl. |
| MJL_Triangle3D | topology: 3D triangle |  | Lazl. |
| Triangle | linear triangle element |  |  |
| Triangulator | 2D triangle mesher for bitmaps |  |  |

Table A.4: Class objects contained in the CSMP++ computational_geometry library.

| CSMP++ Computer Science |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| KD_BinaryTree | search tree for multivariate-keys | yes | UG |
| MBST_Node | tree node | yes | UG |
| SK_DataObject | tree data container | yes | UG |

Table A.5: Class objects contained in the CSMP++ computer_science library.

| CSMP++ Finite Elements |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| IsoparametricLinearHexahedron |  | yes | UG |
| IsoparametricLinearLineElement |  |  |  |
| IsoparametricLinearPrism |  |  |  |
| IsoparametricLinearPyramid |  |  |  |
| IsoparametricLinearQuadrilateral |  |  |  |
| IsoparametricLinearTetrahedron |  |  |  |
| IsoparametricLinearTriangle |  |  |  |
| IsoparametricQuadraticHexahedron |  |  |  |
| IsoparametricQuadraticLineElement |  |  |  |
| IsoparametricQuadraticPrism |  |  |  |
| IsoparametricQuadraticPyramid |  |  |  |
| IsoparametricQuadraticQuadrilateral |  |  |  |
| IsoparametricQuadraticTetrahedron |  |  |  |
| IsoparametricQuadraticTriangle |  |  |  |
| LinearLineElement |  |  |  |
| LinearTetrahedron |  |  |  |
| LinearTriangle |  |  |  |
| LinearTriangle3D |  |  |  |

Table A.6: Class objects contained in the CSMP++3D finite_elements library.

| CSMP++ Generic Finite Volumes |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| BoundaryFiniteVolumeConnectivity | base class FV mesh visitors |  |  |
| ElementFiniteVolumeTraits |  |  |  |
| ExplicitFiniteVolumeAlgorithm |  |  |  |
| ExplicitFiniteVolumeProcessor |  |  |  |
| ExplicitFiniteVolumeTransport |  |  |  |
| ExplicitNodeCenteredFiniteVolumeTransport |  |  |  |
| FEM_CV_BijectiveMapping |  |  |  |
| FV_Parameter |  |  |  |
| FiniteVolumeAlgorithm |  |  |  |
| FiniteVolumeConnectivity |  |  |  |
| FiniteVolumeMonitor |  |  |  |
| FiniteVolumeProcessor |  |  |  |
| FiniteVolumeTransport |  |  |  |
| NodeCenteredFiniteVolumeAlgorithm |  |  |  |
| NodeCenteredFiniteVolumeInterface |  |  |  |
| NodeCenteredFiniteVolumeMonitor |  |  |  |
| NodeCenteredFiniteVolumeProcessor |  |  |  |
| NodeCenteredFiniteVolumeTransport |  |  |  |
| QuadrilateralSurfaceNormal |  |  |  |
| finiteVolumeAuxiliaryFunctions |  |  |  |

Table A.7: Class objects contained in the CSMP++ generic_finite_volumes library.

| CSMP++ Geometry Preprocessor |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| DXFObject |  |  |  |
| DXFPolyline |  |  |  |
| geometryPreprocessor |  |  |  |
| GH_String |  |  |  |
| Line |  |  |  |
| OpeningModeFracture |  |  |  |
| Polyline |  |  |  |
| SKM_FileHandler |  |  |  |
| Vertex |  |  |  |

Table A.8: Class objects contained in the CSMP++ geometry_preprocessor library.

| CSMP++ Hydrothermal |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| AqueousDiffusivityVisitor | Stokes-Einstein but taking into account concentration |  |  |
| H2OLiquidSteamPropertiesVisitor | pure H2O, 2-phase props. |  |  |
| H2OLiquidSteamTransportPropsVisitor | phase mobilities, compressibililities, expansivities |  |  |
| H2OLiquidSteamVelocityVisitor | phase velocities, 2 phase |  |  |
| HeatTransferVelocityVisitor | heat transfer velocity, 2 phase |  |  |
| LiquidSteamRelativeDensityVisitor | reduced densities, 2 phase |  |  |
| NaClH2OLiquidSteamPropertiesVisitor | NaCl-H2O, 3-phase |  |  |
| NaClH2OMassFractionVisitor | mass fractions of different phases |  |  |
| NaClTwoPhaseFVAdvection | salt transport in L & V phases |  |  |
| PoreDiffusivityVisitor | B, phi-dependent diffusivity |  |  |
| RelativePressureToFluidPressure | absolute fluid pressure |  |  |
| TotalFluidVolumeSource | contribs due to expansivities |  |  |
| TotalHeatCapacity | sum of all phases |  |  |
| TotalHeatCapacityVisitor | sum of all phases |  |  |
| TotalMassFractionToSalinity | converts component mass fractions to total salinity |  |  |

Table A.9: Class objects contained in the CSMP++ hydrothermal library.

| CSMP++ Interfaces |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| BinaryFileHandler | save objects to binary files |  |  |
| CSMP++_BinaryFileInterface | fast and space efficient output of results |  |  |
| CSMP++_FRED_Interface | Fracman / FRED from Golder Associates |  |  |
| CSMP++_GoCadInterface | input and output to GoCAD / TSurf |  |  |
| CSMP++_ICEM_Interface | Tetra mesher, ICEM Technologies Inc., (ANSYS) |  |  |
| CSMP++_InputDataManager | model configuration from textfile |  |  |
| CSMP++_JPEG_GroupInterface | 2D Groups to JPEG images |  |  |
| CSMP++_JPEG_Interface | 2D models to JPEG images |  |  |
| CSMP++_Output | scheduling output of multiple properties /variables |  |  |
| CSMP++_TRIANGLE_Interface | reads input from Shewchuk’s TRIANGLE mesher |  |  |
| CSMP++_Tecplot_Interface | linear elements to Tecplot (tecplot.com) |  |  |
| CSMP++_TextInterface | outputs to text column files |  |  |
| CSMP++_VTK_Interface | outputs to Visualisation Toolkit in text format |  |  |
| FRED_Fracture | FRED (Golder Assoc.) fracture attributes |  |  |
| GocadHeader | data header in GoCAD |  |  |
| GocadObject | GoCAD geometric object |  |  |
| GocadPropertyClassHeader | GoCAD header block |  |  |
| ICEM_CSMP++_BoundaryConditions | Extra container initially created for 2.5D models |  |  |
| ICEM_ElementSpecifications | maps ICEM to CSMP++ types |  |  |
| SKM_RhinoSurfaceReader | use render meshes from Rhinoceros in computations |  |  |
| StochasticPermeabilityField-Interface | reads HYDROGEN –generated permeability fields |  |  |
| skmReadWrite | binary I/O for STL types |  |  |

Table A.10: Class objects contained in the CSMP++ interfaces library.

| CSMP++ Interrelations |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Concatenate | combine variables in formulae |  |  |
| ConductivityFromPerm | hydraulic conductivity from permeability and fixed fluid viscosity | yes | UG |
| Conductivity | hydraulic conductivity from permeability and fluid viscosity | yes | UG |
| ConstantFactor | adds fixed quantity to variable | yes |  |
| CozenyCarman | permeability from grainsize, porosity and internal surface area | yes |  |
| Dilatancy | element volume change | yes | UG |
| EnthalpySource | heat source /sink due to H change |  |  |
| ExtractTensorVariable-Component | Assign value (i,j) of tensor to scalar | yes |  |
| ExtractVectorVariableLength | compute length & assign to scalar | yes |  |
| ExtractVectorVariable-Component | Assign value (i) of vector to scalar | yes |  |
| FluidExpansionSource | source term due to PT change |  |  |
| HeatTransfer | compute heat advection velocity from Darcy velocity and thermal properties of fluid and rock | yes | UG |
| MaximumDifference | maximum range between values anywhere in model or group | yes |  |
| HydraulicHead | from piezometric + elevation head | yes |  |
| ModifyPropertyWhere | selectively change property values | yes |  |
| NodeToElementProperty | element average of node property | yes | UG |
| NodalScalarVolumeFlux | nodal velocity -> scalar flux | yes |  |
| PropertyDifference | to compare two variables | yes |  |
| QuartzPrecipitation | dissolution or precipitation due to gradient reaction | yes | UG |
| QuartzSolubility | Craig Manning’s formula | yes | UG |
| QuartzSolubilityNaCl | Fournier & Potter, Crerar | yes |  |
| ScalarMassFlux | v∙ρ | yes | UG |
| StokesEinsteinDiffusivity | temperature & pressure dependent diffusivity | yes |  |
| StreamingPotential | scalar variable propertial to pressure gradient | yes |  |
| ThermalDiffusivity | conductivity / (density x heat capacity) | yes |  |
| TransportStepSize | advection step-size determination | yes | UG |

Table A.11: Class objects contained in the CSMP++ interrelations library.

| CSMP++ PDE Operators |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Integral_NT_op_N_dV | e.g., capacitance right vector | yes | UG |
| Integral_NT_lhsop_N_dV | e.g., capacitance left matrix | yes | UG |
| Integral_dNT_op_dN_dV | e.g., conductance left matrix | yes | UG |
| Integral_dNT_op_dN_v_dNT | diffusion-advection left matrix | yes | UG |
| Integral_N_op_dNi_dV | e.g. buoyancy forces |  |  |
| Integral_dNT_rhsop2_dN_dV | operand multiplied with variable | yes |  |
| Integral_op_NT_dN_-orthogonal_dV | streamfunction operand |  |  |
| Integral_NT_M_dV_Triangle | FV – FEM bijective mapping |  |  |
| Jacobian_Integral_dNT_op_- dN_dV | numerical Jacobian for standard Galerkin stiffness matrix |  |  |
| Jacobian_Upwind_Integral_-dNT_op_dN_dV | numerical Jacobian for upwind Galerkin stiffness contribution |  |  |
| NumIntegral_BT_D_B_dV | e.g., stiffness left matrix |  |  |
| NumIntegral_BT_D_op_dV | e.g., volume strain right vector |  |  |
| NumIntegral_dNT_op_dN_dV | e.g., conductance left matrix |  |  |
| NumIntegral_NT_lhsop_N_dV | e.g., capacitance left matrix |  |  |
| NumIntegral_PT_op_dS | e.g., surface stresses (tractions) |  |  |
| NumIntegral_PT_op_dV | e.g., initial stresses |  |  |
| NumIntegral_op_PT_P_dV | mass, right vec. |  |  |
| NumIntegral_PT_op_dV | body forces, right vector |  |  |
| NumIntegral_NT_op_N_dV | capacitance right vector |  |  |
| PT_op | nodal point forces, right vector |  |  |
| StressesAndStrain | postprocessing, nodal extrap. | yes | UG |
| VelocityAndVolumeFlux | postprocessing, nodal extrap. | yes | UG |
| Upwind_Integral_dNT_op_-dN_dV | stiffness matrix for upstream weighting (lhs) |  |  |
| Upwind_Integral_dNT_rhsop_-dN_dV | stiffness matrix for upstream weighting (multiplied with rhs) |  |  |
| Upwind_Integral_dNT_rhsop_-g_dV | stiffness matrix for upstream weighting, gravity-contribution (multiplied with rhs) |  |  |

Table A.12: Class objects contained in the CSMP++ pde_operators library.

| CSMP++ Solver |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| AMG_Solver. | alg. multigrid, mod. netlib version | yes | UG |
| CG_Solver | conjugate gradient method | yes | UG |
| Gauss_Solver | Gaussian elimination | yes | UG |
| Meschach_Solver | abstract base for CG, GE solvers | yes | UG |
| SAMG_Configurator | specific for SAMG solver | yes |  |
| SAMG_Exception | SAMG (Fortran) error messages passed to CSMP++ | yes |  |
| SAMG_Settings | solver parameter-combinations with names | yes |  |
| SAMG_Solver | commercial algebraic multigrid | SCAI |  |

Table A.13: Class objects contained in the CSMP++ solver library.

| CSMP++ Triangular Finite Volumes |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| AdvectionAlgorithm_Triangle2D |  |  |  |
| AdvectionAlgorithm_Triangle3D |  |  |  |
| CSMP++_FEM_FV_Transporter |  |  |  |
| ExplicitFiniteVolumeAlgorithm_-Triangle3D |  |  |  |
| ExplicitFiniteVolumeSector_-Triangle3D |  |  |  |
| ExplicitFiniteVolumeTransport_-Triangle3D |  |  |  |
| FiniteVolume | FV properties |  |  |
| FiniteVolumeAdvection | single-phase, slightly compr. |  |  |
| FiniteVolumeAdvectionAlgorithms | explicit sequential solution |  |  |
| FiniteVolumeFluidPhasePropagation | 2-phase, adv. of target phase |  |  |
| FiniteVolumeManager | FV mesh |  |  |
| FiniteVolumeSector_Triangle2D |  |  |  |
| FiniteVolumeSector_Triangle3D |  |  |  |
| FiniteVolumeToElementProperty |  |  |  |
| FiniteVolumeTransport_Triangle2D |  |  |  |
| FiniteVolumeTransport_Triangle3D |  |  |  |
| LinearTriangle3D_Advector |  |  |  |
| NodalFluidVolumeSource | divergence of velocity field correction |  |  |
| NodePropertyGradient | least-square grad. estimate at node |  |  |
| NodePropertyGradientLimiter | MINMOD limiter |  |  |
| ThetaLimitedAdvector |  |  |  |
| Triangle3DAdvectionAlgorithm |  |  |  |
| Triangle3DFiniteVolumeSector |  |  |  |

Table A.14: Class objects contained in the CSMP++ triangular_finite_volumes library.

| CSMP++ Unit Tests |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| Algorithm_Test |  |  | UG |
| CommandLineParser_Test |  |  |  |
| ConstantFactor_Test |  |  |  |
| Integral_NT_rhsop_N_dV_Test |  |  |  |
| Integral_dNT_rhsop2_dN_dV_Test |  |  |  |
| Integral_var_NT_lhsop_N_dV_Test |  |  |  |
| Integral_var_NT_rhsop_N_dV_Test |  |  |  |
| Suite | collection of tests |  |  |
| Test | base class for specific class test drivers |  |  |
| TotalDensity_Test |  |  |  |
| TotalMobilityInterpolation_Test |  |  |  |
| Upwind_Integral_dNT_op_dN_dV_Test |  |  |  |
| Upwind_Integral_dNT_rhsop_dN_dV_Test |  |  |  |
| Upwind_Integral_dNT_rhsop_g_dV_Test |  |  |  |

Table A.15: Class objects contained in the CSMP++ unit_tests library.

| CSMP++ Visitors |  |  |  |
| --- | --- | --- | --- |
| Class | Description / Role | Support | Doc. |
| AlterationVisitor | element-by-element carbonate dissolution / precipitation reaction prototype |  | UG |
| Reactant | used by AlterationV. |  |  |
| BoundaryTransferVisitor | fluxes across model and group boundaries | yes | UG |
| CSMP++_ID_Visitor | example, collects ID numbers into IVec | yes | UG |
| DispersivityVisitor |  |  | SG |
| HydroFractureVisitor | overpressure dissipation via conductivity enhancement | yes | UG |
| InterfaceVisitor | fluxes between adjacent groups | yes | UG |
| NodePropertyToElementPropertyVisitor |  |  |  |
| PropertyAtPointVisitor |  |  | AM |
| TransportVisitor2D | property advection on several grids | yes | UG |
| VelocityAndVolumeFlux-Visitor | group-restricted post-processing and nodal extrapolation | yes | UG |

Table A.14: Class objects contained in the CSMP++ visitors library.

| CSMP++ Distributed Examples |  |  |  |
| --- | --- | --- | --- |
| Function | Description / Role | Support | Doc. |
| examples_main.cpp | main() program with menu to choose examples from | yes |  |
| example1.cpp | Triangulator-based meshing, 2D pressure-driven flow, model to binary file and back, CSMP++_Operands | yes | UG |
| example2.cpp | 2D solute transport in a constant velocity field | yes | UG |
| example3.cpp | 2D fluid pressure-diffusion during well testing | yes | UG |
| example4.cpp | linear-elastic displacements in an inhomogeneous solid due to boundary displacements | yes | UG |
| example5.cpp | 3D pressure-driven fluid flow using an Algorithm, output of Groups | yes | UG |
| example6.cpp | integration of ordinary differential equations describing chemical reactions | yes | UG |
| example7.cpp | topography-driven fluid flow | yes | UG |
| example8.cpp | transient thermal convection | yes | UG |
| example12.cpp | 3D elastic deformation / stress fields computed with quadratic elements |  |  |
| example11.cpp | 2D reactive transport: rate-controlled carbonate dissolution or precipitation with K feedback | yes | UG |
| example12.cpp | 2D transport with the finite-volume method | yes | UG |
| example13.cpp | fluid-pressure equilibration using Algorithm and PDE operators | yes | UG |
| example14.cpp | 3D hybride-element volume & surface flow model | yes | UG |
| example15.cpp | 3D CVFE advection |  |  |
| example16.cpp | topography-driven flow |  |  |
| example17.cpp | 1D H2O EOS profile |  |  |
| example18.cpp | calcite dissolution |  |  |
| example19.cpp | GoCAD input |  |  |
| example20.cpp | Rhinoceros surfaces model |  |  |

Table A.15: Functions contained in the CSMP++ distributed_examples library.

### B: Installation

The CSMP++ libraries interface with third-party libraries f2c (g2c), JPEG, Meschach, and SAMG. The complete distribution contains the additional components:

| Library Name | ftp site |
| --- | --- |
| f2C | http://www.netlib.no/netlib/f2c/ |
| Meschach | http://www.netlib.no/netlib/c/meschach/index.html |
| AMG (original) | http://www.netlib.no/netlib/AMG |
| SAMG | commercially marketed by SCAI, Fraunhofer Ges., DE |
| JPEG |  |
| VTK | http://kitware.com |
| Blitz++ | http://oonumerics.com/blitz |

In order to adapt to the particularities of the various platform conditional compilation is used. The installer may have to uncomment a #define statement in the header file “CSMP++_definitions.h” to unveil the name of the operating system before compiling CSMP++-library based code on the platform of choice.

IMPORTANT: Before the compilation of the CSMP++ libraries on the target platform can start, the program CSMP++_number_types_main.cpp must be compiled and executed. It has no make dependencies but establishes the size and availability of C++ inbuilt number types. Once executed it writes the file “CSMP++_number_types.h” which is included into “CSMP++_definitions.h” automatically. This ascertains that results obtained on different platforms and compilers have the same numerical precision.

The user may also need to define which version of the STL library shall be used. There are different implementations which add multithread safety and specific exception handling capabilities to Stepanov’s original STL resource. Different versions can be invoked on different platforms by un-/commenting #define statements in the “CSMP++_definitions.h” header file.

#### CSMP++ Projects in CodeWarrior

CodeWarrior is a powerful C++ code development environment with IDE, debugger, class browser etc. It exists on the Mac, the PC, and on Linux operating systems. SKM  recommends it for the development of CSMP++ applications because the Windows version 8.0 and greater is fully compliant with the C++ ANSI/ISO standard and actually compiles all of the compiler tests presented in Todd Verhuizen’s Blitz++ library.

To use the CSMP++ library in CodeWarrior C++ Project in (description with exact references to CW Pro 8.0 under Windows 2000), a series of configuration steps must be performed. In the following description of these steps a right arrow () indicates menu and submenu choices, for instance:

File  Save  TIFF

means go to the “File” menu, choose the submenu “Save” , and select the option which will save the data as a tagged image file.

1. Start Code Warrior and choose: 	File  New... CW Project

A box will appear prompting the user to select the kind of supplementary files he or she wishes to include into a new project. Click into the list of options for the current operating system, e.g.:

Win32 C++ Stationary

Define a project name and enter the directory in which the new project shall be created. After clicking O.K. choose from the pop-up box

Win32 Console App -> C++ Console App (on PPC, or NT)

as project subcategory. Make sure that the project will be created in an intended subdirectory.

A new project window will now be created, containing the three folders: Sources, MSL ANSI Libraries, and operating-system dependent Win32 SDK libraries. Before adding source code and libraries to the new project, the project  must be configured:

2. Choose: 	Edit   C++ App Console Settings...

and a window with a topic panel will appear. Several of the listed panels need to be opened to set options:

Target  Target Settings:   Enter the intended name of your project configuration.

Target  Access Paths: Set paths to the subdirectories in which the header files of the desired objects live in. The typical set of paths for a CSMP++ project depends on the directory structure of the machine used and the operating system.

Add paths by using the Add... button with the “User Paths” option selected. Do no alter the “System Paths”. IMPORTANT is that the last folder in each path must contain the header files which declare the desired classes. It does not suffice to specify the next higher folder. To find the location of a specific file use the operating system’s “Find” tools with its file name as input.

Target   “...” Target:  Enter the name for the executable program which shall be produced in the “File Name” box.

Language Settings   C/C++ Language: Check the following boxes in addition to those which are already selected:

- Activate C++ Compiler

- ANSI Strict

- ISO C++ Template Parser

- Smart inlining

The “Code Generation   “...” Processor”  menu allows the user to optimize the code for the processor which is being used. Different optimizations may be chosen. The “Code Generation   Global Optimizations” panel offers additional options. The main choices are executable size versus execution speed and a series of optimization levels. Beware of levels beyond 2, if the code still has to be debugged.  If a high level of optimization is chosen variables may be used for a number of purposes and their values will seem to change erratically. Also Code Warrior may crash trying to find the best options.

Linker   “...” Linker:  Check the “Generate SYM File” option to be able to view source code in the debugger window and to set breakpoints in it.

Save the configuration and close the “C++ App Console Settings” panel. Make sure that the Static Single Threaded (Debug) compilation setting is chosen. Also, note that all libraries must be compiled with exactly the same compilation flags. Otherwise errors will appear which are very hard to diagnose.

3. Now, libraries and source code files can be added to the project. Make sure that there is always just a single main() function in the code, otherwise the linker will not know which executable to build after the compilation. Before adding source code files delete the default folder “Sources” by selecting it in the project window and choosing:

Project  Remove Selected Items...

To add libraries choose:

Project   Add Files...  Add

The desired files will have the extension “.lib”. Under Windows, the file type must be set to library files, otherwise the user will not see the libraries.

To add source code files:

Project   Add Files...  Add

Source code files are shown by default.

4. Compile the project:

Project   Make

will invoke a compilation monitored by a compilation progress box. This box will also report when the application program is linked. If one gets no error messages in this process, an executable is produced and can be run by selecting:

Project   Run

Compiler and linker error and warning messages are documented in the Code Warrior reference manual which comes in “.pdf” and HTML formats on the documentation CD shipped with CW. All the best of luck with the programming.

IMPORTANT: As you start to compile the CSMP++ libraries you will get some compilation errors that originate in the Windows header file ‘winnt.h’.  You need to change and save this file so that it complies with the C++ standard. The standard does not allow the declaration of arrays of undefined size:

EVENT_LOG_RECORD  eventLogRecord  recod[];  // wrong

must be changed to:

EVENT_LOG_RECORD*  eventLogRecord  record;  //  a pointer=standard C++

The same has to be done with the DWORD declaration.

#### PC Windows 2000 / XP

Similar to the Macintosh platform, CSMP++ has been ported to Windows NT 4.0, using CodeWarrior Professional Vs 8.0 and greater.

#### Linux

The recommended compiler is Intel’s C++ compiler 7.1 or greater. It generates excellent binary code which executes very reliably and fast. There are corresponding makefiles in the CSMP++ distribution with optimization flags pre-set to the best settings for the different processor types.

KDE offers a very nice integrated develoment environment under LINUX that operates by default in collaboration with the GNU C++ compiler. While the resulting code is significantly slower than that created with the Intel compiler this is still a very powerful combination.

#### SGI / Irix 6.5 or Later

On the Silicon Graphics platform, the freely available GNU compiler, and theoretically also the Silicon Graphics MIPS Pro C++ compiler (vs. 7.3 or later), can be used to develop CSMP++ applications. A very good implementation of the STL library is available on this platform and the Mips Pro compiler in the Developer’s Workshop package is robust and has relatively good warning and error reporting. However, this compiler always creates automatically a directory “ii_files/” in which it stores linker information for the object files. It fails to track changes of header files in this repository. Often enough it is therefore necessary to delete the “ii_files”; for instance, if a compilation will fail to produce a library at the link stage. The compiler sometimes also fails to instantiate templates such that the object code for these is reported as missing when the libraries are linked at the end of the compilation. In this case, a sure way to get the necessary object code is to create dummy template instances in the main program which match the type of the missing ones. It seems much easier just to use the GNU compiler !

The Developer Pro Workshop on the SGI also includes the CaseVision debugger (cvd) which facilitates graphical debugging of C++ code. However, this debugger was not written for C++ and it is terribly slow and cumbersome to take a look into class objects. Given that the SGI developers workshop also is a commercial add-on software package, the academically freely available Sniff development environment is recommended for the SGI platform.

# CSMP++ Glossary

The Glossary below is a brief and by far not comprehensive alphabetical listing of terminology employed in the documentation of CSMP++. Please provide feedback where items are missing and where explanations are too cryptic. There are also some explanations of C++ jargon. If the listed expression is C++ terminology, I have indicated this by putting (C++) behind it. Specific CSMP++ terminology like CSMP++ object names is underlined.

Stephan K. Matthäi, August 31, 2004

aggregate. Association of objects that collaborate to achieve functionality but none of them manages the others in contrast to a composite.

AMG. Algebraic multigrid method (Ruge & Stüben 1987) used inside the CSMP++ solver. See Briggs, W.L.’s multigrid tutorial (SIAM 1987) for a description of the method.

Argument. Input or output value of a method or a function. This value is entered into or retrieved from the function by writing it into the braces behind the function name, e.g. myFunction( argument1, argument2, argument3 ). In the function declaration placeholders are used for the arguments. These defined the type of the expected input or output, e.g., MyFunction( int argument1, House& argument2, Car& argument3 ). The & means that a reference to the actual input/output object is passed. Otherwise the function will make a copy of the argument for internal use (logically, large objects should not be copied and therefore always be passed by references).

Baseclass. A class object which describes the commonalities of an object  hierarchy. The base class defines an interface for inherited objects, thereby specifying to the program how to handle these even when these are not known at the time when the base class is implemented.

composite. Object that consists of several other classes which it manages, example: Model.

const. C++ type qualifier which means  that the expression which follows to the right is not modified after its definition or at least by the function to which it is passed, for example: const int pi=3.1414; or MyFunction( const Region& g ) const;

constructor. Default method with the same name as the class object, which it initializes when an instance of the class is created. Arguments to the constructor method provide initial data for the new object.

enum. C++ construct associating string variables with integer values such that these can be used as array indexing variables and for debugging.

Region. Collection of finite-elements, the global mesh ID numbers of which, are stored in an STL  map. Use groups to manipulate geological entities of your model selectively.

Interface. List of operations supported by a piece of software or hardware; typically, the sum of the methods(=functions) of  a class object which have been made accessible to the public.

Operand. A handle class (Operand) to a distributed CSMP++ physical variable described in the PropertyDatabase object and stored in the CSMP++_MemoryManager (both objects inside the Model object). Operands allow you to access variables in computations. To do this you construct an Operand with the variable name as constructor argument.

Method. A function that is associated with a class so that it cannot be called by itself in contrast with normal functions like double sqrt().

Polymorphism. The ability to treat different objects the same way which is defined in a base class.

Subclass. A class which is inherited from a base class. The base class thereby captures the commonalities of all objects to which the subclass belongs, for example, automobiles when the subclass implements a VW beetle. A class A is a subclass of a baseclass B when the statement “A is a B” is true.

NAN can be tested for using the standard C++ function isnan() but its initialization varies from compiler to compiler.

Since property values can be placed on the regions an akward situation arises when regions overlap. Then, dependent on which region is queried, reading the same property may return a different value.