# Abaqus .inp file format documentation

H. Agheshlui 22/2/2017

Keyword lines:

Keywords (or options) always begin with a star or asterisk (*). Keywords are often followed by parameters, some of which may be required. For instance:

*ELEMENT, TYPE=T2D2, ELSET=FRAME (optional)

Parameters are separated by a comma and a comma at the end of the line means that the next line is continuing from the previous one.

List of commands for defining geometry:

| *NODE | Defining node coordinates |
| --- | --- |
| *ELEMENT | element connectivity |
| *Elset | Creates element sets |

Keywords and parameters are case independent and must use enough characters to make them unique.

Data Line

Keyword lines are usually followed by data lines. Examples:

*NODE

101, 0., 0., 0.

102, 1., 0., 0.

103, 2., 0., 0.

104, 0.5, 0.866, 0.

105, 1.5, 0.866, 0.

*Element, type=C3D10

1,    825,   9671,    948,   1900,  18753,  18677,  16384,  18756,  18748,  18749

*Elset, elset=Set-1, generate

1 (first element),  62252 (last element),      1 (increment)

Element types:

2D Stress/displacement elements

Plane Strain Elements:

CPE3 		3-node linear

CPE4	 	4-node bilinear

CPE6		6-node quadratic

CPE8 		8-node biquadratic

#### Plane stress elements

CPS3		3-node linear

CPS4    	4-node bilinear

CPS6    	6-node quadratic

CPS8    	8-node biquadratic

3D Stress/displacement elements

Element naming convention

C3D4		4-node linear tetrahedron

C3D6 		6-node linear triangular prism

C3D8 		8-node linear brick

C3D10 	10-node quadratic tetrahedron

C3D15 	15-node quadratic triangular prism

C3D20 	20-node quadratic brick

## Defining Regions in Input Model

Use parts. As below:

*Part, name=Part-1

*Node

1,  0.852882206,           2.,    2.5292635

.

.

.

*Element, type=C3D10

1,  1,  9,  4,  2, 12, 11, 10, 14, 13, 15

.

.

.

**The next four lines are optional and not needed for geometry definition. But they can be used if they make it easier for CSMP purposes.

*Elset, elset=Set-1, generate

1 (first elem),  249653 (last elem),       1 (step)

** Section: TopSeal

*Solid Section, elset=Set-1, material=Shale

*End Part