# How To: SKUA to ANSYS

SKM 26-4-2012

Meshing workflow for volumetric mesh generation from “faceted geometry:”

1.  Output model as (AutoCAD) DXF from SKUA and read it into Ansys via:

Import -> DWG

2. Build a Topology needed for Patch-Independent meshing and use color-coded new curves and points to verify that this topology does indeed include all the features of your input model.

3. Mesh using Tetra / Hexcore and convert structured tetrahedral mesh into Hexahedrons

4. Rename the automatically generated material points

5. Identify boundary flags

6. Output meshed model to CSMP

# Faceted Geometry Meshing Features of ANSYS

NB: Ansys keywords / menu options are printed in bold

1. Bringing in the geometry:  Import Geometry

- Edit:   Mesh -> Facets

From menu: Settings -> Model

Topo Tolerance

- Restore: to default = 0.05% of bounding box diagonal

- determines how closely spline surfaces are approximated

- use a high tolerance for large models,

but low tolerance when meshing

- recommended value 0.001

- not used for faceted surfaces

-Triangulation Tolerance

=maximum distance allowed between mesh  triangle edge and

surface /  curve

- in absolute terms (needs to be bigger for large models)

- not in effect for faceted input geometry

- use: Unitless Tolerance to rescale in terms of min edge length of

a surface

- Repair Geometry

- Create Modify / Faceted

Faceted Cleanup family of options

- Geometry -> Close faceted holes (to triangulate holes in surfaces

that are used as input geometries for meshing

- ZZZ = Restore Dormant Entity

- Tolerance

(after Build Topology and dependent on what tolerance was set.

colors indicate what was found:

- green = freestanding curves

- yellow = curves are interpreted as adjacent to only one surface

- (where 2 surfaces meet at that location tolerance should be increased

- to capture this)

- red       = curves joining 2 surfaces

- blue     = 3 or more objects / curves coincide because more curves edges if    the tolerance of the model is so large that they are  regarded as coincident leading to their elimination)

- grey = dormant curves

- use smaller tolerance to rebuild topology than for meshing;

- tolerance should be at least 1/10 of the smallest distance

to be resolved

- Use Local Tolerance

- for models with multiple length scales

(however, explanation in fig. 44 suggests that this

feature should not be applied in this case)

- Build Topology (rebuild diagnostic topology)

- creates missing curves and points

- deletes duplicate geometry that coincides within given tolerance

- can damage input geometry

- Single curve cleanup (only to give edge curves extra tolerance)

- Split Surface at T-connections

(achieves a conforming mesh even if the T-contacted surface

is not split at T joint;  splits surface if curves form closed loop)

- Split Surface at Interior Curves

(get mesh to confirm to intersection curves that are restricted to

interior of a surface and do not form closed loops)

NB: these curves may be dormant.

# Cuu Long Meshing Experience

Even if the topology of the model is completely correct a few bad choices of parameters can completely corrupt the mesh:

- When a too small edge criterion is choosen (i.e. 0.001), intersections may not get meshed at all

- Model: A too small topo tolerance may lead to poor tracking of curved surfaces

- Model: A too low triangle tolerance may lead to very poor tracking of the geometry

- A too large scale factor may cause the mesher not too be able to stay on the geometry

- Any missing curve intersection point will lead to penetrating / overlapping elements and non-manifold vertices

- Any too low mesh size will lead to orders of magnitude too large element numbers

Good parameters for the 13-km NURBS model were

Scale factor=1

Max element=400

Curvature-based refinement min element=20

Edge criterion=0.1

Fixing non-manifold vertices

To get rid of the Non-Manifolds, you need to disconnect the common node that is shared between elements that are NOT lying in the same plane.

To do so, you simply need to rebuild the mesh topology with the default parameters (Edit Mesh > Repair Mesh > Build Mesh Topology)... this is the same trick that I told you to use last time already, it works wonders on the mesh you've sent.